San Francisco PSInSAR Study

Final Report

Dataset: ERS DESCENDING

To the attention of:

Prof Roland Burgmann

SEISMO Lab.

University of Berkeley

California

Date: 28 August 2007

Ref: 06.073C

Prepared for:

SEISMO Lab

University of Berkeley

California

Prof. Roland Burgmann

by:

Tele-Rilevamento Europa

Via Vittoria Colonna, 7

20149 Milano

tel: 02.4343.121

fax: 02.4343.1230

http://www.treuropa.com

tre@treuropa.com

Ref:

Engineer entitled of this processing: Fabrizio Novali

Project Manager: Fabrizio Novali

Index

1	Data-Set used for PS analysis: RSAT F4 TRACK 106	4

1.1	Images discarded	5

2	PS report	5

2.1	Information on LOS velocity field and target motion	6

2.2	Velocity Field	6

2.3	Velocity Standard Deviation Field	7

# Data-Set used for PS analysis: RSAT F4 TRACK 106

| # | Acquisition date | Satellite | Dc * | Bn ^ | Bt [day] |
| --- | --- | --- | --- | --- | --- |
| 1 | 19920506 | ERS-1 | 0,261 | 0,221057 | -2614 |
| 2 | 19920819 | ERS-1 | 0,2476 | 0,014221 | -2509 |
| 3 | 19930106 | ERS-1 | 0,2167 | -0,41138 | -2369 |
| 4 | 19960329 | ERS-1 | 0,2406 | -0,28869 | -1191 |
| 5 | 19960330 | ERS-2 | 0,0649 | -0,37349 | -1190 |
| 6 | 19960504 | ERS-2 | 0,0963 | 0,647118 | -1155 |
| 7 | 19961130 | ERS-2 | 0,0779 | 0,763051 | -945 |
| 8 | 19971011 | ERS-2 | 0,0523 | 0,394099 | -630 |
| 9 | 19971220 | ERS-2 | 0,1234 | 0,147887 | -560 |
| 10 | 19980404 | ERS-2 | 0,0578 | -0,30341 | -455 |
| 11 | 19980509 | ERS-2 | 0,0982 | 0,531348 | -420 |
| 12 | 19980718 | ERS-2 | 0,0627 | -0,50944 | -350 |
| 13 | 19980822 | ERS-2 | 0,0635 | -0,22801 | -315 |
| 14 | 19980926 | ERS-2 | 0,0696 | 0,527276 | -280 |
| 15 | 19981031 | ERS-2 | 0,0554 | 0,678675 | -245 |
| 16 | 19981205 | ERS-2 | 0,0738 | -0,16291 | -210 |
| 17 | 19990109 | ERS-2 | 0,1191 | -0,7142 | -175 |
| 18 | 19990213 | ERS-2 | 0,1162 | 0,701335 | -140 |
| 19 | 19990320 | ERS-2 | 0,1306 | 0,19668 | -105 |
| 20 | 19990424 | ERS-2 | 0,1059 | -0,05017 | -70 |
| 21(M) | 19990703 | ERS-2 | 0,0797 | 0 | 0 |
| 22 | 19990807 | ERS-2 | 0,0185 | 0,552383 | 35 |
| 23 | 19990911 | ERS-2 | 0,0576 | -0,44716 | 70 |
| 24 | 19991016 | ERS-2 | 0,0648 | -0,09217 | 105 |
| 25 | 19991120 | ERS-2 | 0,0572 | -0,20157 | 140 |
| 26 | 19991225 | ERS-2 | 0,1017 | 0,350846 | 175 |
| 27 | 20000129 | ERS-2 | 0,1047 | 0,055911 | 210 |
| 28 | 20000304 | ERS-2 | 0,3524 | -0,22241 | 245 |
| 29 | 20000408 | ERS-2 | 0,0316 | 0,098363 | 280 |
| 30 | 20000617 | ERS-2 | -0,3022 | 0,006411 | 350 |
| 31 | 20000722 | ERS-2 | -0,3004 | -0,21677 | 385 |
| 32 | 20000826 | ERS-2 | -0,2378 | 0,606834 | 420 |
| 33 | 20000930 | ERS-2 | 0,051 | -0,19921 | 455 |
| 34 | 20001104 | ERS-2 | 0,0174 | 0,157235 | 490 |
| 35 | 20001209 | ERS-2 | 0,2154 | -0,28389 | 525 |

^ Fraction of critical baseline    	* Fraction of PRF 	(M) Master scene

## Images discarded

| Acquisition date | Satellite | Bn ^ | Dc * | Bt [day] | Motivation |
| --- | --- | --- | --- | --- | --- |
| 19920715 | ERS1 | -0.034 | 0.185 | -2544 | No Data |
| 19951215 | ERS1 | 0.620 | 0.150 | -1296 | No Data |
| 19980817 | ERS2 | -0.132 | -0.044 | -1050 | Bad IFC |
| 19990529 | ERS2 | 0.619 | 0.005 | -35 | Bad IFC |

^ Fraction of critical baseline    	* Fraction of PRF

# PS report

| Site Name | SAN_FRANCISCO_ERS_D_T70_2853_BERKELEY |
| --- | --- |
| Customer | BERKELEY |
| Reference Documents |  |
| Engineer entitled of this processing | Fabrizio Novali |
| Project manager | Fabrizio Novali |

| Process Date (final result) | 28/08/2007 |
| --- | --- |
| Software used | PSproc |
| Version | 3.0 |
| Analysis type | Standard PS Analysis |
| Interpolation factor | 2 |
| Satellite data used | ERS |
| Descending or Ascending Orbit | DESCENDING |
| Critical Baseline [m] | 1286.86 |
| Pulse Repetition Frequency [Hz] | 1679.02 |
| Number of scenes used | 35 |
| Date range of analysis | 06 May 1992 – 09 December 2000 |
| UTC Time / Local Time | 18:50:32 |

| Reference data used for georeference | LandSat |
| --- | --- |
| Projection system used | UTM10 – WGS84 |
| Reference Point location | NORTH: 4147315.99 EAST: 596286.76 |
| Hypothesis on reference point motion | motionless |
| Extension of the Area of interest | 8700 Kmq |
| Number of PS identified | 128133 |
| False Alarm Rate | 10-05 |
| Average PS density (PS/km2) | 14.73 |

## Information on LOS velocity field and target motion

| Line Of Sight Versor Coordinates | H: 0.9193 | E: 0.3857 | N: -0.0781 |
| --- | --- | --- | --- |
| Tilt or phase trends have been removed during PS processing? |  |  |  |
| Files delivered | SAN_FRANCISCOERS_D_T70_REF.dbf SAN_FRANCISCOERS_D_T70_TS.dbf |  |  |

## Velocity Field

## Velocity Standard Deviation Field