A rectangular weir was made from wood boards and mounted on the outlet of the small pond around the Calistoga Old faithful Geyser to measure the discharge on May 6, 2010.

Weir geometry:

- shape: rectangular

- channel width: 27.7 cm

- length: 1.2 m

- slope of weir during flow measurement: 0.03 radian

Depth of water in the weir (8 mm) was measured against a steel ruler in the weir, as read from closeup photos taken by Jonathan Glen.

Discharge through the weir were estimated by two methods: 1. Measurement of flow velocity, and 2. measurement of discharged volume per unit time.

- Measurement of flow velocity:

The mean velocity of flow in the weir was estimated by two methods: 1) Measurement of surface velocity of the flow using a fast camera, and 2) using the Manning formula.

- Surface velocity measurement with a fast camera (Ben Andrews)

The sequence of measurements, together with the means and standard deviations, are listed in the attachment. The mean surface velocity is shown as a function of time in the following diagram. Fluctuations occur between 61 and 72 cm/s.

The mean of the measured velocity is vo ~ 66 cm/s, from which we estimate upper bound of the average velocity of the water layer by  v ~ (2/3)66 = 44 cm/s - the factor of 2/3 is that to convert surface velocity to mean velocity for a 2-dimensional flow on an inclined plane (i.e., no lateral edges). Accepting this, we estimate the mean discharge through the weir as

Q = A v = (0.277 x 0.008) x 0.44 = 0.98x10-4 m3/s = 0.98 liter/s.

The discharge so estimated does not account for the retarding effect at the lateral edges of the weir, thus should be taken as the upper bound.

- Estimate of mean velocity from using Manning’s equation:

v =  R2/3 S1/2

where R is the hydraulic radius, or the ratio of the cross-sectional area of the flow (in m2) divided by the wetted perimeter of the weir (in m), S is the slope of the water surface (or the weir), and n is the Manning roughness parameter.

Measurements:

- width of weir: w = 0.277 m

- depth of water in the weir: d = 0.008 m

- slope of weir: S = 0.03 radian (measured from water level marked on the side of weir)

- R = (0.277 x 0.008)/(0.277 + 0.008 + 0.008) = 0.00756

Thus the mean velocity is

v =  0.0068.

For the wooden weir, n may lie somewhere between that for straight unlined earth canals (0.02) and that for smoothed concrete (0.012). Assuming n = 0.02, we get

v = 0.34 m/s

and

Q =  v A = 0.34 x (0.277 x 0.008)

= 0.00075 m3/s

= 0.75 liter/s

General comment: (1) The agreement between the above result and that from volume measurement (below) may be fortuitous because n = 0.02 was assumed in the calculation of velocity.

- Discharge estimated from measurement of water volume discharged per unit time:

Water discharged through the weir in time durations  of about 10 seconds was collected using a large plastic bag; the volume of this water V is measured with a calibrated bucket. Discharge Q is calculated from Q = V/. Following is a listing of the measurements:

(sec)		V (liter)		Q (liter/sec)	Comments_______________________

10.78		6.757		0.627		first measurement; may have large error

11.09		8.446		0.762

10.25		7.320		0.714

10.67		6.475		0.607		measured just before geyser eruption

10.58		7.433		0.703		measured during geyser eruption

10.42		7.320		0.703		measured after geyser eruption________

General comments: (1) Measurements covered only one cycle of eruption. The variations in Q may either represent experimental errors or real variations in discharge. (2) Measurements were made by Jennifer Frederick, Shaul Hurwitz and Chi Wang.

Attachment: Data from fast camera (Ben Andrews):

| FILE | Time (s) | d (cm) | frames | Del t(s) | v(cm/s) | avg(cm/s) | s.d(cm/s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 11.MTS | 124010 | 24.6 | 10 | 0.333333 | 73.8 | 73.1 | 0.96 |
|  |  | 24.5 | 10 | 0.333333 | 73.5 |  |  |
|  |  | 24 | 10 | 0.333333 | 72 |  |  |
| 12.MTS | 124110 | 25 | 12 | 0.4 | 62.5 | 65.83333 | 2.97 |
|  |  | 25 | 11 | 0.366666 | 68.18181 |  |  |
|  |  | 24.5 | 11 | 0.366666 | 66.81818 |  |  |
| 14.MTS | 124210 | 23.5 | 12 | 0.4 | 58.75 | 63.40151 | 4.17 |
|  |  | 23.7 | 11 | 0.366666 | 64.63636 |  |  |
|  |  | 24.5 | 11 | 0.366666 | 66.81818 |  |  |
| 15.MTS | 124310 | 24 | 11 | 0.366667 | 65.45455 | 69.81818 | 3.78 |
|  |  | 24 | 10 | 0.333333 | 72 |  |  |
|  |  | 24 | 10 | 0.333333 | 72 |  |  |
| 16.MTS | 124410 | 20 | 10 | 0.333333 | 60 | 61.81818 | 1.56 |
|  |  | 23 | 11 | 0.366667 | 62.72727 |  |  |
|  |  | 23 | 11 | 0.366667 | 62.72727 |  |  |
|  | 124415 | 25 | 11 | 0.366667 | 68.18182 | 61.39860 | 8.46 |
|  |  | 22.5 | 13 | 0.433333 | 51.92308 |  |  |
|  |  | 23.5 | 11 | 0.366667 | 64.09091 |  |  |
|  | 124425 | 23 | 11 | 0.366667 | 62.72727 | 65.77273 | 4.15 |
|  |  | 23.5 | 11 | 0.366667 | 64.09091 |  |  |
|  |  | 23.5 | 10 | 0.333333 | 70.5 |  |  |
|  | 124440 | 23 | 10 | 0.333333 | 69 | 66.7 | 5.90 |
|  |  | 24 | 12 | 0.4 | 60 |  |  |
|  |  | 23.7 | 10 | 0.333333 | 71.1 |  |  |
|  | 124500 | 23.5 | 10 | 0.333333 | 70.5 | 64.91667 | 4.91 |
|  |  | 21 | 10 | 0.333333 | 63 |  |  |
|  |  | 24.5 | 12 | 0.4 | 61.25 |  |  |
|  | 124505 | 25.5 | 10 | 0.333333 | 76.5 | 71 | 4.82 |
|  |  | 22.5 | 10 | 0.333333 | 67.5 |  |  |
|  |  | 23 | 10 | 0.333333 | 69 |  |  |
|  | 124515 | 24 | 12 | 0.4 | 60 | 61.25 | 2.17 |
|  |  | 25.5 | 12 | 0.4 | 63.75 |  |  |
|  |  | 24 | 12 | 0.4 | 60 |  |  |
|  | 124530 | 25 | 10 | 0.333333 | 75 | 71.72727 | 3.41 |
|  |  | 24 | 10 | 0.333333 | 72 |  |  |
|  |  | 25 | 11 | 0.366667 | 68.18182 |  |  |
|  | 124550 | 24.5 | 10 | 0.333333 | 73.5 | 71.52727 | 2.90 |
|  |  | 25 | 11 | 0.366667 | 68.18182 |  |  |
|  |  | 24.3 | 10 | 0.333333 | 72.9 |  |  |
| 17.MTS | 124610 | 21 | 10 | 0.333333 | 63 | 67.5 | 4.50 |
|  |  | 24 | 10 | 0.333333 | 72 |  |  |
|  |  | 22.5 | 10 | 0.333333 | 67.5 |  |  |
|  | 124620 | 25 | 12 | 0.4 | 62.5 | 61.32576 | 2.23 |
|  |  | 23 | 11 | 0.366667 | 62.72727 |  |  |
|  |  | 23.5 | 12 | 0.4 | 58.75 |  |  |
|  | 124630 | 24 | 10 | 0.333333 | 72 | 64.90909 | 6.30 |
|  |  | 24 | 12 | 0.4 | 60 |  |  |
|  |  | 23 | 11 | 0.366667 | 62.72727 |  |  |
|  | 124640 | 24.5 | 11 | 0.366667 | 66.81818 | 64.33333 | 5.53 |
|  |  | 25 | 11 | 0.366667 | 68.18182 |  |  |
|  |  | 23.2 | 12 | 0.4 | 58 |  |  |
| 18.MTS | 124710 | 21.5 | 10 | 0.333333 | 64.5 | 65.90909 | 4.08 |
|  |  | 23.5 | 10 | 0.333333 | 70.5 |  |  |
|  |  | 23 | 11 | 0.366667 | 62.72727 |  |  |
| 19.MTS | 124740 | 25.8 | 12 | 0.4 | 64.5 | 65.36364 | 1.86 |
|  |  | 23.5 | 11 | 0.366667 | 64.09091 |  |  |
|  |  | 22.5 | 10 | 0.333333 | 67.5 |  |  |