Update to Preliminary channel description, SAFOD optical fiber strainmeter.

Tuesday, July 9, 2013

| Name | Description | Int.# | Prec. | Smpl /s | Units | conversion | comment | Channel. location |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| e1a | ellipse a | 1 ref | real*4 | 10 | volts | -- | ellipse parameter, x-axis radius | AXA.01 |
| e1b | ellipse b | 1 ref | real*4 | 10 | volts | -- | ellipse parameter, y-axis radius | AXB.01 |
| e1d | ellipse d | 1 ref | real*4 | 10 | volts | -- | ellipse parameter, phase offset | AXD.01 |
| e1x | ellipse x0 | 1 ref | real*4 | 10 | volts | -- | ellipse parameter, x-axis center | AXX.01 |
| e1y | ellipse y0 | 1 ref | real*4 | 10 | volts | -- | ellipse parameter, y-axis center | AXY.01 |
| p1f | phase fast | 1 ref | real*8 | 400 | radians | 0.124 ne/rad | reference interferometer phase | DCZ.D1 |
| p1f32 | phase fast | 1 ref | real*4 | 400 | radians | 0.124 ne/rad | reference interferometer phase | DCZ.S1 |
| p1s | phase slow | 1 ref | real*8 | 40 | radians | 0.124 ne/rad | reference interferometer phase | SCZ.D1 |
| p1s32 | phase slow | 1 ref | real*4 | 40 | radians | 0.124 ne/rad | reference interferometer phase | SCZ.S1 |
| x1 | raw x | 1 ref | int*2 | 400 | counts | 0.305 mV/count | raw fringe signal, x-axis, direct | AYX.01 |
| y1 | raw y | 1 ref | int*2 | 400 | counts | 0.305 mV/count | raw fringe signal, y-axis, PSD | AYY.01 |
| e2a | ellipse a | 2 bhl | real*4 | 10 | volts | -- | ellipse parameter, x-axis radius | AXA.02 |
| e2b | ellipse b | 2 bhl | real*4 | 10 | volts | -- | ellipse parameter, y-axis radius | AXB.02 |
| e2d | ellipse d | 2 bhl | real*4 | 10 | volts | -- | ellipse parameter, phase offset | AXD.02 |
| e2x | ellipse x0 | 2 bhl | real*4 | 10 | volts | -- | ellipse parameter, x-axis center | AXX.02 |
| e2y | ellipse y0 | 2 bhl | real*4 | 10 | volts | -- | ellipse parameter, y-axis center | AXY.02 |
| p2f | phase fast | 2 bhl | real*8 | 400 | radians | 0.136 ne/rad | borehole interferometer phase | DSZ.D2 |
| p2f32 | phase fast | 2 bhl | real*4 | 400 | radians | 0.136 ne/rad | borehole interferometer phase | DSZ.S2 |
| p2s | phase slow | 2 bhl | real*8 | 40 | radians | 0.136 ne/rad | borehole interferometer phase | SSZ.D2 |
| p2s32 | phase slow | 2 bhl | real*4 | 40 | radians | 0.136 ne/rad | borehole interferometer phase | SSZ.S2 |
| x2 | raw x | 2 bhl | int*2 | 400 | counts | 0.305 mV/count | raw fringe signal, x-axis, direct | AYX.02 |
| y2 | raw y | 2 bhl | int*2 | 400 | counts | 0.305 mV/count | raw fringe signal, y-axis, PSD | AYY.02 |
| a4 | analog 4 | 1 ref | int*2 | 400 | counts | -0.098 rad/count | fringe counter, ref. inter | AY4.01 |
| a5 | analog 5 | 2 bhl | int*2 | 400 | counts | -0.098 rad/count | fringe counter, ref. borehole | AY5.02 |
| a6 | analog 6 | -- | int*2 | 400 | counts | 3.05 mC/count | thermometer, trailer | BKT.00 |
| a7 | analog 7 | -- | int*2 | 400 | counts | 3.05 mC/count | thermometer, wellhead | BKW.00 |