Oman Survey 2006 - GPS site description

Site Name : SAHM

Location: 	JEBEL SAHMAN

Long.: 54º42’ 41’’E   Lat.:17º06’ 43’’N    Alt: 1314m

Timing : 1.5 hour from Salalah

Notes : May be difficult to find

Site description :  From Salalah, take the western road to Mirbat. About 4 km after Taqah, turn left to TAWI ATAIR (PK 0). At the entrance of Tawi Atair, turn left (PK 17.6) on the road indicated Jebel Sahman. Then, turn right 400m after to the direction of Jebel Sahman. Drive for 20 km on this road. You pass a high red and white radio antenna. At PK 36.7, there is a fork. Take on the right.

Arrive at PK 37.2 and park the car.

On February 2006, the road was under construction. The fork was at 54º42’24E and 17º06’46N.

The benchmark is on the top of a rocky hill. About 60 m from where you park the car.