Using SHALSTAB Tools to map 
shallow landslide potential

Introduction

SHALSTAB Tools is a suite of software tools that maps shallow landslide potential in a Microsoft Windows-based operating environment. The software has been implemented as an extension of the ArcView geographic information system (GIS) program from Environmental Systems Research Institute Inc. (ESRI). Using SHALSTAB, you can analyze digital elevation model (DEM) grid data and generate statistics that compare mapped distribution of landslides with similar statistics of random locations.

To perform DEM modeling calculations, SHALSTAB Tools uses computer routines written in the C++ programming language. These routines are contained in a customized, dynamic link library (DLL) and in an ArcView extension file, and are called from within ArcView for specific computations. In this way, you can direct SHALSTAB tasks through an ArcView session from a DEM to manipulate data, and generate landscape models and statistics that you can analyze to determine landscape stability.

ArcView overview

ArcView provides the framework for using the SHALSTAB tools. This user guide assumes that you are already familiar with the basic principles of geomorphology and with the concepts of the ArcView environment. However, the tutorial section contains all the steps that you need to analyze your data, so you will still be able to run perform a SHALSTAB analysis even if you have little experience using ArcView.

NOTE: ArcView contains a useful online Help system that you can consult for more details if you’re not familiar with its features. For complete information about how to use the ArcView GIS program, consult the ArcView documentation.

Working with projects and views

ArcView stores your work in a file called a project. A project typically contains the views, tables, charts, layouts, and scripts that you use for one application. Project files have an .apr extension. A project file doesn’t contain the actual data you use in ArcView. It contains references to the location of these data sources. The reason for this is so you can use the same data for any number of projects without duplicating them. When you create a new project, the components in the project are listed in the project window. From this window, you can create new components, rename existing ones, or remove components from the project.

ArcView displays maps onscreen in view windows, and SHALSTAB also performs its DEM data manipulations in these view windows or “views.” To start a new view, select File—>New Project and click the New button in the Untitled view menu. This opens an empty window, View1, to which you can select properties and add themes, which represent data features that you import in the form of DEM files.

System requirements and installation

Because SHALSTAB Tools builds on the geographic analysis abilities of ArcView, you must have the the following software installed on your computer’s hard drive:

- ArcView GIS Version 3.1 for Windows

- ArcView GIS Version Spatial Analyst Version 1.1 for Windows, which is a file extension that provides routines for operating on grid geographic data

- SHALSTAB Tools Version 1.0, which is also an ArcView extension package

- Finally, to run SHALSTAB Tools, you must be using the Microsoft Windows NT, or Windows 95 or 98 operating system software

To run SHALSTAB Tools efficiently, we recommend that you have at least 32 Megabytes (MB) and optimally 128 MB of RAM on your computer.

Installation

The SHALSTAB toolset contains two files, which should be placed on your computer in the following locations:

- shalstab.avx—This is the ArcView extension file, which will enable you to select SHALSTAB Tools from within the ArcView program. Copy this file to the directory called arcview/ext32.

- shalstab.dll—This is the dynamic link library file that contains the SHALSTAB routines. Copy this file to the directory arcview/bin32.

- shaltutorial.doc—This is the tutorial. Copy this file to the directory arcview/etc. to make this document available through the SHALspecial submenu.

Data files

In addition to copying the core SHALSTAB files to your computer, you should create a “work directory” on your hard drive that contains the geographic spatial data source files for the project you want to create. For this tutorial, create a work directory on the C drive, and name the directory shalstab\redwood. To use SHALSTAB Tools, these data files must be in one of the following formats:

- Arc/Info ASCII grid files

- U.S.G.S. DEM grid files

- ArcView grid themes

- Surfer ASCII grid (.GRD) files

- Arcview  shapefiles for  the mapped landslides (if they exist)

These sample data files are included for your use in this tutorial. You should load them to your working directory:

- Work directory: c:\shalstab\redwood

- Elevation data: c:\shalstab\redwood \elev

- Landslide data: c:\shalstab\redwood \shallow.shp, shallow.shx,shallow.dbf,shallow.sbx,,shallow.sbn

The sample files are from a tributary of the Noyo basin in Northern California. We use this example because it is most similar to that which will be encountered in land used for timber production in the Pacific Northwest. Topographic data come from digitized 7.5' U.S.G.S. topographic maps grided to 10-m spacing using TOPOGRID (from ArcInfo). The landslides were mapped from aerial photographs (by John Coyle) and in some cases include both the initial failure site and some of the runoutthis is a common problem in data derived without ground checking. The typically poor fine-scale defintion of the topography found on 7.5' quadrangles means that landslide locations are not highly accurate. The watershed has experienced repeated timber harvests over various scales for many decades and now consists of mixtures of second- and third-growth redwood trees, and various hardwoods (primarily tan oak and madronne). It should be noted that inaccurate mapping tends to make the observed landslide locations appear to differ little from randomly located landslides.

Creating a SHALSTAB Tools model

This section provides step-by-step instruction on how to use SHALSTAB Tool in the ArcView program. In the following steps, you will use sample data files to create a simple model for analysis. For this tutorial, the c:\shalstab\redwood directory holds these sample files. If you do not see the c:\shalstab\redwood directory on your computer, create a c:\shalstab\redwood directory and copy to that directory the sample DEM and landslide files provided.

WARNING: SHALSTAB Tools performs all of its calculations with the assumption that the elevation data you provide is in meters, and it will in fact, not allow you to run the model unless the units are in meters.

This tutorial assumes that you have a basic working knowledge of the ArcView program. However, even if you don’t have much experience with ArcView, the tutorial instructs you on every step that you’ll need to take in order to create a SHALSTAB Tools stability model.

NOTE: You can use your own data for this exercise. If you do, simply import the data files from your disk, or you can copy them to the c:\shalstab directory. It doesn’t matter where the data files are located; SHALSTAB Tools writes all the files to the working directory that you have defined

Step-by-step procedure

For this tutorial, SHALSTAB, ArcView, and the ArcView Spatial Analyst extension are already installed on your computer. The ArcView icon should be displayed on your desktop. If it is not, ask your workshop administrator to check this. Now you’re ready to begin creating your SHALSTAB model.

Starting ArcView and loading the SHALSTAB tools

- Start ArcView within Microsoft Windows either by clicking twice on the ArcView desktop icon or by selecting 
Start—>Programs—>Esri—>ArcView GIS in Windows. An untitled project window opens.

- From the File menu, choose Extensions. Select both the SHALSTAB checkbox and the Spatial Analyst checkbox. Click the OK button. This loads the shalstab.avx extension file so that the program can operate within ArcView. Make sure you DON’T make the extension a default extension. This will prompt nasty error messages on the screen, although the program will still run.

- A dialog box asks you to enter the directory where your data files are located. SHALSTAB will place all the files that are generated for the model in this directory. It’s best to keep all your data files for a specific project in the same folder. For this example, the c:\shalstab directory is displayed in the field. Click OK. NOTE: If the c:\shalstab directory is not displayed, go ahead and create it now.

- Click the Open button in the contents window list box to display the View SHALSTAB window and the SHALSTAB Tools menubar. You can extend the borders of the main window to the size of your monitor screen so that you can more easily see the maps you’ll be generating in this session. Drag the View SHALSTAB window to the right of the project window so that they are side by side. The View SHALSTAB window is where the maps of your data will be displayed.

The SHALSTAB Tools menubar has five menus in addition to the standard ArcView menus: SHALtopo, SHALSTAB, SHALSTABco, SHALtest, and SHALspecial. Most of the functions of these menus are introduced in this procedure. For complete descriptions of the SHALSTAB Tools menus and submenus, see the section “SHALSTAB Tools menus” in the last section of this tutorial.

Loading elevation and landslide data

Before you create the SHALSTAB Tools model, you need to load the elevation and landslide data. The elevation data should be an ASCII grid file in ArcView, Arc/Info, or Surfer format. The landslide data should be an ArcView shapefile. For this tutorial, the sample files, elev and shallow.shp are provided in the c:\shalstab directory.

- If your elevation data is already in ArcView format you can add it to your view without converting it to a grid. To do this, select View—>Add Theme and then in the Add Theme dialog box choose Grid Data Source from the Data Source Types drop-down list. Then select the file name from the left list box and click OK. Now click in the empty checkbox next to the El10m theme to draw the map in the view window and proceed to Step 5.

If your data is NOT in ArcView format complete the additional bulleted steps as follows:

- Select File—> Import Data Source. In this dialog box, you can specify the file type for your elevation data.

- Choose ASCII Raster and click OK.

NOTE: If you are loading DEM files from the U.S.G.S. site, Select File—> Import Data Source and choose USGS DEM.

For more information about DEMs, consult the ArcView Help.

- The Import ASCII Raster Files dialog box is displayed. Double-click the c:\shalstab\redwood folder in the Directories list box and select elev.asc from the left list box, and then click OK.

- SHALSTAB assigns a grid name in the Grid Name field of the Output Grid dialog box, in this case imgrd1. You can enter a short (less than eight characters) meaningful name or use the default that SHALSTAB assigns. Click OK.

- The Import Grids dialog box asks if you want to import cell values as integers. Click Yes. I THOUGHT I WAS TOLD TO ALWAYS CLICK NO ON THIS! AM wrong?

- The Import Data Source dialog box asks whether you want to add the grid as a theme to the view. Click Yes. The contents frame shows the range of elevation for your DEM data in meters.

- Now select the empty checkbox in front of the grid name to draw a map of the landscape elevation data theme in the view window.

- Select SHALspecial—>Contours. This step creates lines of constant value on a point, grid, or TIN theme using a specified interval. The Contour Parameters dialog box is displayed with a default contour interval of 10. Click OK.

NOTE: You cannot change the contour interval for a theme once it is generated, but you can generate a new contour theme at any time from the original elevation grid or the theme with sinks removed.

Select the checkbox for the Contours of the Elrem theme to draw the contours of El10m with no sinks.

NOTE: As you create themes, you can click once on the “i” button in the second row of the ArcView toolbar and then click on the map to display information about the active map view.

Now you will import the landslide data and add it to your view.

- To add the mapped landslides, Select View—>Add Theme.

- With Feature Data Source shown in the Data Source Types field, select the shallow.shp file in the left list box (the Directory field shows c:\shalstab) and click OK.

- Click the empty checkbox in front of the Landslides.shp theme to draw the landslides on top of your elevation map in the view SHALSTAB window.

- Select the landslide theme by selecting SHALspecial—>Define Main Themes. This will ensure that your landslide theme is displayed on the top layer of your ArcView session overlaying the SHALSTAB Tools themes you create.

ArcView displays themes in layers and places the most recently loaded theme at the top of the view frame. You can control which theme is drawn in the foreground by clicking and holding the left mouse button over a theme and dragging it to the top of the contents frame. You can deselect any theme to hide it from view. This feature is useful for creating and displaying maps of the same landscape area with specific themes emphasized.

NOTE: To give your maps contrast and clarity, you can change the colors and patterns of themes by using the Legend Editor. This tool provides you with a variety of palettes to customize the colors and patterns of map elements. For more details about how to customize your project views with the Legend Editor, consult the ArcView online Help.

For this tutorial, the landslides are displayed with outlines. If you want to change them to solid colors, you can do so using the Legend Editor as follows:

- Open the Legend Editor by double-clicking on the Landslides.shp theme and double-click the rectangle in the Symbol column. This opens the Color Palette.

- Click once on the paint bucket in the toolbar and once on the solid fill box—the black button in the top-middle row. Click Apply. The landslides are redrawn as solid shapes.

Running the SHALSTAB Tools analysis

Now that you’ve loaded the elevation and landslide data, you are ready to use the SHALSTAB Tools to perform a stability analysis.

- Select SHALtopo —>Check Sinks. This step tells you the total number of cells that are considered sinks which exist in your landscape area.

- Select SHALtopo —>Show Sinks. This step tells you where the sinks in your landscape area are located.

- Select SHALtopo —>Remove sinks. This step fills all closed depressions in your elevation data.

- The Check elevation units dialog box asks if the elevation units are in meters. The default is Yes. If your data is not in meters, you should convert it to meters before continuing.

WARNING: SHALSTAB Tools is designed to work on a predefined hydrological watershed where the sinks have been removed. If your data includes a large flat area or a body of water such as a lake or part of an ocean, SHALSTAB Tools will not be able to perform the sink removal properly without the creation of a boundary for these areas. Typically DEMs that include a coastline will have this problem. We recommend that you mask them out by setting all elevations that are less than or equal to zero as “NODATA.” You may also skip this step (removing sinks) but keep in mind that your final contributing area/slope value calculations will be affected by not removing sinks. If your study area contains non-computer-generated sinks, artifacts of interpolation methods, we recommend that you run the SHALSTAB Tools routines without removing the sinks.

- Another dialog box asks if you still want to remove sinks from your elevation theme. Click OK.

- Select an elevation grid theme. The name you’ve given your elevation theme should be displayed in the field. In this case, Imgrd is displayed as the elevation grid theme. Click OK.

- SHALSTAB assigns a short name for the grid and theme with sinks removed. For this example, it’s Imrem. You can enter a short name (less than eight characters) or keep the default name. Click OK.

- The Output Grid dialog box displays imrem to confirm in which folder the grid will be placed. You can enter a new folder location at this point, or keep the one that’s displayed. Click OK.

- When you click the checkbox in front of the name, Elrem, the content window now displays a third theme—the elevation data with sinks removed—in the view window.

- Select SHALtopo —>Contributing Area. This step uses elevation data with the sinks removed to calculate the drainage area to each grid cell. A dialog box asks you to confirm the name of the elevation theme to use in the calculation. A dialog box asks if you want to confirm the elevation theme you want to use. You can either select the one displayed, or enter a short name for the grid and theme. Click OK if this is the theme you’d like to use. To use a different theme, select SHALspecial—>Define Main Themes and choose a theme from the drop-down list.

The Output Grid dialog box confirms the grid name and location; click OK. The drainage area for the landscape is now displayed in the contents window when you click the checkbox. At this point you will see two theme names displayed in the contents window: one with log 10 values (Log 10 of imab) and one with simply the grid theme name imab. The reason for this is because ArcView can only display a limited range of values, so the Log 10 of imab enables the program to display the entire range of contributing area.

WARNING: SHALSTAB uses the imab file to calculate the q/T value so do not delete this theme; simply leave the checkbox empty and it will not clutter your view window.

- Select SHALtopo —>Slope. This step uses the elevation data with the sinks removed to calculate the local average slope for each grid cell. Enter a short name for the grid and theme name in the dialog box; click OK to use the default name.

- The Output Grid dialog box confirms the name and location. Click OK.

- Now click the checkbox next to the new theme called Elslp in the contents and the slope range is shown in percent for each landscape category.

Calculating the q/T ratio

The next step is to calculate the ratio of steady-state effective precipitation (rain minus evapotranspiration) to transmissivity (the ground’s subsurface ability to convey the water downslope), or the q/T ratio. You can specify that SHALSTAB Tools calculate this ratio from either the elevation grid or from the slope and area grids.

NOTE: This function is applied to all of the landscape without consideration of whether there is material available to be mobilized as a shallow landslide or whether other processes may be more important. For example, steep valleys (say greater than 50 percent) may be mantled with colluvial fills, have a channel draining the axis or be scoured to bedrock. Clearly shallow landsliding cannot occur if there is no colluvium to slide. Steep bedrock-dominated cliffs are labeled as “chronic,” meaning that precipitation is not required to induce landsliding, although the shallow landslide mechanism modeled here may not apply to bedrock outcrops.

- Select SHALSTAB—>q/T from slope and area grids, since you have generated these themes already. A dialog box asks if you want to use the elevation theme that you’ve defined as your main theme, as well as slope theme and a/b theme. You can accept these themes, or you can change them by selecting the SHALspecial—>Define Main Theme submenu.

- The Default SHALSTAB Parameters dialog box displays two fields in which you can enter the angle of internal friction and soil bulk density. The value of the soil bulk density should be the approximate saturated bulk density and typically lies between aboout 1700 and 2000 kg/m3). We strongly recommend using 45 degrees unless you have data to suggest otherwise. We recommend 45 degrees and 1700 kg/m3 as the standard value for comparison purposes. Note that SHALSTAB Tools lacks a cohesion term and it would be inconsistent to use geotechnical data to change the friction angle (typically lowering it) without also changing the cohesion value (typically adding root strength). If you want to explore the relative influence of friction angle and cohesion on slope stability, use the next pulldown menu to run SHALSTSBco. To accept these default values, click OK.

NOTE: If you are running the SHALSTABco—>q/T tool, you can add soil depth (default of 1) and cohesion (default 2000 (n/M2) parameters to your model. Note that the assumption of spatially constant soil depth is, in general, a poor assumption and makes the performance of SHALSTABco not greatly different from that of SHALSTAB. The advantage is that one can explore the influence of root strength change on relative slope stability.

- The Check elevation units dialog box asks if your elevation units are in meters. Click OK.

- The Select Raw Elevation Theme dialog box asks you to select the raw elevation theme imslp. Click OK.

- Another dialog box confirms the theme name and grid location; in this case the theme name is Elslp and the grid location is d:\shalstab\elslp. Click Yes to continue.

- SHALSTAB now calculates the landscape stability (q/T) ratio (this may take a few seconds, please be patient). A dialog box asks you to enter a short name, imqt, for the q/T grid. The Output Grid dialog box displays the grid name and location; click OK to confirm this.

- SHALSTAB Tools calculates the landscape stability ratio and displays for this theme a range of log values (logarithmic values) from Stable to Chronic Instability. If you click on the checkbox next to this theme, you will see the map of these values drawn in the projects window.

Examining model performance

SHALSTAB Tools offers two means of examining model performance. One is to compare the distribution of q/T values for mapped landslides with that created by a biased random distribution, and the other is to compare the drainage area per cell size (a/b) and slope values for each landslide with that computed for various q/T values in the model. The first method relies on comparing the distribution functions of q/T, which are displayed as color-coded histograms and the other methods relies on plotting data on a graph with model curves.

WARNING: If you intend to run more than one SHALSTAB Tools session—for example if you plan to change parameters in the model (such as cohesion) and compare those statistics with the basic SHALSTAB analysis—you should save the data table values that generate the histograms from your original project to another program, such as Microsoft Excel. Then you will be able to further manipulate it). The table values are dBase files that can be opened by other programs, for example, Microsoft Excel, using Windows Explorer.

- Select SHALtest—>Distribution-Landscape. A dialog box asks you to confirm the theme for this calculation. You can choose the displayed theme or you can define a different one by selecting SHALspecial—>Define Main Theme. It’s important to define the themes for each test that you generate, so that you can draw comparisons between sets of data. This step then creates a histogram showing the percentage of land area in each slope stability category as well as the cumulative percentage. A statistical table showing the numeric values for these categories is also displayed.

NOTE: After generating each test, you can always return to the SHALSTAB Tools menubar, by selecting the View SHALSTAB window.

- Choose SHALtest—>Select Landslide Theme. The Select Mapped Landslides File Type dialog box is displayed with a drop-down list of two choices: Shapefile (Vector) and Grid (Raster). Select Shapefile and click OK. If you are performing this step from a previously saved project file, you need to define the slope, contributing area, and q/T themes first by using the SHALspecial—>Define Main Themes submenu.

- The Select Landslide Theme dialog box asks you to choose the name of the mapped landslide theme for which you want to generate statistics. Select the name shallow.shp from the list box and click OK. A dialog box displays the mean and median landslide areas as well as the number of records and total area of the theme.

- Select SHALtest —>Distribution-Landslide. This creates a histogram that shows the percentage of mapped landslides that are located in each stability category (in log values). A landslide is classified by its minimum q/T value; that is, the landslide takes the value of the minimum q/T cell it touches.

Now you want to generate a grid that will randomly distribute the shapes in your landscape so that you can compare the random landslide distribution against the mapped landslide distribution. Note that we call this a “biased” random model because we select the minimum q/T found under each random placement. By consistently picking the lowest q/T, we are biasing the values towards the least q/T, just as we have done for the observed landslides. While this may not seem like a very severe test, given the inaccuracies of the landslide maps, the topographic maps and the simplifying assumptions contained in the model, this test does provide a significant evaluation of the model performance.

- Select SHALtest—>Generate-Random Grid. The Default Random Validation Values dialog box displays two parameters: the number of shapes to be randomly placed (the default is equal to the number of landslides that the your landslide shapefile has), and the number of random cycles to run (the default for the cycles is not fixed but running more than one cycle will take a long time for this test). We recommend in practice that you select 10 cycles in order to generate a sufficient number of trials to examine the difference between the one landside map and the population of randomly placed landslides (see examples in Dietrich et al., 2000).  For demonstration purposes here, enter 1 for the number of random cycles and click OK.

NOTE: If you don’t have a landslide theme, you can still run the random model. In this case, the default number of shapes is 100 and the number of cycles is 1.

- A dialog box asks you to select the point-buffer radius calculating method, either Median or Mean. This refers to the average size of the radius; in general, selecting Median results in fewer instability values, because it is typically smaller than the mean. Selecting Mean uses a larger buffer radius and therefore larger shapes, ultimately resulting in a greater percentage of high instability values (because of enhanced likelihood of overlapping a low q/T value).

One random theme is displayed in the contents window, with the random points displayed as circles in the view window.

- Select SHALtest—>Distribution-Random. This generates a histogram that shows the percentage of randomly placed shapes of the minimum q/T value that fall in each stability category. A dialog box asks if you want to use the SHALSTAB theme. If not, define the theme you want to use by selecting the SHALspecial—>Define Main Themes submenu.

Creating an area-slope scatterplot

- Select SHALspecial—>Area-Slope Scatterplot. Dialog boxes will be displayed asking you to confirm the SHALSTAB, slope, and log A/B themes.

You can make scatterplots of either the entire landscape properties or the properties of just the landslides and compare them with the curves representing the SHALSTAB theory.

- Choosing the Landslide method creates the plot by selecting the landslide cell that has the minimum q/T value from each landslide, and overlapping that cell on corresponding cells in the area grid and the slope grid. The number of points in the plot should be approximately the same as the number of landslides.

- Choosing the Landscape method creates the plot in the same way except that it places random points (a default of 500) on the landscape, overlapping them on top of the area and slope grids to obtain their values, rather than starting from the minimum q/T value of each landslide.

NOTE: Placing a large number of points will greatly increase the plot processing time for this method; for example, placing 5,000 points may take as long as 20 minutes to process on a 266-MHz Pentium PC with 128 MB RAM. Given that large areas may contain more than one million points we recommend that you plot only a small subset of the values.

Note that the resulting plot will display a series of curves each labeled with its corresponding log (q/T) value and will also show the two vertical lines separating the fields of “unconditionally stable,” potentially unstable, and “unconditionally unstable.” Any site on the landscape whose a/b and slope values places it to the left of the “unconditionally unstable” line is predicted not to fail even when saturated. Any site that falls to the right of the “unconditionally unstable” line is expected to be unstable even when dryand therefore likely to be exposed bedrock. Each log (q/T) curve also defines a threshold: for a given q/T value, points plotting above the curve are predicted to be unstable whereas below the curve the points would be stable (due to insufficient effective precipitation). By comparing these curves with the landslide points one can see what value of log(q/T) places the majority of points above the line. The lower the q/T (e.g. a log (q/T) of -3.1 versus -2.5) needed to place the majority of the landslides in the unstable field (above the line), the better the performance of the model. This is because the lower the threshold q/T, the smaller the area that would be delineated as high landslide risk. Also note that the diagonal curves that start in the stable zone also delineate which elements of the landscape will be saturated for a given q/T. Any place with area-slope values that plot above the curve is predicted to be saturated for that q/T value.

The position of the vertical and curved lines in the slope-area plot will vary with parameter selection. Decreasing the friction angle moves the two vertical lines to the left (lower threshold slope values). If you use SHALSTABco, adding cohesion will move the two vertical lines to the right (higher threshold slopes) and raise the log(q/T) lines. Unless you have additional information, we recommned you use the default values.

Evaluating your results

The SHALSTAB and SHALSTABco Tools determine the spatial pattern of the hydrologic ratio, q/T, needed for slope instability at each grid cell. If you lack data on observed landslides, then it is recommended that you use the default values of q/T for hazard classification. These values are given in the Set Parameters (in the SHALspecial menu) properties for the q/T. Studies by Dietrich et al., (in review) suggest that in order to have more than 70% of the landslides occur in the high hazard category, a threshold of log (q/T) of -2.5 appears to be needed for 30 m, whereas -2.8 may be sufficient for 10 m.

The histograms (and associated statistical tables) and the graph of a/b versus slope enable you to examine the distribution of observed landslides with respect to log (q/T) values and with respect to a biased random sample. The model is considered successful if the majority of the observed landslides occur in the lower q/T categories and if the observed landslides have q/T values that are significantly different from the random calculations. The classification of relative hazard using SHALSTAB Tools can either follow a standard definition, or can be determined using knowledge of observed landslide location, economic tradeoffs resulting from hazard classification, apparent risk associated with landslides (loss of fish habitat, death to individual researcher).

In the example data set provided with the model you will find a mixed result. While the a log(q/T) value of -2.8 would successfully capture most of the landslides, a large number were also found to occur with a/b and slope values that place the landslides in the “unconditionally stable” zone. We have found that this misclassification arises mostly from poor topographic data and from inaccurate mapping. We have elected to include this example to avoid any sense that SHALSTAB Tools can overcome these problems and produce perfect results. It is recommended that where possible, the user review model performance in the field to ascertain what contributes to misclassification of hillslope stability.

As a final note, be aware that for any given storm event or limited observational period it is expected that only a fraction of those sites with values of q/T at or below the threshold line will experience a landslide. We propose that those sites that have appropriately low values of q/T but have not failed have different local conditions (for example, friction angle, cohesion, transmissivity, and subsurface flow field), different actual topography, or did not experience the same precipitation. Extensive field investigations are required in most cases to delineate which properties of individual sites that, based on topography, have a high probability of failure but because of other factors are relatively stable.

Credits

SHALSTAB Tools is a product of a collaborative effort between the Department of Geology and Geophysics (renamed Earth and Planetary Science), University of California (Berkeley) and Stillwater Sciences (Berkeley, Ca.). It was developed through a grant from the California Department of Forestry. The model’s theory is based on reasearch undertaken by Dr. William E. Dietrich. The core program was written by Dino Bellugi (Geology and Geophysics), the user interface was developed by Rafael Real de Asua (Stillwater Sciences), and the tutorial was composed by Lisa Stanziano.

While this software is made freely available to all users, copyright resides with the University of California. No portion of this software may be included in any for-profit software.