Draft Report of the

Network Architecture and Interconnection Subcommittee

6/10/2001

# Table of Contents

Table of Contents	1

Executive Summary	3

Overview	4

Introduction	6

Topologies	6

Strawman	11

Approach	15

Functional Design	18

General System Attributes and Issues	19

Data flow among elements of the ANSS	20

Distribution within nodes of the ANSS	23

Rapid Data Delivery	26

Offline or Earthquake Post Processing	31

Interface with Non-ANSS networks	31

Performance Standards	31

Operation of the System	34

Appendices	37

Appendix A - Meetings of the NAI subcommittee	37

Appendix B - Acronyms used in this document	39

Appendix C - ANSS Data Rate/Volume Estimates	41

Appendix D - ANSS Goals	43

Appendix E - ANSS Design Goals	45

Appendix F - ANSS Data Products	47

Appendix G - ANSS Design, Development, and Implementation	49

Appendix H - ANSS Total System	51

# Executive Summary

# Overview

As part of the process of developing the guidelines and standards for the establishment of the ANSS, the Technical Integration Committee formed five subcommittees to address particular aspects of the system.  These included Instrumentation, Site Installation, Data Analysis and Products, Network Architecture and Interconnection, and Archiving and Distribution.  Figure 1 illustrates the responsibilities of the various subcommittees in a functional diagram.

Figure 1:  Functional diagram of ANSS components, color-coded by the responsibility of the TIC subcommittees.  Primary elements include stations, processing centers, archiving centers, and the links among them.  The white ovals indicate ANSS users.

The subcommittee on Network Architecture and Interconnection (NAI) was charged to

Develop consensus specifications for the integrated design of national, regional, and urban monitoring components and for communications among the components (sensors, data centers, and real-time recipients of data and products).  This includes the technical design of communications protocols and data center design and functionality, but does not include specifications of data formats.  The system should include consideration of appropriate levels of timeliness, completeness, robustness, and redundancy.  The design must examine distributed data acquisition and processing, and must allow for distributed data archiving and distribution.  The capability to incorporate data from sensors and networks owned and operated by other organizations should be included.  Performance specifications for the data acquisition and exchange hardware/software system nationwide need to be identified.

The NAI subcommittee consists of core members from the seismological and engineering communities, representing both scientists and software developers.  Membership of the committee includes:

Lind Gee, UC Berkeley, Chair

Doug Given, USGS Pasadena

Steve Goldstein, NSF

Kent Lindquist, University of Alaska

Phil Maechling, Caltech (now at Cysive Inc.)

Gary Pavlis, University of Indiana

Tony Shakal, California Division of Mines and Geology

Mitch Withers, University of Memphis

As an initial member of the committee, Phil Maechling made many important contributions to the outline and structure of the report.  Although he was unable to continue participating, the committee is extremely grateful for his input.  Gary Pavlis, Doug Given, and Steve Goldstein graciously agreed to join the NAI subcommittee mid-way through the process and provided valuable input during the writing process.

In addition to these participants, the NAI subcommittee was supplemented by liaison members from the Technical Integration Committee:

Tim Ahern, IRIS

Ray Buland, USGS Golden

Steve Malone, University of Washington

The NAI subcommittee released a draft report in March, 2001.  Although the primary audience was the TIC, community input was invited in several ways.  The NAI subcommittee presented a poster at the annual meeting of the SSA in April and advertised the report on several email lists.  The NAI subcommittee began revising the report in late May.

# Introduction

## Topologies

In considering different ANSS topologies, the NAI subcommittee has come to appreciate the importance of a well-defined nomenclature.  Discussions were often complicated by differing interpretations of common terms.  In particular, the use of the term "regional" in the ANSS differs from the traditional definition in the seismic network community.

As a system, the ANSS will be comprised of various functions conducted at one or more facilities.  Examples of ANSS functions include instrumentation, acquistion, processing and archiving.  The NAI subcommittee identified five main functional activities:

Instrumentation:  The activities related to the installation, operation, and maintenance of sites where seismological instrumentation (either weak-motion, strong-motion, or both) is deployed.

Data Concentration:  The process where seismological data from multiple stations are consolidated onto high-bandwidth communications circuits.

Data Processing:  Activities related to the acquisition and processing of seismological data to produce summary parameters.  Real-time data is pushed to critical users from the processing centers.

Data Archiving:  Functions related to the archinving of seismological data and distribution of data to users.

Data Outlet:  Activities related to the interpretation of seismological data for emergency managers, the media, and the public.

Any ANSS facility may perform one or more of these functions.  ANSS facilities generally can structured into different "levels":

National:  Activities or operations pertaining to the Nation as a whole.

Regional:  Activities or operations pertaining to one of the ANSS regions.

Local:  Activities or operations in one of the participating networks in the ANSS.  The “urban” networks of Circular 1188 are included in this nomenclature.

Contributing:  Activites or operations in networks that are not an integral part of the ANSS but that exchange data with  or contribute to the ANSS in some way.

Figure 2 presents elements of the system architecture as defined by the NAI subcommittee.  Subsequent figures will draw upon the color scheme and symbology described here.

Figure 2: Elements of ANSS architecture.  The colors and symbols shown in this figure will be used to illustrate different models.

Many models of ANSS architecture may be constructed from a combination of functions and facilities.  Before considering alternatives, however, we will begin with the current system.

The status quo is illustrated in Figure 3, where there are activities at both the local and national level.  At the local level are the existing seismic networks, some partially supported by the USGS and others not.  The activities of the local networks (station operation and maintenance, data processing, information distribution, and archiving) are generally characterized by a wide-range of hardware and software solutions and are not uniform across all networks.  No “regional” activities or facilities are illustrated in Figure 3.  Although the local networks are beginning to organize along regional lines, no regional centers have been "officially designated.  The NEIC represents the current level of national activities, involving station operations, processing, and information distribution.  All of the centers which receive some USGS funding have established data exchange with NEIC and, in some cases, with other local centers.  There is no true national archiving facility at this time, although some local networks archive data at IRIS while others have established local archives.

Figure 3: In the current system, the local processing centers are characterized by a wide range of hardware and software solutions.   Functionality of the system elements is highly variable.

Although many aspects of the seismic monitoring in the United States have improved in the last 3 years, the current operations do not truly represent a true "system".  Although some components perform extremely well, others do not.  Overall, the current state of seismic monitoring lacks a coherent and integrated structure.  This is one of the major problems which the ANSS must address.

The status quo is a complex topology, which is highly heterogeneous (different hardware and software in the different components) as well as highly asymmetric (different levels in the system performing different functions).  The existing system also has some fundamental vulnerabilities.  Many of the local networks are located in areas of significant seismic risk.  Although effort has been made at some facilities to mitigate the impact of a major earthquake, this remains a problem for the ANSS.

Figures 4-6 illustrate a spectrum of possible architectures for the ANSS.  One end member is the highly centralized model illustrated in Figure 4.  This figure illustrates the ANSS as an entirely "new" system, with no carryover from the current infrastructure.  For example, all stations are standardized with modern, digital instrumentation.  In some areas, local sites might be used for data concentration in order to facilitate efficient communications.  Nearly all stations have multiple communication links.  There are two national processing centers, each with an associated long-term archive.  The model is simple, homogeneous (all ANSS nodes use the same software), and symmetric (all nodes perform the same level of processing).

Figure 4:  One end-member model for ANSS topology, illustrating a highly centralized system.   All stations send data to both processing centers, which are 100% redundant.

Figure 4 presents a very simple model.  Although appealing in some ways, it does create the problem of extremely long communication paths.  Many local network operators have emphasized the complexity of their telemetry due to the remoteness of their stations.   Although satellite communications may solve this problem in the long term, terrestrial communications are likely to remain a consideration for the ANSS for the next few years.  The long paths of this model increase the potential for problems or failure.

Many variants on the centralized model may be constructed.  For example, Figure 5 illustrates a model where the existing local network infrastructure is utilized to provide local communications hubs.  This model can be constructed so that some low-level processing is conducted at the local level (e.g., picking phases and estimating amplitudes and coda durations), with all event-based processing at the national level.  Data outlets provide local intrepretation of results for emergency response operators as well as other users of ANSS data.  This model is relatively simple and homogenous, but is asymmetric, as not all nodes in Figure 5 perform identical processing.

Figure 5:  A variant on the centralized model, with local functions such as data concentration and data outlets.

Figure 6 presents a more regional architecture for the ANSS.  In this model, local networks would coordinate to create a "regional" processing center which would be the authoritative source for earthquake information within a geographical area.  While this model provides flexibility for regional variation, it introduces complexity by expanding the number of processing centers and the need for coordination among them - and with the national centers.

Within the seismological community, there are proponents for the highly centralized models of Figure 4 and Figure 5.  These topologies minimize the number of tiers in the seismological hierarchy and therefore reduce the number of connections and interfaces.  Processing would be highly centralized and standardized, and national earthquake products would be created uniformly and available with one-stop shopping.  The issues associated with partial or incomplete views of events are reduced and the hierarchy of authority is easily defined.  Because the number of facilities is limited, expensive resources, such as technical people, are minimized.

However, there is also strong support in the community for a regional model such as Figure 6.  The highly centralized model may create a large and potentially unwieldy operation, with limited responsiveness to local and regional issues.  For some, the centralized model is a deterrent to local buy-in at the state and local level.  The highly centralized models do not necessarily build upon the existing infrastructure, which is significant in some parts of the country.  There is also some doubt that a centralized system can serve all needs.

Figure 6:  This figure illustrates a more regional model for the ANSS, with local networks coordinating to form a regional centers.

All potential models of the ANSS call for multiple processing centers.  This is required to avoid single points of failure.  Therefore all models require robust and reliable exchange of a variety of data and information products among nodes or components of the system.  The main difference between the national and regional models is the number of processing centers.  The complexity of the system grows as the number of processing centers increases.  This is particularly true if the processing centers are not clones of one another in terms of both software and input data.

An additional issue for the ANSS is the need for a smooth transition from the existing state to a new system.  The ANSS exists "now", with new stations being deployed as part of the existing local networks.  Whatever architecture is selected, it will be important to define a evolutionary path from the current situation to the final model, without interrupting operations.  This is particularly important if ANSS funding continues to grow slowly.

## Strawman

The NAI subcommittee recommends a regional, rather than a centralized, architecture for the ANSS.  The regional model provides multiple processing centers for redundancy, while allowing for customization for regional needs.  The regional model also provides a natural scale for addressing issues of data quality control, scalability of processing systems, redundant reporting, data product quality control, and flexibility and responsiveness to local contacts.  However, the NAI subcommittee does not recommend that every existing local network be transformed into a regional center.  The regional model of the ANSS implies some consolidation and coordination of effort among the participating networks.  The facilities of some local networks may become data concentrators, with processing activities moved to the regional center.  These local network operators may continue to be involved in station operation and maintenance – and may participated in the design and customization of their regional center.  These facilities are also likely to participate as ANSS data outlets.

The complexity of the regional model depends on the number of processing centers.  This is a function both of the number of regions – and of the number of centers within a region.  The NAI recommends establishing one processing center per region, but recognizes that there may be exceptions.  The NAI subcommittee also recommends that the regional processing centers either be located away from areas of significant seismic risk or that appropriate mitigation efforts be implemented to ensure operation during the maximum credible event.

To discuss the roles of different elements in the ANSS topology in more detail, we consider the “map” view of the regional hierarchy (shown in Figure 7) to discuss the different elements - and associated assumptions.

Figure 7:  A map view of the hierarchy illustrated in Figure 4.  This figure shows 4 adjoining regions with data concentrators, regional processing and in one case archiving centers, and national processing and archiving facilities.

The fundamental building block of Figure 7 is the seismographic station.  Three levels of stations - national, regional, and urban – are recognized by the ANSS.  When fully deployed, the ANSS is expected to comprise approximately 10,000 stations.  Bulk purchase agreements will allow the managers of the ANSS to impose significant standardization in the station hardware.  However, the evolution of the ANSS out of the existing system means that the ANSS must deal with “non-ANSS” systems.  The ANSS must be designed so that data from non-ANSS stations may be used if appropriate.  The existence of ANSS and non-ANSS stations imply that the ANSS will need to address a heterogeneous environment.

The lines in Figure 7 illustrate communications links.  Since seismic data are the cornerstone of the ANSS, it is likely that the consideration of system reliability will lead to the implementation of multiple communication links.  These secondary links may be an alternate path to the primary recipient or a link to another processing center.  Not all stations have multiple telemetry links but some number of critical stations will.  Figure 7 do not differentiate between continuous and on-demand communications.  Certainly many sites will require continuous telemetry, but some, such as structural strong-motion arrays, may not.

The flow of data to multiple processing centers may create potential problems in terms of data loops and for determining "authoritative" copies for archiving, so the issue of tracking the flow of waveform data will be important.  It seems unlikely, however, that all waveform data will be processed by all centers.  A more likely scenario is one where neighboring centers engage in complete - or nearly complete - waveform exchange in order to unify their view of events.

The seismographic stations will typically produce parametric data in addition to waveforms or timeseries.  The waveform data may be continuous or triggered, or a combination of both.  Triggered data present additional problems, particularly from the perspective of rapid earthquake processing, since they introduce a non-deterministic load on the system and because of delays in data delivery.  The ANSS processing system must be able to handle these issues.

Between the seismographic stations and the regional processing centers, there may be an intermediate levels.  It is likely that the need for these levels will vary from region to region.  For example, in some instances data may be collected from several stations at a single point, described here as "data concentrators".  There are two possible models for this tier.  One is that these facilities just forward the data to the next level.  In this case, there may or may not be local storage at this site (assuming that the dataloggers have local storage), but essentially, this level would be a telemetry collection point.  A second model is that some low-level processing is done at the facility and only selected waveforms are forwarded to the next level.  In this case, a system might perform subnet triggering or some type of event detection and then forward the triggered waveforms (and associated parametric data) out.  This type of model might be useful in areas where it is difficult to obtain sufficient bandwidth to transmit all of the waveform data.  These may or may not be ANSS-supported operations.  For example, this might be a local utility that has deployed strong-motion instrumentation and is willing to contribute data to the ANSS.  In another example, this might be data from a local array of stations, with a single, convenient point of outward communication.  These data concentrators are likely to play a significant role in the ANSS as it transitions from the current situation toward the long-term architecture.

The regional processing center will be the central hub of operations in an ANSS region.  As envisioned, the regional center will have primary responsibility for rapid earthquake processing and notification.  While the regional center will receive parametric data from other levels in the system - and from adjoining regions - it will be the authoritative source of information on earthquakes in its region and will have responsibility for distribution of this information.  This role in the rapid processing and distribution of earthquake information makes the regional centers critical elements of the ANSS and is the primary source complexity in the the design.  For example, this role requires high levels of robust and reliable operations from the regional centers.  The multiplicity of the processing centers raises issues in terms of unified earthquake reporting and the establishment of backup capabilities.

The regional center should also be responsible for quality control of waveform data and maintenance of the station infrastructure information.  The NAI subcommittee believes that this responsibility is best held as close to the station as possible.  The regional processing center may or may not have a local archive, but will contribute data to the ANSS archiving facilities.  In order to facilitate data access, data should flow from the processing centers to the archiving facilities in near-real time.

Figure 6 and Figure 7 illustrate a model with multiple regional processing centers and a single national processing center.  In a change from the conventions established by the CNSS, the ANSS regions are fully adjoining, but do not, in general, cover seismicity outside of the U.S. borders.  The U.S. has a national interest in monitoring global seismicity and the NAI subcommittee believes that the national center should be responsible for monitoring earthquakes on a global scale.

USGS Circular 1188 identified the national center as contributing to the reliability of the system by acting as a “backup” to the regional processing centers.  The reliability of the system also requires that one or more of the regional centers have the capability to “backup” the national center.  The question of how to implement robust delivery of rapid earthquake products has some complexity.  Although the ANSS could implement a “nearest-neighbor” regional backup model, having the national center act as the backup to each region is the simplest model.  However, there are a number of issues related to this question.  For example, must the national processing center reproduce the full capabilities of each regional center?  Must the regional centers reproduce the full capabilities of national center in terms of global monitoring?  What are the implications for data flow?

Rapid notifications require rapid review, in order to confirm their validity or to remove spurious events.  It is anticipated that each operational processing center will require having staff on call at all times, as is the current norm in many parts of the country – with the expectation that earthquake alarms will be reviewed within a few minutes after notification.  Although the acquisition of ANSS hardware and the development of software are expensive, in the long run, the most expensive component of the ANSS is the time of experienced staff.  Thus, although it does not seem cost effective to require 7x24 staffing at all regional centers, it is reasonable to support this at the national processing facility.

In the earlier discussion of responsibilities, the role of the regional centers in automated earthquake processing and rapid notification was identified.  The need for rapid review and revised or post-earthquake review was also identified.  Naturally, the rapid review would be performed at the center that produced the alarm (with potentially related activity in neighboring regions and the national center).  However, the question of  revised review – and the construction of reviewed earthquake products is less clear cut.

The generation of reviewed earthquake products could be performed at either the regional processing centers, the national processing centers or the archive centers.  Strictly speaking, the system architecture is relatively insensitive to the location of this activity.  However, the NAI subcommittee recommends that routine analysis and review be implemented at the processing level, rather than the archiving level.  Although some aspects of design are simplified if this is a centralized process in the national processing center, the NAI subcommittee further believes that each regional center should be responsible for its earthquake products.  If this decentralized model of product generation is adopted, the ANSS will require mechanisms to synchronize earthquake information among the regional and national centers.

There are several implications to this recommendation.  The national center should have the responsibility to construct a seismicity catalog that is a superset of the regional and global events.  It may also be appropriate for the national center to determine a uniform set of earthquake products, based on its own processing.  The national center should serve as an arbiter among the regions in resolving divergent views of the same earthquake.  Appropriate business rules and mechanisms will be required to keep the national and regional catalogs synchronized.

The archiving function of the ANSS may be handled in several different ways.  Each regional processing center could have an accompanying archiving facility.  This would lead to a distributed system, which complicates uniform and simplified access to ANSS data.  In contrast, the archiving function could be centralized.  At the very least, the design of the ANSS archiving facilities must address the issue of redundancy.  In either case, the repercussions for the scope of the NAI subcommittee are the same:  data need to be provided to the archiving facilities in a timely fashion.

An issue related to review and analysis is the question of when responsibility migrates from the individual processing center to the archiving system.  Is the “authoritative” information on an event always located at the regional processing center – and the information at the archive always a copy?  Certainly in the initial hours and days after an event, the authoritative information rests with the processing center.  But there is a natural time scale at which point the archiving center hosts the authoritative information.  Revisions after that point would have to be “checked out” (like a library book) from the archiving center.  This transition does not change the responsibility of a regional center for a particular product, rather, it changes the location of the authoritative information.

An important component of Figure 7 is the ANSS data outlets.  The TIC has identified seven categories of users:  global seismologist, regional seismologist, seismic hazard analyst, engineering seismologist, earthquake engineer, emergency management responder, and media/public officials/educators/general public.  Another important user is the researcher or practitioner who serves as a local point of contact for the media, public, and others.  Although not identified in Figure 1, the local "data outlet" is a concept that should be considered in the design of the ANSS.  An example might be a seismology professor who provides advice to local emergency managers during an earthquake sequence.  From the perspective of the NAI subcommittee, the paramount issue with respect to users is their need for time-critical information.  As described later, the NAI subcommittee believes that it is important to differentiate between users with real-time needs (such as emergency managers) and "others".  While "other" users may desire rapid access to data, we believe that there is value to identifying the core group who require real-time access.  This differentiation is important from the perspective of putting predictable loads on the processing centers.  This user group will drive some aspects of data delivery mechanisms because of the time-critical (and life-safety) nature of their application.

Finally, the linkages – or pipes – among the components of the ANSS are important elements of the system.  Figure 6 and Figure 7 show considerable complexity in the connections among the elements - the ANSS will require both horizontal and vertical connectivity.  The further up in the hierarchy, the more connectivity is required.  Certainly a great deal of connectivity is anticipated among the regional centers and between the regional centers and the national centers.

The size of the connections has been the subject of some discussion.  Estimates range from 50 Mbps in the worst case to 16 Mbps in the best, with differences due to assumptions about compression and continuous versus triggered data (for a discussion of this issue, please see Appendix C).  In general, this problem will be largely solved by the rapidly evolving technology in networking.  Where we still face limitations is in the links from the station to the processing center and at the archive.  Although the communications field is evolving rapidly, the choices in remote areas, where many seismographic stations are located, are limited, often forcing choices between data latency and data completeness.  In some sense, this is an expression of the digital divide, where rural seismic sites have limited communications options.  Similarly, although storage technology is expanding quickly, some of the "worst case" numbers may stress current capabilities.

## Approach

Figure 1 and Figure 6 were used to structure the thinking of the NAI subcommittee.  Since Figure 1 emphasizes the role of the NAI subcommittee in forming the linkages among the many components of the ANSS, we viewed ourselves as the "pipe" committee and focused on several key questions:

Where do the pipes go?

What is in them?

What are they made of?

What is at either end?

In order to address these issues, we drew some lines and made some assumptions.

- We assume that the ANSS will be divided into a number of regions, somewhere between 2 and 10, with the potential for more than one processing center within a region.  The regional centers are assumed to be responsible for processing - and reporting - on earthquakes in their geographical area.  The regions are assumed to adjoin, that is, all states fall within a region.  This differs from the current model of the Council of the National Seismic System (CNSS), where some areas of the United States do not fall within the boundaries of a network.  This is a particularly critical assumption, as multi-source reporting adds complexity to the design.

- We assume that the ANSS will have some number of long-term archives, somewhere between 2 and the maximum number of regions.  We assume that the long-term archive may be decoupled from the real-time processing center.  The NAI subcommittee views archiving as a separable function that does not require collocation with the processing centers, although there may be other reasons to do so.  In particular, while the processing centers will send real-time data (waveforms and parametric information) to the archives, the processing centers will not rely on storage capacities at the archives for access to these real-time data.

- The ANSS must provide mechanisms for distribution of earthquake information and products to a wide variety of users over a broad range of time scales.  In particular, it should be a goal of the ANSS that the archiving system be designed to distribute earthquake information products in near-real time.  However, the NAI subcommittee feels that the distribution of real-time and near-real time data is not a mission-critical responsibility of the archiving centers.  The regional and national processing centers have the responsibility to provide earthquake information to time-critical users, while the archiving facilities distribute information to non-time-critical users.  Furthermore, the system should be designed so that the processing centers can fulfill their critical functions without access to the archiving components of the ANSS.  That is, failure of the archiving system or failure of communications between the processing and archiving systems cannot compromise the delivery of rapid earthquake notifications.

- Many of the constraints on the ANSS derive from the expected products.  For example, early warning represents one end of the rapid notification spectrum that could drive many of the design issues.  On the other hand, if the ANSS is to provide continuous waveform data from a subset of stations as an archived product, this has significant implications on design.  In many ways, early warning and continuous waveform archives place contradictory demands on system design.  This issue will require particular attention as plans for the ANSS move forward.

- On the particular issue of early warning, the NAI subcommittee recommends that the ANSS adopt the perspective of continuous performance improvement versus early warning.  That is, focus on  generating information as quickly as possible without specifically identifying their most rapid notifications as early warnings.

- While system security is important, the NAI subcommittee agreed that CTBT-level security (digital signatures, for example) is not required for the operation of the ANSS.

- The NAI subcommittee makes no distinction between weak and strong motion data.  From the perspective of the pipes, a waveform is a waveform is a waveform.  Network design goals for real-time strong-motion stations are in principle the same as for other seismic instrumentation.  However, the majority of triggered data in the ANSS is likely to be generated from strong-motion sites and this has implications for system design.  The primary implication is that the ANSS must be designed to handle late data.  The NAI subcommittee assumes that the Data Analysis and Products subcommittee will address the issues of specialized strong-motion products, noting only that this will require additional processing modules.

- Responsibility for station quality control is assumed to be the responsibility of the local or regional authority.  Issues such as calibration, instrument response, and clock quality are best addressed as close to the station as possible.  The NAI assumes that the local/regional authority are the source of the infrastructure information and quality control.

- One of the issues we struggled with is "What is the ANSS?"  Is it a totally new system comprised of new hardware and software?  The ANSS budget does not seem to allow for the development of an entirely “new” system.  A likely path for the development of the ANSS is an evolution from existing systems, with new components developed to address specific problems and enhance capabilities.  This has implications for the level of homogeneity within the system and raises the question of what degree of integration will be achieved.

- The NAI subcommittee has a diversity of views on the issue of standardization, particularly as it relates to software.  One model for the ANSS holds that software will be identical throughout the system.  At the other end of the spectrum is the model where software is compatible through the use of standardized protocols and interfaces, but is not identical.  In between these extreme views is a model where much of the software is largely standardized, but local customization is allowed.  Regional processing center A might choose to use location module X while processing center B uses module Y.  It is clear that no one on the subcommittee recommends the identical software solution.  However, there is a difference of opinion about the degree of acceptable heterogeneity within the NAI subcommittee.

- To address the issue of heterogeneity, the NAI subcommittee agrees that the use of well-designed APIs will allow the ANSS address a number of issues, including compatibility with many different dataloggers; compatibility with many common real-time data servers; real-time support for data-processing, archiving, and distribution; network transparency; independence from particular data formats; compatibility with common archive formats; and compatibility with commonly used communications protocols.

- We encourage the ANSS management and development groups to consider the ANSS as a system.  A system is "hardware, software, and people working together to solve a particular problem, or to produce a desired effect."  We feel that all components of the “system” need to be considered in order to produce a satisfactory system.  The systems approach implies there is some balance between these areas.

- The specifications for the ANSS will evolve as the system develops.  We recommend establishing a mechanism to allow changes and evolution in the technical and functional specifications as prototype efforts are evaluated and improved.

# Functional Design

To address the issues of functional design for the ANSS, we have organized this section into several subsections:

General System Attributes

Distribution among nodes

Distribution within nodes

Data Processing

Rapid data delivery

Interface with Non-ANSS networks

Performance Standards

Operation of the system

To facilitate the discussion of these issues, Figure 8 illustrates ANSS design in greater detail.  This figure uses the symbology of Figure 2, but does not attempt to identify different levels of functionality.   for example, a processing facililty (square) includes components for data acquistion, "online" or automated processing, "offline" or post-processing, and quality control.  More prose here.

Figure 8:  Schematic illustration of ANSS functions and data flow among ANSS elements.

## General System Attributes and Issues

### Evolution

The ANSS is being designed at a time when technology is changing very rapidly.  From telecommunications to computational capabilities to networking to storage capacity, the technological underpinnings of the ANSS will change dramatically over its lifespan.  It is essential that the ANSS be developed so that it can grow and evolve over time, both to take advantage of these changes as well as to respond to changes in the needs of the user community.  In order to evolve, it is critical that the ANSS system be based on an open and extensible architecture.

### Standard Interfaces, Protocols, and Formats

The ANSS should use standard networking techniques including TCP, IP, domain name servers, and route determination using commercial standard routing protocols.  It would be a mistake for the ANSS engineering staff to take on the design of replacement networking techniques as well as designing earthquake-monitoring systems.

In addition, all data flow, whether within or between elements of the ANSS, should use the same interfaces, protocols, and formats.  All data access functionality must be provided to data clients via a standard, well documented Application Programming Interface (API).  This approach will reduce costs, and increase flexibility, scalability, and maintainability.

### Modularity

The ANSS software and hardware components should be highly modular.  That is, each function in a system should be encapsulated in a separate module and each module should operate independently.  While this design principle may not be possible to adhere to in all cases, it has a number of advantages.  In particular, it allows new functionality to be added without compromising existing functionality.

### Scalability

The ANSS must be a scalable.  The current proposals for the ANSS call for approximately ten thousand monitoring sites.  However, given recent advances in micro-electronics, this number could increase by several orders of magnitude as small sensors are distributed widely and used to monitor buildings, bridges, and lifeline networks.

Although many developers approach the scalabilty issue by solving the problem with hardware by buying additional machines or bigger machines, the ANSS must also address scalability in software design.  In analyzing scalability issues, one should recognize that functionality (number of fancy features in the system) and performance tend to be inversely related.  For scalability reasons, therefore, it may be advantageous to have the system communication substrate avoid lots of extra fancy processing.  Wherever possible, it is good to heavily leverage the capabilities of existing communications protocols and operating systems features.  For example, this could mean relying on the checksums and packet-dropout detection of TCP/IP where possible, or relying on operating system features such as threading as much as possible for multitasking.

### Security

The ANSS must be a secure system.  For example, the ANSS must ensure that critical computers for data acquisition, processing, and archiving are secure against intrusion, denial of serivce attack, and “hacking”.  Rapid notifications should also be protected against "spoofing".  However, the NAI subcommittee does not recommend the level of security required for the CTBT (a 16-character random serial number and a 16-character password with encryption is required to access to the datalogger).

### Health and Status Monitoring

The ability to measure the health and status of the ANSS will be a critical aspect of the design.  The operators of the ANSS need to identify problems or failures in the system, ranging from loss of data due to communication problems to the failure of processing computers or equipment.

We anticipate that the implementation of a monitoring system will include elements such as the assessment of sensor health, connectivity to sensors, communication between and among tiers, data latencies, status of processing elements, status of computers, routers, and other hardware, and ability to perform data distribution.  In addition to monitoring individual elements, it is necessary to provide “total” assessments of ANSS components, such as a local or regional processing center.  The ability to evaluate a component’s capabilities will be critical to the success of the ANSS.

### Ease of Operation

Experience operating large seismic networks shows that substantial time and effort is required to operate these seismic networks.  The operational personnel needs of these networks can become a limiting factor on the size of networks that can managed.  This situation can be alleviated somewhat by developing system which are easier to operate.  The ANSS development should have significant effort devoted to making the system easy to operate.  Issues that should be addressed include configuration, initialization, and reconfiguration of the system, capabilities for offline testing, alternate systems for maintenance, diagnostic tools, operational procedures, documentation, and knowledge transfer.

### System Reliability

The simplest statement of the ultimate reliability specification of the ANSS is that a total system failure during any significant event is unacceptable.  In practical terms this means that enough of the ANSS must be operational to provide acceptable quality information about any possible potentially damaging event in the U.S. throughout the life of the system.  On the other hand, it should be expected that the full performance of the system in terms of response time and accuracy should be available a very high percentage of the time.  This is inevitably a cost-performance issue.  100% availability is, in practice, impossible to guarantee (e.g., against "acts of God") and the cost of closely approaching this ideal grows exponentially.  This is the best single argument for multiple, geographically isolated, data processing centers.  Finally, it seems obvious that the goal should be for the ANSS to function at the highest possible level possible under any possible circumstances.  That is, performance should degrade gracefully as components of the system fail.

## Data flow among elements of the ANSS

### Standardized interfaces

Because of the decentralized nature of the ANSS, data exchange among the operational components is essential.  A wide variety of data products are likely to be exchanged, but they fall into several distinct classes:

continuous waveforms (with triggered waveforms as a special case)

discrete parameters (such as picks, hypocenters, moment tensors, station information)

binary, large objects (a.k.a. blobs, such as ShakeMaps).

The ANSS must be able to handle these three fundamental product classes through the establishment of standardized interfaces for data exchange.  In addition to supporting these data classes, the ANSS will require two types of exchange protocols.  The first can be described as a standing order request, while the second is a discrete request.  An example of the standing order request might be the exchange of continuous waveform data between processing centers.  The exchange of associated station information, such as coordinates and instrument response, would also be considered a standing order – but of the parametric class.  In contrast, a request for a particular time segment, say, to fill in a data gap, would be an example of a discrete request.  Necessary attributes of the data exchange protocols are that they be robust and reliable.  Possible implementation considerations are the ability to prioritize data and allow for graceful degradation, using heartbeats and handshaking to insure completeness, error detection to avoid corrupted packets, and filtering to allow selection of particular data products.

### Data volume

An important issue for the design of the ANSS is sizing of pipes for data flow.  This is a particular issue for communication from the data logger to the acquisition site.  Something here about compression, completeness, and catching up

### Data delivery speed

The requirements of early warning and calculation of real-time location and magnitude dictate that data delivery delay (latency) be minimized.  The data delivery system should add no latency to that already caused by delivery from the data source.  Realistically, some additional latency will be caused by any store-and-forward approach.  A data packet cannot be resent until it is fully received.  Additional latency can be minimized, however, by designing data buffers to operate at memory speeds and by minimizing the number of  "hops" in the data path.

### Lineage

The ANSS must be able to track the lineage or provenance of its data.  Given the potential for distributed data acquistion and exchange among multiple components, tracing data as it flows through the system will be important.  In addition to tracking the path, the lineage tag also assures the data provider of appropriate attribution.  Although the complete solution is beyond the scope of this document, lineage tracking should at least include identification of the initial source of the data as well as the ability to indicate any processing or modification of the data and the agency that performed those operations.

### Buffering

The system must be capable of error free transmission of seismic data between all nodes of the system.  This mean sufficient buffering must exist at all nodes to guarantee at least one week can pass after a link goes down before any data is dropped.  When a link is broken, mechanisms must exist to pick up the transmission where it left off without overlap and without dropping loss of any data.

## Distribution within nodes of the ANSS

The requirements for data flow within an ANSS processing center are very similar to those for communication among centers and from centers to users.  Data requests are of two types: requests for continuous data streams or discrete requests for data satisfying some criteria.  Either of these request types may be for waveforms, parameters, or blobs.

Data flow within a center may be within a single computer or among many different computers running different operating systems.  This creates a requirement of platform independence for the system.  These systems may be in the same room or geographically separated.  Therefore, all data flow, whether within or between centers, should use the same interfaces, protocols, and formats where feasible.  All data access functionality must be provided to data clients via a standard, well documented Application Programming Interface (API).  This approach will reduce costs, and increase flexibility, scalability, and maintainability.

However, there are requirements specific to a data delivery system within a center These requirements are controlled by the processing function or functions with the most extreme demands for data volume, data delivery speed, and buffering (data persistence).

Data volume - The total data volume within any center attempting to process the entire ANSS data set would be greater than the volume of data transferred between any two nodes.  The total data volume of a system is determined by waveform data, which is orders-of-magnitude greater than the volume of parametric data types.  Therefor, the data transport system must be capable of handling all 24,600 channels of the ANSS system (Appendix C).  The design should also allow for future expansion.  That capacity may be achieved by dividing processing tasks among many instances of the same processing module; each handling a subset of the total data set.  This modular "divide and conquer" approach, if correctly designed, can provide open-ended scalability.

Data delivery speed - The requirements of early warning and calculation of real-time location and magnitude dictate that data delivery delay (latency) be minimized.  The data delivery system should add no latency to that already caused by delivery from the data source.  Realistically, some additional latency will be caused by any store-and-forward approach.  A data packet cannot be resent until it is fully received.  Additional latency can be minimized, however, by designing data buffers to operate at memory speeds and by minimizing the number of  "hops" in the data path.

Buffering - The system must buffer the data long enough for client applications to have a reasonable chance of successfully getting the data they need to do their work.  Data buffering also allows out-of-order data to be unscrambled before delivery.

If data buffering is not provided within the data transport system, each application will be forced to provide its own.  This makes the programmer's task more difficult and causes duplication of the data into many, application-specific buffers.

A two-level buffer approach has been successful in some existing systems.  The first level is a buffer containing from seconds to minutes of data in a fast memory resident buffer.  This serves real-time applications where low latency is the primary constraint.  The second level is a disk-based buffer that has a greater latency but greater persistence (hours to days) and serves applications that value persistence over speed.  The API can be designed so that this two-buffer implementation is hidden behind a single interface.

Application-specific buffers may be unavoidable in some exceptional cases.  If they are needed, they should be implemented with a standard API with the same look-and-feel as the one used to interact with the ANSS data transport system.

Some applications, particularly those that are not time-critical, may benefit by accessing data from a distributed database rather than from the real-time transport and buffering system.  The feasibility of this approach would depend on the requirements of the application and the timeliness and completeness of such a distributed database.

Other requirements

Standard API's - All interfaces, protocols and formats should be the same as those used between centers.  If specialized interfaces are needed to meet special requirements of processing applications, these interfaces should be implemented as a single, simple, well-documented API.

Security and client management - The data transport system should manage clients to assure predictable behavior and guarantee service to mission critical clients.  Authentication should be used to allow data service only to authorized clients.

System management - The data transfer components should be simple to configure, run and monitor.  Tools should be provided to report and monitor the performance and load of the system.

System topology - The system topology should be flexible.  It should be possible to distribute modular functions over multiple hosts and they should be free to move from host to host.  There should be no single-point-of-failure.  The system should be distributed across multiple host systems.  All critical functions should be offered by redundant servers.  Switching from one server to an alternate should be simple, fast and, where possible, automatic.  The system should be able to handle additional data volume by adding modules or hosts.  Dynamic load balancing would enhance the robustness of the system.

Stable load - Where possible, the behavior and resource consumption of the system should be steady and predictable.  Seismic events, telemetry "glitches" or other events should not introduce spikes in the data flow.

Graceful degradation - Components should fail only under exceptional circumstances.  When they do, they should fail gracefully, produce error analysis information and provide alarms to alert operators of problems.  Priority schemes should be used to insure mission critical clients receive the most important data when bandwidth degrades.

Exception handling - Exception handling must be part of the API.  The API must fix or report corrupted, out-of-order, or incomplete data.  It must allow clients to specify and handle time-outs for data requests.

Late and Out-of-order Data - The data delivery system must accommodate both late and out-of-order data delivery.  Data of this type includes triggered data, data retrieved via dial-up systems, and data delivered over slow or "lossy" telemetry paths.

Data Clients and the API

Data clients include components of the data distribution system, real-time processing modules, post-processing modules, quality control tasks, and transfer to the archive.  Data requests to the system fall into two general categories:

- Requests for continuous real-time data streams of waveforms or parameters

- Discrete requests for data satisfying some criteria; for example, waveform segments, parameters for a given time period, channel response data, etc.

In both cases the system must allow the client to specify what type of data is to be sent.  However, data servers should be stateless, that is, they should retain no memory of previous connections.  Each new connection must negotiate the data stream.

The number of client modules in an operating center may be large.  If data transport is achieved with point-to-point connections the data bandwidth required by the whole system will rise arithmetically.  For example, given ten waveform clients (pickers, amplitude calculators, QC modules, etc) each waveform stream would cross the transport system ten times.  This strain on resources tends to force designers to lump many functions into each module to minimize redundant traffic.  This pressure to lump functions is contrary to the design goal of modularity and makes development and maintenance difficult and costly.  It also limits the scalability of the whole system.

A possible solution to this one-to-many dilemma is to multicast the data.  A multicast packet traversed the network only once but can be read by multiple hosts.  They have no impact on non-listening hosts.  This reduces the bandwidth requirement and can reduce latency.  Only one process per host can listen to a multicast address.  Therefore, to support multiple clients on a single host would require either multiple network interfaces or a common buffer on the host populated by a multicast listener.  Some commercial messaging oriented middleware (MOM) products implement this second approach.  Use of commercial, off-the-shelf products could accelerate the development process at the risk of binding the system to a single vendor's product.

## Rapid Data Delivery

This section addresses ANSS rapid data distribution, which is distinguished from data distribution from the archiving system primarily because of the time-critical need for the data.

Rapid earthquake notifications will be one of the ANSS’s most important responsibilities – and most valuable products.  The ANSS should be developed so to support a wide variety of rapid notification products.  The system should be capable of rapidly determining many types of earthquake information, and each information type must be delivered in standard formats via multiple mechanisms.  The ANSS rapid notification system should be designed to be fast, accurate, reliable, robust, and flexible.  These and other characteristics of the rapid notification system are discussed below.

### Speed

The ANSS earthquake processing must be fast.  We recognize that regardless of how fast the system is, someone will want the information faster.  The goal for the ANSS should be to support continuous improvements in speed of production over the life of the system and to avoid building in systematic delays.

There will always be requests for faster production of rapid notifications.  Interest in earthquake early warning keeps constant pressure on system designers to produce information quickly.  Indeed, USGS Circular 1188 calls for "where feasible, for sites at a distance, broadcast an early warning seconds before strong shaking arrives."

One approach to the issue of earthquake early warning is to adopt the strategy of generating information as quickly as possible without specifically identifying the most rapid notifications as early warnings.  While this distinction -- continuous performance improvement versus early warning -- may seem semantic, it does provide a reasonable development path for the ANSS.  The ANSS should be designed so that the system produces and distributes earthquake information as quickly as it is determined.  The system must allow for improvements in processing capability that accelerate rapid notifications.  When the notifications are fast enough, early warning will be available.

### Precision

The ANSS should provide highly precise information.  The ANSS designers should evaluate the standard precision of the essential earthquake parameters and meet or exceed the current practices.  We anticipate that users will ask for more and more precision in the future.  Design in the precision now.

### Accuracy

The accuracy of the information provided by the ANSS will depend on many aspects of the system.  The ANSS should be designed using the assumption that the accuracy of the information it produces will improve during the system use.   Accuracy and speed are often a tradeoff.  Users will often accept lower accuracy for higher speed.  The system should provide accuracy information along with each data product.   Also, the system should give users options when more than one source of information is available, identifying the speed and accuracy tradeoffs when applicable.

### Best Answer

It has been proposed that the ANSS produce one answer for any earthquake in the U.S.  This goal is motivated by the desire to speak with one voice.  With multiple processing centers producing earthquake information, implementing the best-answer approach will be challenging (particularly since region boundaries may cross seismic zones).  However, the trouble involved in explaining multiple answers is so great that the goal of producing one answer for any event is highly desirable.  The NAI subcommittee recommends that the ANSS be designed to provide one answer.

### Improvement with Time

A fundamental principle of the ANSS should be that information about an earthquake improves with time.  Behind this principle is the idea that as time passes, the acquisition system acquires more data to work with and the processing system has more time to process the data.  Given more processing time, more sophisticated algorithms can be used.  Given more time, human interaction with the data is possible, allowing even more improvements in information content.

The ANSS should be designed to support the principle that earthquake information improves with time.  While this is a significant design issue, certain principles will be part of the solution.  One such principle is that information should be uniquely identified.  The standard example of this is event id.  By labeling parametric information with a unique identifier, we distinguish between new data items, and updates.  In addition, data items should be versioned.  We need to be able to identify updates, and the order in which data items were produced.  Finally, data that has been human reviewed should be identified including the human's identity, and possibly the time and date of the review.  The ANSS alarming system should support version information on all data that it distributes.  When information is updated, the updates should take precedence.

### Quality Flags

All data distributed by the ANSS must include an associated assessment of quality, as defined by a national standard.  The assessment of quality will naturally vary by data type.  Within ANSS, data quality tags are essential to the distributed model of the ANSS for decisions such as the "one answer".  For product delivery, the quality tags are needed to provide recipients with a measure of how the information can and can not be used.  When products improve with time, quality measures become particularly important.  These measures may also be used as flags for some users who are interested in only the highest quality information.

### Reliability

It is very important that the ANSS rapid notification system be highly reliable.  By reliability we mean that the system continues to run without problems.  The emergency response users of the ANSS, in particular, need fast and reliable information.  The reliability of rapid notification systems during large events is not too good.  One reason for this is that large events are rare and it is difficult to simulate the conditions that the system experiences during a large event.  Reliability for large events requires not only the outstanding system design and implementation but also expert system operation.  To achieve reliability, good design, good implementation, system testing, and competent operation are all important.

### Robustness

The ANSS rapid notification system should be robust.  Robustness implies an ability to continue to work when there are problems.  For example, the ANSS rapid notification system may be implemented using a primary and a backup processing system.  This would add robustness by providing information from the backup system if the primary system fails.  Robustness in the ANSS can be a costly issue.  Robustness can often be purchased by duplicating hardware, or communications systems.  However, the costs for duplicate system are often prohibitive.  The ANSS designers should present robustness options to the ANSS management, and they should identify the costs associated with each level of robustness.  The ANSS management can then make the cost/benefit decisions necessary while building the ANSS.

Another level of robustness can be achieved by designating an external “backup”.  For example, various scenarios can be constructed which would impact an entire processing facility – not just its primary processing system.  In order to avoid this failure point, the ANSS design must include rules for designating alternate reporting sources.  In a truly regional system, this could be implemented through a nearest-neighbor approach.  However, in the quasi-regional model proposed here, having the regional centers “fail-up” to the national center may be more simple to implement.  However, the design must also consider who will act as the backup for the national center?

### Flexibility

The ANSS should support many distribution methods for its rapid notifications.  As earthquake information is produced, it must be delivered to users.  At this time, delivery may involve email, paging, posting to web sites, and printing data sheets.

The ANSS should provide significant flexibility in the delivery methods it supports.  The system should be designed with the understanding that delivery methods will change.  The system should not be designed around a single delivery method, such as the Web, because that delivery method may be obsolete by the time the system is fielded.

The National Weather Service has an Emergency Managers Weather Information Network program that uses multiple distribution mechanisms.  This program could be used as a model to provide guidance in developing the ANSS distribution system.  For some applications and products it may be appropriate to leverage the EMWIN and inject products directly into the extant NWS distribution system.  By employing multiple broadcast mechanisms the potential audience is widened and perhaps more importantly, robustness and reliability are enhanced.

### State of Health

The system should have the capability to be excersized to ensure periodic end to end operation.  During times of low activity, mechanisms need to be employed to assure both ANSS operators and users that the system is functioning despite lack of apparent activity.   When broadcasts are made, ANSS operators need positive confirmation that the message was delivered.

## Offline or Earthquake Post Processing

The ANSS must support multiple levels of product review and analysis.  Although "review" is probably a continuum process, it is possible to distinguish two types.  The first is the rapid review of alarm events, while the second is the typical "post-earthquake" review and analysis.

Rapid review is performed immediately following an alarm and must be completed within a few tens of minutes after the origin time of the earthquake, with the goal of insuring the completeness and correctness of the rapid notifications.  "Post-earthquake" review is typically completed between tens of minutes to days to weeks (in some exceptional cases), with the goal of improving the automatic solution or including additional data.  The continuum between the two types is the update of products as additional information becomes available.  While the two types may have identical systems and procedures, it is possible that they may not.  While the rapid review may be based on the real-time buffers (as described above), the revised review may be based on the intermediate to long-term buffers.  In both types of review, it is necessary to insure that updated information is distributed to ANSS processing and archiving nodes as well as to subscribing users as needed.

## Interface with Non-ANSS networks

As stated in the [February 2001 draft of the] ANSS management plan, "The basic building blocks for implementation of the ANSS are existing regional and national seismic networks and data centers.  These elements are in place and functioning today, and it is logical to base the implementation of the ANSS on this foundation.  "  Also, as acknowledged in the management plan, while the USGS operates some of the existing regional networks, others are supported by but not exclusively operated by the USGS.  Also, there are approximately another 20 seismic networks operated by individual States and other Federal agencies.  These external resources provide a critical asset to integrated seismic monitoring that cannot be ignored by ANSS.  Building ANSS up from the existing monitoring efforts is the responsibility of the ANSS.  In terms of communications strategies and protocols, this means that ANSS will need to provide waveform and parametric data-exchange interfaces or interface solutions (e.g., APIs) that are compatible with the technologies of monitoring efforts external to ANSS.  This is within reason even with existing technology.

## Performance Standards

It will be critical for the ANSS to establish performance standards.  Regardless of the ultimate architecture, the ANSS must define appropriate standards for operation.

The NAI subcommittee has been urged to quantify some aspects of the ANSS.  We recognize that some of these aspects lie outside of the scope of the subcommittee, but we present these numbers to generate discussion about appropriate performance standards.  A broader dialogue is necessary to formulate appropriate performance standards for the ANSS.  This is an important process and should be initiated soon.  It is particularly critical if the ANSS is designed as a regionalized system.

We have addressed those issues that seemed most relevant to the NAI subcommittee.  Other tables might be constructed for topics such as station specifications (backup power for 3 days, onsite recording capability; no single point of failure in telecommunications for more than 10% of the stations, changes in station infrastructure information reported within 24 hours, etc.); data products (earthquake catalog complete to M1.8; human response to earthquakes greater than M3.5, ShakeMaps for events of M4.0 and higher, etc.) or for data distribution (Web servers capable of rendering 50,000 hits per minute).

We have broken this preliminary effort to quantify performance into three sections: data acquisition, data processing, and data availability.

| Data acquisition |  |  |
| --- | --- | --- |
| Item | Latency range | Comments |
| Continuous data acquisition | < 1 sec - 10s of sec | Different telemetry systems have different latencies. Overall, it is probably best to specify a range. Latency also depends on the data rate, with lower rates having longer latencies. |
| Triggered data acquisition | 10 sec - 10s of min | The "call out" systems can be variable in response. Current systems depend on local phone lines, which will lack deterministic response during a disruptive earthquake. |
| Real-time processing access to waveform data | 1-2 sec after acquisition | This timetable may be ambitious but the goal of the design should be to minimize delays in accessing waveform data. |
| Data archive access to waveform data | 30 sec to several minutes after acquistion | Data should be available to the archiving facilities as soon as possible. In some cases, slight delays may be imposed in order to provide in-order data. |

| Data processing |  |  |
| --- | --- | --- |
| Product | Latency range | Comments |
| Preliminary automatic hypocenter | 30 sec | The automatic determination of the hypocenter will depend on the location of the earthquake with respect to the network. Shorter time frames will be available in areas with denser instrumentation |
| Updated automatic hypocenter; Preliminary automatic magnitude | 60 sec - 120 sec | Updated estimates of the hypocenter should be issued within 1-2 minutes. An automatic estimate of the magnitude should be available on the same time scale. |
| Preliminary automatic ShakeMap | 3 - 5 min | Preliminary maps of strong ground shaking should be issued within a few minutes following a qualifying event. As with hypocenters, this will depend somewhat on the distribution of stations. Depending on the location of the event, some maps may depend more on predicted motions than on observed data. |
| Updated automatic magnitude; Preliminary automatic seismic moment tensor | 5 - 6 min | Updated automatic magnitude estimates will be issued for some events as more stations become available or as additional processing is completed. |
| Rapid review | 5 - 10 min after notification | Alarm events will require rapid review within a few minutes after notification. |
| Revised review | within 72 hours |  |

The numbers in this Table are applicable to normal system operation.  If a component fails (say, if one of the regional processing centers is crippled), then the latencies are naturally longer to allow for the propagation of seismic waves to more distant stations.

| Data availability |  |  |
| --- | --- | --- |
| Item | Performance | Comments |
| Completeness of available waveform data | 95% | This bound applies to operational stations, for example, and not to stations with hardware failures. |
| Station uptime | 95% | The goal for the CTBT is in the range of 95-98%. Based on previous seismological experience, this may be an ambitious goal. |

As noted in the introduction, aspects of the system architecture are constrained by the products.  In this report, we have focused almost exclusively on issues related rapid earthquake notification.  What we have not addressed are the ramifications of products such as an archive of continuous waveform data.

## Operation of the System

Experience operating large seismic networks shows that substantial time and effort is required to operate these seismic networks.  The operational needs of these networks are becoming a limiting factor on the size of networks.  This situation can be alleviated somewhat by developing systems which are easier to operate.  The ANSS development should have significant effort devoted to making the system easy to operate.  The following section describes some ideas for improving the operability of the ANSS.

### Configuration of System

Network and monitoring configurations should be easy to modify.  Configuration information includes things such as station location, instrument gain information, hostname information, channels used in processing and others such information.  Establishing, tracking, and updating a configuration on an earthquake monitoring system should be simple and automatic.

The ANSS should establish standardized configuration files.  For example, the BRTT Antelope system has a standard parameter file format which all of its program use for their configuration files and Java programs use “properties” files.  This type of standardization of configuration files is a significant help when configuring and operating a system.

The ANSS configuration should be a “dynamic” if possible.  That is, if the network changes, new information should be entered into the system and the system should incorporate and use the new configuration information.

The ANSS should support configuration versioning.  If a new configuration is put in place and is discovered to have problems, it should be easy to revert to an earlier configuration.  Also, users often are interested in what configuration was in use during a certain event.  The ANSS should support tracking of configurations and the ability to recover a operational configuration by date and time.

In cases where the ANSS uses configuration files, the ANSS designers should consider storing configuration information in a database.  Use of a database is frequently a helpful organizing principle.  If application programs do not, or cannot be written to access a database directly, the ANSS may store the necessary configuration information in the database, and then run programs to extract the configuration information from the database and to format the information into an appropriate configuration file.

### Initialization of System

When the system is started, it should initialize itself and provide positive information that has restarted properly and is operational.  It would be particularly valuable if the system could demonstrate that all of the critical portions were operating.  It may be possible to design a system to inject some test data, possibly waveform data, into the system on initialization.  By injecting the proper data, the entire processing thread could be exercised, data acquisition, phase picking, location, magnitude and alarm reporting.  Verification of the critical processing thread in this way would be very valuable.  It increases the reliability of the system, and decreases the operation load by making verification of a new configuration simpler.

### Reconfiguration of System

The ANSS should be reconfigurable while it operates.  Frequently systems are designed so that they read their configuration files on startup, and then run from memory based configurations after that.  The programs in the ANSS should accept a reconfiguration signal, or command, which cause the programs to re-read their configuration files without exiting.

### Offline Testing

The ANSS should allow offline testing.  It is particularly important to perform offline testing of new configurations before they contribute information to the public.  The ANSS might be designed operate offline using simulated, or stored data.

The Earthworm system has, in the past, used a Quality Assurance suite of events.  The developers inject these events (sets of waveforms) into the system.  The number and location of events in the data set is known and can be compared to what the system produces.  This is a very valuable capability that should be supported by the ANSS.

### Alternate computers for maintenance

Hardware redundancy will be a robustness issue that the management and system engineers will need to resolve.  Redundancy improves robustness but also make the system more expensive and more complicated.  At a minimum, however, the ANSS should be designed so that a replacement system can be configured and installed rapidly while the original system is taken down for maintenance.  The suggestion here is that as each essential ANSS component is installed, the operations group should know how to replace it if it fails.

### Diagnostic Tools

The ANSS should provide the ability to diagnose software and hardware problems while the system is in operation.

Various monitoring tools are needed, such as tools to monitor hardware systems.  Commercial hardware monitoring techniques and protocols such as SNTP should be considered.  Network monitoring tools should be developed which can diagnose problems such as high packet loss.

Software monitoring tools are also important.  Software such as real-time waveform viewing applications can be helpful.  Also software tools that allow the operators to view results any where in the processing chain would be very valuable.  A standard debugging technique is to follow the data, and see where the answers begin to get unreasonable.  The ANSS diagnostic software should support such data monitoring.

### Operational Procedures

The ANSS needs to establish systematic operational procedures to insure the health and maintenance of the system.  For example, the establishment of a routine maintenance schedule would identify various activities as being performed daily, weekly, monthly annually.  This operations and maintenance system would used each day to identify which tasks are due.  It would also log and track what was done and when it was done.  This type of systematic operational procedure may be needed to operate a system as large as the ANSS.

### Documentation

The ANSS should provide some operational documentation.  Documentation is frequently neglected.  The ANSS must provide documentation on how to operate the essential hardware and software used by the system.

### Knowledge Base and Discussion Forum

Once the ANSS is in development, the users very quickly will learn it and will become a willing and capable support staff.  The ANSS should provide two things (1) a knowledge base so that users and find information about the system on their own, and (2) a discussion and problem-solving forum of some type such as an email distribution list which reaches other ANSS operators and users.  Providing these tools will allow the ANSS operators to help themselves.

# Appendices

## Appendix A - Meetings of the NAI subcommittee

In order to develop this report, the NAI subcommittee held several meetings and conference calls.

November  13-14, 2000 - Meeting in Pasadena, CA

Members:  Lind Gee,  Kent Lindquist, Phil Maechling, Tony Shakal, Mitch Withers

Liaisons: Ray Buland

Visitors:  Kuo-wan Lee (for Tony Shakal)

January 11, 2001 - Conference call

Members:  Lind Gee,  Kent Lindquist, Tony Shakal, Mitch Withers

Liaisons: Ray Buland

January 24-25, 2001 - Meeting in Golden , CO

Members:  Lind Gee,  Kent Lindquist, Tony Shakal, Mitch Withers

Liaisons: Ray Buland, Steve Malone, Rick Benson (for Tim Ahern)

Guests:  Alex Bittenbinder, USGS; Danny Harvey, BRTT

February 7, 2001 - Conference call

Members:  Lind Gee, Kent Lindquist, Gary Pavlis, Tony Shakal, Mitch Withers

Liaisons: Ray Buland, Steve Malone

February 14, 2001 - Conference call

Members:  Lind Gee, Doug Given, Steve Goldstein, Kent Lindquist, Tony Shakal, Mitch Withers

Liaisons: Ray Buland, Steve Malone

February 20, 2001 - Conference call

Members:  Lind Gee, Steve Goldstein, Kent Lindquist, Gary Pavlis, Tony Shakal, Mitch Withers

Liaisons: Tim Ahern, Ray Buland, Steve Malone

Visitors:  Kuo-wan Lee

February 28, 2001 - Conference call

Members:  Lind Gee, Kent Lindquist, Tony Shakal, Mitch Withers

Liaisons: Tim Ahern, Ray Buland

Visitors:  Kuo-wan Lee

March 5, 2001 - Preliminary Report released

March 28, 2001 - Conference call

Members:  Lind Gee, Doug Given, Kent Lindquist, Mitch Withers

Liaisons: Ray Buland

April 17, 2001 - Meeting in Berkeley, CA

Members:  Lind Gee, Doug Given, Kent Lindquist, Gary Pavlis, Mitch Withers

Liaisons: Ray Buland

Visitors:  Harley Benz

April 18, 2001 – Presentatation at the SSA

April 25, 2001 - Conference call

Members:  Lind Gee, Doug Given, Kent Lindquist, Gary Pavlis, Mitch Withers

Liaisons: Ray Buland

May 17, 2001 - Conference call

Members:  Lind Gee, Doug Given, Kent Lindquist, Mitch Withers

Liaisons: Tim Ahern, Ray Buland, Steve Malone

May 30, 2001 - Conference call

Members:  Lind Gee, Doug Given, Mitch Withers

Liaisons: Tim Ahern, Ray Buland, Steve Malone

## Appendix B - Acronyms used in this document

ANSS		Advanced National Seismic System

API		Application Programmer Interface

BER		Bit-Error-Rate

BLOB		Binary Large OBject

CNSS		Council of the National Seismic System

COTS		Commercial-Off-The-Shelf

CTBT		Comprehensive Test Ban Treaty

DBMS		DataBase Management System

DWDM	Dense Wave Division Multiplexing

IP		Internet Protocol

LAN		Local Area Network

NAI		Network Architecture and Interconnections subcommittee

NAS		National Archive System

NEIC		National Earthquake Information Center

NPC		National Processing Center

QC		Quality-Control

REIC		Regional Earthquake Information Center

RFC		Request For Comments

SEED		Standard for Exchange of Earthquake Data

SOH		State of Health

TCP		Transmission Control Protocol

TIC		Technical Integration Committee

UDP		User Datagram Protocol

UDWDM	Ultra-Dense Wave Division Multiplexing

USNSN 	United States National Seismographic Network

WAN		Wide Area Network

WDM		Wavelength Division Multiplexing

## Appendix C - ANSS Data Rate/Volume Estimates

A spreadsheet (available on request) has been prepared as a tool for examining ANSS data handling loads.  By adjusting parameters, most of the common data handling strategies currently in use can be modeled.  The model starts with a description of network elements including the number of stations per element, the number of components, sample rate, and field triggering percentage for both seismometers and accelerometers at each station, and the average number of bits per sample.  The bits per sample represent the mean output of a compression algorithm (including compression overhead) or the raw number of bits output from a digitizer.  Note that only the highest sample rate is considered.  This is a good approximation if there are no lower rate data streams or if the highest rate data stream is telemetered continuously.  It is a poor approximation if the highest rate data stream is triggered and a lower rate data stream is continuous.

Given this information, telemetry data rates, intermediate data buffer volumes, and archival volumes are estimated.  Data rates include 20% telemetry overhead (data record and protocol packet headers, etc.  ).  Intermediate data buffer storage is a common element of many data handling schemes, but may or may not be part of the ANSS design.  A typical implementation is that data is acquired, triggered, and buffered.  The triggers are associated into events.  The buffered data is then available for later processing for some short period of time to augment the automatic triggers.  This process is particularly important if not all of the continuous data is to be archived.  Archive volume calculations provide for the possibility of a different compression amount and for discarding additional data after telemetry from the field.

The examples provided start with about the right number of stations and some educated guesses about the configuration and rates of various ANSS elements.  In all examples, all urban stations contribute 200 Hz triggered data for about 1 hour/year.  (On the average, this is probably an extravagant estimate as only a few minutes are typically recorded for each event of interest---typically magnitude 5 or larger within a few 10s of kilometers.  )  In all examples, the national backbone stations are limited to a sample rate of 50 Hz due to the large distance between them, but are archived continuously to support teleseismic work.

Four examples are provided: a worst case (maximum data load), a minimum (data load) model, and two compromise models.  Things that affect the model are the initial sample rates, whether data is compressed from the field and/or into the archive, and whether all data is telemetered and/or archived.  The figures in the worst case model are truly frightening: nearly 50 Mbps telemetry rates and over 150 TBytes/year archival storage.  Note that these estimates don't include any data exchange for redundant monitoring or archiving purposes.  The final ANSS aggregate would probably be at least two or three times this value.  Using compression (100 sps model) reduces the telemetry rates to about 16 Mbps and archiving only events from regional and urban stations reduced the archive to about 3.  7 TBytes/year (near Tim Ahern's original estimate).

## Appendix D - ANSS Goals

In order for the NAI subcommittee to proceed, it was necessary to identify the performance goals of the ANSS.  From these goals, system specifications may be determined and appropriate design decisions discussed.  Unfortunately, this has been a difficult process, in part since the USGS Circular 1188 did not fully specify the performance goals.

### Products

Advanced infrastructure for seismic monitoring

Gather critical technical data

Provide earthquake information products

Do strong-motion monitoring (nothing mentioned about doing it in real-time.  )

Automatic broadcast of information for immediate assessment of earthquake's impact.

In large urban areas regional centers will produce ground shaking maps.

Provide warning for tsunamis and volcanic eruptions.

Where feasible do early warning seconds before strong shaking arrives.

Information products must be timely and also appropriate for long-term needs.

Innovative and customized services provided to users needing info and assistance.

### Network organization and structure

Do continuous monitoring nationwide but focus on regions of moderate to high risk

Modernize and expand infrastructure.

Use modern communication networks and processing centers

Components must function in a well-organized way.

Common infrastructure integrated with robust application software.

Should be designed, created and operated using a systems engineering approach.

Install robust hardware and software for real-time data acq.  , processing and automatic exchange of network data among national and regional network recording centers.

Modernize regional centers to uniform standards to communicate with each other, to the national net and to the public.

NEIC is focal point for all seismic monitoring.

NEIC should serve as backup for all regional networks and data centers and be able to replicate their services should a regional center fail.

Integrate signals from all ground-deformation sensors including GPS

### Standards

High performance standards

National and regional monitoring centers should have robust capabilities for RT acq, processing and exchange.

Set standards and performance goals.

Standardize collection, exchange and archiving

Integrate existing capabilities and expertise and use a system engineering approach to create a master plan which includes performance goals, standards and procedures.

Standardize data acq and processing software.

NEIC should lead in setting standards for data formats, processing and exchange.

Standards for data collection do NOT require stopping using existing hardware and software but rather prescribe guidelines to integrate with ANSS.

Performance goals should come from a review of regions throughout U.  S.  to develop specific plans to ensure:

1.  continuous surveillance

2.  reliable delivery of time-critical info in emergency situations

3.  real-time responsiveness of national system 24x7

### Archive

Establish effective data management scheme for integration, archiving and distribution of seismic data.

Investment is needed in data management facilities to organize and distribute raw seismic data for research purposes.

NEIC is the national distribution point for parametric earthquake data, catalogs and general information.

Facilities larger (than NEIC) with more specialized functions are need to archive and distribute raw seismic data.

ANSS should expand IRIS DMS and/or various regional centers such as NCEDC to accommodate increased data or build new facilities based on IRIS and NCEDC models.

### Specific Numbers

Install new stations: 100-NSN, 1000-modern regional, 3,000-free-field SM 3,000-structural SM

Funding for up to 20 regional centers

## Appendix E - ANSS Design Goals

In order for the NAI subcommittee to proceed, it was necessary to identify the performance goals of the ANSS.  From these goals, functional specifications may be determined and appropriate design decisions discussed.  Unfortunately, this has been a difficult process, in part since the USGS Circular 1188 did not fully specify the performance goals.

- Data collected by ANSS need to be accessible in a seamless manner designed for seismological and engineering applications, for the weak-motion and strong-motion communities, and for the researcher and practitioner.

- Avoiding single points of failure, data communication and connection strategies must assure reliable delivery in spite of damage to the system.  Waveform data must be delivered in a timely fashion.  This requires choosing a strategy that accounts for the possibility of larger bandwidth needed for signals from large earthquakes.

- The system should not depend on any single hardware, software, or service provider.  A modular design will insure no single source of dependency.

- The data exchange mechanisms must be designed such that either direct interfacing with standard seismological applications or appropriate interfaces to these applications is supplied.  Mechanisms for data distribution need to present a standardized interface for the end user that is not confusing, allowing straightforward customization for their interests.

- The ANSS infrastructure must integrate component and contributing networks.  This involves accepting input from the broad cross-section of agencies that contribute to seismic monitoring in the United States.  The ANSS provides a unique opportunity to blend these efforts together in a synergistic way, through construction of a networking infrastructure that allows algorithmic diversity, local and regional systems heterogeneity, and nevertheless integrates the input from all components in a seamless way, with final output products available to the community in a standardized form.

- The need for consistent earthquake reporting in ANSS, combined with the need to integrate data and processing results from contributing networks, suggests specifying quality-control mechanisms for contributed data.  These could take the form of threshold levels of data timeliness, reliability, and timestamp accuracy for waveform data; or perhaps a graded approach for incorporation of several different quality-levels of data.

- The current case, and also a possibility at the outset of ANSS implementation, is that not all datalogger manufacturers will adhere to a common format.  Therefore, it may be advantageous to consider a high degree of interoperability in the system, namely an ability to mesh with existing data formats and protocols.

- Scalability decisions need to be based on an understanding of how much of the scalability can be done directly with hardware combined with appropriately designed software.

- The system should be designed, developed, implemented and operated using a systems engineering approach to ensure that  both the whole and its parts meet the desired performance goals in a cost-effective way.

- The system should have a master plan for nationwide seismic monitoring and information flow, including performance goals, standards, and procedures.

- To improve understanding and public safety , the ANSS earthquake data products must be available to a broad user community.

## Appendix F - ANSS Data Products

Although this topic is properly the subject of the Subcommittee on Data Analysis and Products, we wish to address a few words on this subject.

One or more of the current seismic networks supports the following data types.  Most, or all, of these should be provided by the ANSS.  As discussed above, other derived data items will most certainly be developed and must be supported by the system.  This list should be used as an indication of the type of data currently generated by earthquake monitoring systems.  This list focuses primarily on real-time products.

- Real-time seismograms - Media, installation crews, researchers, and public like access to real-time waveforms.

- Phase picks - Currently single component phase picks are common.  Recently three component picks have been developed.

- Origin time - Time should be available in GMT format and local time formats.

- Hypocenter – Locations should be available in both degrees minutes and decimal degrees format.  Depth should be available.  Quality information such as number of phases used should be available.

- Distance from – Reports often include distance from small city, big city, quarry, faults, and historical large events.  Distance from stations is often requested.

- Magnitudes - A wide variety of magnitudes should be available including Mcd, Ml, Me, Mw.  Quality information should be available such as number of stations, and components used in solution.

- Earthquake reports - Event summaries are distributed in many ways including emails and pages.

- Acceleration reports - Peak accelerations recorded in the network should be available.  Current systems send pages as well as email messages listing acceleration recordings.

- Ground Motion Amplitudes – A wide variety of ground motion types are calculated including acceleration, velocity, displacement, spectral acceleration and velocity, and energy.

- Intensities listed by city or landmark – Intensity reports on a city-by-city basis may provide more useful information to the public than magnitude reports.

- Simpson maps – Web based maps showing recent events in a geographical area are popular and important.

- Catalog of quakes – Some users continue to favor text based earthquake catalog formats.

- Event review pages – Rapid evaluation of automatic solutions is supported by specially designed review pages that show waveforms, automatic phase picks and other information.

- Focal mechanisms – A variety of techniques are supported.

- ShakeMaps – Emergency response information is rapidly becoming essential information expected from all networks.

- Community intensity map – The public contributes to produce an intensity map based on public reports.

- Event and continuous waveform data - The seismological research community will look to the ANSS to provide continuous waveform data for studies in earthquake and volcano processes and Earth structure.  This will require both archives of continuous waveforms and event-based waveforms.  In the past, limits on storage capacity have restricted the continuous archives to lower sampling rates and the event-based archives to higher sampling rates.

- Derived time series – The engineering community is vitally interested in both raw time series as well as in derived time series (acceleration, velocity and displacement).  Details about the processing steps used to obtain the product are essential to its use.

- Spectral data series – The engineering community also want response spectra, distributed in standard formats with documentation of damping values, periods, and the computational algorithm used.

- Station infrastructure information - Station coordinates, datalogger and sensor type, instrument response.  For strong-motion data, additional information about the near-surface geology and shear wave velocity are needed

Also, the system should allow specialized reports on subsets of all data - at all stages.  For example, it is increasingly common to request a subset of all the data available, and then to perform additional processing using just this data subset.  For example, network partners may be interested in information from ANSS stations co-located at their facilities.  The ANSS should support this type of data report.

## Appendix G - ANSS Design, Development, and Implementation

In preparing this report, the NAI subcommittee discussed the process of ANSS development in some detail.  USGS Circular 1188 calls for the use of "a systems engineering approach to create a master plan for nationwide seismic monitoring and information flow, including performance goals, standards, and procedures”.  When complete, the report of the NAI subcommittee will specify the functional design of the system architecture   However, there are many steps between the functional design and the implementation of the ANSS.  The NAI subcommittee feels that this process could be facilitated by the explicit identification of necessary tasks - and possible means for their completion.

For the ANSS, the process of moving from the goals to the final system can be divided into several steps:

- Derivation of detailed functional specifications from the broad goals of the ANSS mandate

- Inversion of the functional specifications to obtain a self-consistent set of detailed design specifications that meet the functional needs

- Implementation and testing of a prototype system based on the draft design goals, and/or a review of existing systems for component parts and workable strategies that already exist and can meet part of ANSS needs

- Modification of the initial design to incorporate the lessons of the prototype test

- Implementation and acquisition of the necessary components for the ANSS operational system

- Evaluation of system performance

The completion of these tasks requires a wide variety of skills:  seismological and engineering expertise, facility in system and software design, and competence in software development and installation to name a few.  Some of these tasks, especially guidance tasks, may be available from the seismological community without requiring full-time commitment.  However, other aspects, especially the system/software construction work, will require full-time effort from multiple persons.  With a software and hardware project the size of ANSS, even the specification of a detailed design will probably require full-time effort by multiple parties.  Possible sources of labor for these tasks include pro-bono work from the seismological and engineering communities, consulting work from technical management firms and networking specialists, contracting work from commercial companies, and USGS staff.

For clarity, these major tasks can be placed in a matrix against the possible sources of labor (Table 1).  This matrix is intentionally mostly blank, and has been made available to promote discussion.  The work of the NAI and other TIC subcommittees naturally fall under the first task and represent the use of pro-bono effort from the seismological and engineering communities.

While it may be beyond this particular subcommittee to define roles in ANSS implementation, we recommend that the source of labor for each of the major implementation steps be explicitly clarified.  And we urge the TIC to give careful thought to the most appropriate source of labor for each task.  Although much of the ANSS effort thus far has come from the pro-bono efforts of the community, the NAI believes that that some of these tasks are beyond what should be expected from pro-bono efforts alone.  Pro-bono committees providing direction and guidance to either USGS staff or a combination of USGS staff and external consultants or contractors may provide the best form of community inclusion and oversight while satisfying the need for professional systems engineering.

Table 1:  ANSS Development and Implementation Matrix

| Necessary Tasks | Means to accomplish tasks |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Pro-bono, non-full-time work by seismological community members | Paid work by seismological community members: Request for Proposals | Paid consulting advice from technical management firms, networking specialists etc. : Request for Quote | Paid contracting work from commercial companies: Request for Quote | Consulting guidance from USGS staff | Construction work by USGS staff |
| Derivation of detailed functional specifications from ANSS circular | Technical Integration Committee and Subcommittees |  |  |  |  |  |
| Inversion of functional specifications for design specifications |  |  |  |  |  |  |
| Implementation and testing of prototype system based on draft design goals; and/or review of existing systems |  |  |  |  |  |  |
| Modification of initial design after prototype test |  |  |  |  |  |  |
| Implementation of components for final ANSS operational system: main construction work |  |  |  |  |  |  |
| Seismological guidance to make sure ANSS goals will be met |  |  |  |  |  |  |

## Appendix H - ANSS Total System

We understand that an important aspect of the ANSS project is to move the state of seismic monitoring in the U.S. from the current a patchwork of independent networks to an integrated and cohesive system.  Our discussion concerning this goal of an integrated system caused much debated and we would like to present some of the issues discussed for consideration.

#### Systems Engineering Approach

We encourage the ANSS management and development groups to consider the ANSS as a system.   Our definition of a system is "hardware, software, and people working together to solve a particular problem, or to produce a desired effect."  We feel that all components of the “system” need to be considered in order to produce a satisfactory system.  The systems approach implies there is some balance between these areas.

One way of evaluating whether an area is receiving sufficient consideration is to ask if it is receiving any funding.  If the funding for hardware is 2 or 3 times the funding for software, maybe the system will have outstanding hardware, but the processing system will be not reliable, or robust, or will not produce the desired information.  If the funding for software is large, but there is no funding for system operators, there may be problems keeping the system up to date.

Another metric focuses on the deliverables as an additional component of the system.  Several levels of these components may be considered, including but not necessarily limited to, a network level and a management level.  Each level typically includes all components.  For example, the network needs seismometers, processing, and analysts, to produce seismic bulletins; management needs copiers, word processors, and administrative staff to formulate plans.  The system must include an appropriate measure to ensure a balance among the various aspects and an appropriate tool or feedback mechanism to adjust resource allocation.

#### Assemble versus Build

Our discussions lead us to the following dilemma for the ANSS management.  Building a new system leads towards a more integrated and cohesive system.  However the cost of developing a system is significantly higher than assembling one from existing working parts.  Assembling an ANSS from existing networks leads to a less cohesive system but increases the speed of development and reduces costs.  It is difficult to assemble existing parts, and to cause them to behave as a cohesive and integrated whole.

It is very common in seismological networks to assemble their networks from a variety of pieces.  As network increase in size, they become more and more heterogeneous in seismic equipment, computing hardware, and processing software.  Based on the size of the ANSS, this type of heterogeneity is going to be a big issue for ANSS.

#### Standardization

The NAI subcommittee has not achieved consensus on the issue of standardization, particularly as it relates to ANSS software.  As noted in the Introduction, one end-member model is that all components of the ANSS are built with identical software.  The other end member is that all components are built with compatible software and that integrated operation comes through well-defined interfaces and protocols.  A point between these end members provides for largely identical software with locally customized modules.

This is an issue where reasonable people disagree.  There are benefits - and detriments - to both end members.  Since "people costs" are one of the expensive components of any complex system, the ANSS should attempt to minimize the amount of duplicated effort and resources.  Identical software is one means to achieve this, as support can be centralized.  On the other hand, "one size" does not generally fit all - and some local customization will be essential.  The requirement of identical software may also reduce the innovation and limit the evolution of the system.

The NAI subcommittee does not feel that the ANSS should require identical software at all processing nodes.  The need for local customization is too evident.  However, the degree of acceptable hetereogeneity is still being actively discussed.  One solution may be for the ANSS to establish software performance standards and then to "qualify" packages or modules if they achieve those standards.

#### Operational Roles

The existing ANSS documentation presents a thorough analysis of the users of the system.  These users include global seismologists, regional seismologists and others.  We encourage the management to develop a similar analysis of the roles needed to develop and maintain the system.  These roles would include software designers and developers, maintenance programmers, network administrators, database administrator, technical writers, technical trainers, duty seismologists, seismic analysts, technical support staff, and technical management staff.  Then, given the roles involved, the system should budget support for properly trained operations staff to perform these roles.

The experience among network operators in our group indicates that the operations roles are almost always under estimated, and understaffed.  Often, only one person is knowledgeable about some part of the system, introducing a human single-point-of-failure.  The technical sophistication level proposed for the ANSS is fairly high involving technologies such as distributed real-time computing, LAN and WAN networking, and relational databases.  There is a very common practice in networks to assign geologists, post-docs, and seismologist to perform seismic network operations tasks.  They fairly quickly are no longer seismologists, but system operations staff.  ANSS may get better system operations, and happier seismologists if it hires operations people for network operations work, and seismologists for seismological positions.

#### System Lifecycle

One more perspective offered by the “systems” view is to consider the lifecycle of ANSS.  There are several issues related to the lifecycle of the system worth considering.

To begin, how long is the system expected to last? What is the expected lifespan of the ANSS? Computer and software system typically have a fairly short lifespan.  A realistic lifespan would probably be about 10 years.  Within 10 years, the hardware and software technology will have progressed to the point that anything developed now will look obsolete.  By establishing a nominal lifespan for the system, now during the planning stages, you relive the developers of the burden of trying to consider what will be needed 10 years from now.  They can just assume that another system will be build by them.

An alternate approach is to assume system development will be on-going. In this model the parts of the system will evolve over time to incorporate new technology and requirements. This will require yearly resources for software development and hardware replacement.

Another issue worth considering is how the ANSS staff roles change during the systems lifespan.  During early development you need visionaries, and politicians, and system and software designers.  Then, once designed, you need software developers, and installation groups, and project managers.  Then, once operational, you need seismic analysts and duty seismologists, and maintenance programmers, and technical writers.  It has been a common mistake within existing networks not to recognize the changing personnel roles as a project progresses.  Just as there is a difference between a global research seismologist and an operations oriented duty seismologist, there are differences between software designers and maintenance programmers.  It is important to make these types of distinctions in all the roles in the system during its life cycle.

Groups familiar with software system development report that 50% of the cost of software development occurs during the maintenance phase of a project.  Rather than reducing the software development budget by half to reserve funds for maintenance, maybe the software development budget should be doubled with the increase earmarked for the maintenance phase of the project.