PROGRAM DOCUMENTATION

QPAGER Version 2.80

November 25, 1996

Program and documentation by

Peter T. German Geophysical Consulting,

under contract to the California Institute of Technology.

Copyright (c) California Institute of Technology, 1991-1996.

All rights reserved worldwide.

Hardware and System Requirements:

QPAGER is designed to run on PC/AT compatible computers under the

MS-DOS operating system, version 3.0 or later.  The program is

designed for an EGA, VGA, or VESA compatible graphics system and

monitor.  Operation on other graphic systems has not been tested.

Communication with the Motorola Advisor pager requires an RS-232C

serial port configured as COM1 or COM2.  A math coprocessor is

not required, but will speed up the program if present.

QPAGER requires a minimum of 265 Kbytes of system memory to

execute, and uses up to 385 Kbytes of system memory when running.

QPAGER program files and map files for Southern California

require 575 Kbytes of disk space.

Disclaimer of Warranty

This software and documentation are provided "as is", without any

expressed or implied warranties whatsoever.  Because of the

diversity of conditions and hardware under which this program may

be used, no warranty of performance, merchantability, or fitness

for a particular purpose is offered.  The user is advised to test

the program thoroughly before relying on it, and the user must

assume the entire risk of using the program.

Acknowledgments

PC/AT is a registered trademark of International Business

Machines Inc.

MS-DOS is a registered trademark of Microsoft Corporation.

For graphics library and fonts included in the executable file:

Portions (c) Copyright Genus Microprogramming, Inc. 1990-1993.

------------------------------------------------------------------

What's New ?

Update November 25, 1996: Features New to QPAGER Version 2.80

A.   The CUBE "E" and "C" message formats are modified to allow

the broadcaster to indicate that the message has been

confirmed by a seismologist, by using lower-case letters

in the "location type" field.

B.   Earthquake messages that have been confirmed by the broadcast

source are now displayed in a contrasting color in the event

list, and the word "VERIFIED"  is added to the data display

window.  Colors for the list text and the data text can be

assigned by the user in the INI file.

Update August 9, 1996: Features New to Version 2.70 of QPAGER

A.   A new message format is added, for compressed ground motion

data.  For format specifications, see section 9.1.9 CUBE "g"

Format, page 31.

B.   A new type of disk file, for seismic station databases, is

now required for QPAGER to translate the new compressed

message format for ground motion values.  See section 5.6

(page 19) for information on seismic station database files.

C.   The QPAGER program diskette now includes the program

MAKESDB.EXE, which is used to create seismic station

database files from a text list of seismic stations.  See

MAKESDB.DOC for information about MAKESDB.EXE.

D.   New menu selection:

E    "Edit" sub-menu

S    Show 15 Minutes.  A filter to show only

the events occurring in the time period from the

currently selected event up to 15 minutes

following the currently selected event.  To return

to the normal display, use the "Restore List"

command.  This command is disabled if the /NOE (No

edit) command line switch is used.

E.   New command line switches:

/NOM      No Menu.  This command turns off display of

the menu at the top of the screen.  All the menu

commands will function normally, only the display

is affected.

/NOQ      No Quit.  Use this command to disable the

"Quit" menu selection, so the "Q" key will not

cause the program to terminate.  You can still

terminate the program using `Ctrl'+'C'.

/NOGA     No Ground motion Alarm.  This disables the

alarm for all ground motion messages.  The alarm

for epicenters and centroids is not affected.  Use

/NOA to completely disable all alarms, or /An.n to

set a minimum magnitude level for the alarm.

F.   Video hardware problems that caused the diamond map symbols

to be drawn irregularly on some systems have been fixed.

------------------------------------------------------------------

1. Summary of QPAGER program functions.

This program receives earthquake information broadcast by Caltech

and others to a Motorola Advisor radio pager or similar device

connected to the computer's serial port.  The program displays

the earthquake information on the screen on a map and also as

detailed text information; sets off an alarm; and saves the

earthquake information in an ASCII text disk file.

The program is designed to receive earthquake information

broadcast by the Caltech-USGS Broadcast of Earthquakes (CUBE)

project, by the UC Berkeley Seismographic Station Rapid

Earthquake Data Integration (REDI) project, and by several other

broadcasters conforming to CUBE standard message formats.

For each earthquake message received, the program displays the

event date, time, magnitude, latitude, longitude, depth, and a

descriptive text message.   In addition to earthquake epicenters,

QPAGER receives and displays two other types of earthquake

information -- earthquake "centroid" locations, and measured

values of ground motion at individual seismic stations.

1.1 Earthquake list:

The program keeps a time-ordered list in memory of up to 512

earthquake messages, which the user can scroll through to see the

earthquake information and location on the map. (Depending on

available memory, the maximum size of the list may be less than

512.)  If the program receives more messages than will fit in the

list, the oldest message is deleted from the list to make room

for the newest message.  The user can set a time limit in days

for the earthquakes to be shown on the screen -- earthquakes

older than the time limit are not shown.  Each day at midnight,

the program deletes from memory the earthquakes that have past

the time limit, and re-draws the map.  The user may also set a

magnitude threshold, so that earthquakes smaller than the

threshold will not be shown.

The date and time of the earthquakes in the list may be displayed

as either GMT or local time (GMT in 24-hour format; local time in

12-hour format), selected in the "Options" dialog box.  The

default local time zone is Pacific Time (Standard or Daylight, as

appropriate), but the local time zone may be changed by the user.

For more information, see section 7, 'Event Time', below.

You can edit the list, by deleting selected earthquakes, or by

restoring the complete list from disk file.  The delete and

restore commands are in the "Edit" menu.

You can choose to display ground motions only, earthquakes only,

or both ground motions and earthquakes (the default), using the

"Earthquakes" and "Ground motions" selections in the "View" menu.

Earthquakes that have been confirmed by the broadcast source are

shown in a different color in the list.

Editing or disabling display of some events deletes them from

memory only, not from the file.

1.2 Earthquake Map:

QPAGER displays seismic event locations superimposed on a

geographical base map.  The base map is drawn from latitude-

longitude data read from disk files supplied by the user, which

may describe geographical polygons, poly-lines, symbols, or text

labels; with user defined colors and line-styles.

Included on the program diskette are files containing Southern

California coast, borders, faults, and highways; they cover

36�50'N to 32�10'N, 115�W to 122�W.  The user may define

customized base map files in the program initialization (INI)

file.

The locations of seismic events are shown on the map with

different types of symbols of variable size and color: epicenter

locations are shown as circles; centroid locations are shown as

stars; and ground shaking amplitudes are shown with diamonds. The

symbol size increases with magnitude or amplitude, and the symbol

color varies with the age of the event.  The user can customize

the symbol colors as a function of age (colors are set in the

initialization file).

A map legend explaining the symbol meaning, symbol sizes, and

symbol colors may be displayed using the "Legend" command in the

"View" sub-menu.

The program's map display is an equi-rectangular projection

(equidistant cylindrical), with initial map boundaries set by the

user in the initialization ("INI") file.  The "View" sub-menu

gives the user control of the map view:  "Zoom" will zoom in on

the selected earthquake; "Refresh" will re-draw the map without

changing the view; and "Restore" will re-draw the map with the

initial view parameters.

A map overlay may be added or deleted from the base-map display

by using the "Map Overlay" menu selection.  The overlay is

defined by the user in the same way as the base-map: using map

files declared in the program initialization ("INI") file with

the "overlay:" prefix.

1.3 Earthquake history file:

The program saves the earthquake information on disk in an ASCII

text file, so that information for past earthquakes can be

retrieved from this file each time the program is executed.  Each

earthquake is saved to the file at the time it is received, and

not deleted from the file even if it is deleted from the

program's list.  The file is allowed to grow arbitrarily large.

See also section 5.1 (page 17), and section 9.2 (page 36).

1.4 Archive files

The earthquake history file can be prevented from growing

arbitrarily large by enabling file archiving with the /ARC

command line switch.  At the end of each calendar month, the

program copies data from the earthquake history file to an

archive file covering a one month period. The archived data are

deleted from the current working history file, except for events

that are still within the time history shown on the screen.  When

QPAGER starts up, it also checks to see if monthly archiving is

needed, in case the program was not running at the end of the

previous month, or in case this is the first time that archiving

has been requested.  The archive process is activated by the

system clock, but the data are copied based on the GMT time of

the events in the file. Therefore, the archive data will be filed

correctly, even if the system clock is not correct.

The archive files are written into the same disk directory as the

current working history file, and named "QPyyMMM.dat" where yy =

year and MMM = month.  Example: QP94OCT.DAT

The archived data are flagged in the working history file with an

"***ARCHIVE" text line indicating date and time of archive, and

the archive file name.

If the /ARC switch is not used, the program does no archiving and

works the same as earlier versions -- the history file grows

continuously.

1.5 Earthquake Centroid

The earthquake "centroid", developed by Hiroo Kanamori at

Caltech's Seismological Laboratory, is a new way to characterize

the location of an earthquake. The centroid is essentially the

geographical "center of gravity" of strong ground shaking during

an earthquake, based on amplitude of shaking for the entire

duration of the earthquake.  An earthquake's centroid may be in a

different location than its epicenter, because the epicenter is

the location where the shaking initiated, but not necessarily

where it was the strongest.  The magnitudes for the epicenter and

centroid may also differ slightly.

The centroid is displayed on the QPAGER map using a star symbol,

and indicated in the list with an asterisk next to the magnitude.

A new "C" message format was developed to broadcast centroid

information.

1.6 Measured Ground Shaking:

QPAGER can receive and display the measured values of ground

motion at individual seismic stations -- peak ground

acceleration, peak ground velocity, or peak ground displacement.

The site where the ground motion was measured is shown on the map

with a diamond symbol, and the value is displayed in the

"message" text line on the QPAGER screen.

Use the "Ground Motions" selection in the "View" menu to enable

or disable display of ground motion data.  Data will be saved in

the file even when ground motion display is disabled.

Ground motion data are broadcast via the "G" and "g" message

formats.

1.7 Estimated ground shaking:

Based on the magnitude of an earthquake, QPAGER can estimate the

probable ground motion resulting from a quake of that magnitude.

The ground motion estimate is shown on the map as a region where

the motion exceeds a pre-set minimum value.  The minimum value

can be set using the "Options" menu selection.  Ground motion

estimates are available for peak acceleration and for peak

velocity, for quakes of magnitude 2.0 - 8.0; and for modified

Mercalli intensity for quakes of magnitude 3.0 - 8.0.

Ground shaking estimates are based on statistics for past

earthquakes in the California region.  Acceleration and velocity

estimates are based on empirical relations by Joyner and Boore

(1981), using 50 percentile values for "rock" surface.  Intensity

is estimated using empirical relations by T.R.Toppozada

(B.S.S.A., v.65, no.5, 1975).

WARNING: The ground shaking estimates are statistically PROBABLE

values;  actual ground shaking may be significantly larger or

smaller.

1.8 Screen colors:

Users can define custom screen colors for text fields and map

background.  The colors are defined in the "[colors]" section of

the initialization ("INI") file.

1.9 Screen saver:

To prevent the program display from burning into the video screen

over prolonged periods of time, QPAGER has a built-in screen-

saver.  When the program has been inactive for more than a pre-

set length of time, the screen display switches to an image

designed to minimize screen wear.  The foreground screen display

is restored whenever a pager message is received, or whenever the

user presses a keyboard key.

1.10 Background pager buffer:

When QPAGER is not running, the program cannot receive earthquake

messages.  However, messages can be saved in a memory buffer by

program PAGERTSR 1.2 when QPAGER is not running, even when other

programs are running.  The earthquakes are retrieved from the

background buffer by QPAGER each time it is executed.

Documentation for PAGERTSR is included in a separate document.

------------------------------------------------------------------

2. Installing QPAGER.

To install the program on your computer's fixed disk drive, copy

the following files from the program diskette into a directory on

your hard disk:

QPAGER.EXE   = executable program

QPAGER.INI   = map initialization file, for S. California

CA_COAST.BIN = binary map file: California & Nevada

CA_LAKE.BIN  = binary map file: salton sea and lake isabella

BORDERS.DAT  = map data file: Nevada and Mexico borders

FAULTS.BIN   = binary map file: geological faults in S. Cal.

CA_TOWN0.SMB = map symbol file: major towns in S. California

CA_TOWN1.SMB = map symbol file: smaller towns in S. California

CITTOWNS.SMB = reference point file: towns in S. California

PAGERTSR.EXE = TSR program for background serial port buffer

SYMBOL.EXE   = program to display array of QPAGER symbols

POLYBIN.EXE  = program to convert map files to binary files

QFORM221.EXE = program to convert list files to ver 2.2 format

MAKESDB.EXE  = program to make seismic station database files

You may also run the program directly from a floppy disk, if you

wish.

------------------------------------------------------------------

3 The Pager and the Serial Port

QPAGER is designed to receive messages from a Motorola Advisor

pager through the computer's RS-232C serial port.  It can also

receive messages from Motorola NewsStream pagers, Kantronics KPC-

3 packet radio TNC, or any similar RS-232 compatible serial

device that can deliver CUBE standard format messages.

If you are using the Motorola Advisor pager, you will need a

Motorola "Pager Mechanical Interface" (NRN8576A) to connect the

pager to the computer.  Connect the Pager Mechanical Interface

unit to your computer's serial port with the appropriate cable,

and plug the power supply cable into the interface unit and to a

110 VAC electrical outlet. Put the pager (turned on and with

fresh battery) into the interface unit.  The small red light on

the front of the interface unit indicates proper contact, and it

must be on for messages to be received by the computer.

The program uses either serial port COM1 or COM2, selected by the

user on the command line.  QPAGER uses the standard PC configura

tions for COM1 and COM2:

COM1 = IRQ level 4, UART base address 03F8, interrupt 0C.

COM2 = IRQ level 3, UART base address 02F8, interrupt 0B.

If your computer system has a built-in serial port, it is

probably configured as COM1.  If you have more than one serial

device on your system (such as a modem), you may need to set the

computer's hardware switches to designate a serial port

configuration for the pager that does not conflict with any other

serial devices installed on your computer.  See your computer's

manual for instructions on configuring the serial ports.

As with virtually any other serial communications program on a PC

computer, QPAGER requires that the COM port assigned to it have a

unique hardware configuration (IRQ level, UART address) that is

not shared by any other device or port connected to the computer.

By default, the QPAGER program expects the pager to communicate

through the serial port at 1200 baud, 8 data bits, no parity, one

stop bit.  This is the normal configuration of the Motorola

Advisor pagers supplied by AirTouch Paging.  However, you can

configure the serial port communication parameters to suit other

devices by using the /COM command line switch, described in

section 4.2.

------------------------------------------------------------------

4. Running the program.

1. Change to the directory containing the program files:

At the DOS prompt, type:   CD [directory path name] <Enter>

where [directory path name] is the name of the disk drive

and directory where you installed QPAGER.

2. Execute the QPAGER program:

At the DOS prompt, type:    QPAGER [commands] <Enter>

The [commands] are optional, but may be one or more of the

commands listed below, in any order, upper or lower case,

separated by a space.  All commands must start with the

slash character `/'.

4.1  Commands-Line Switches:

/A<x.x>   Alarm magnitude threshold, where <x.x>

is a decimal number 0.0 to 9.9.  Events with

magnitude less than the threshold will not set off

the alarm.  If set to 0.0, events of unknown

magnitude will trigger the alarm; if set > 0.0,

events of unknown magnitude will NOT trigger the

alarm.  Default = 0.0.

/ARC      Saves the earthquake history file to a

monthly archive file at the end of each month.

Deletes the archived data from the current

earthquake history file, and writes an ***ARCHIVE

text flag in the history file.

/COMn:baud,p,d,s

Serial port communication  configuration,

in the same format as the DOS MODE

command:

n    = port number (1 or 2) (default = 1)

baud = baud rate            (default = 1200)

p    = parity (N, E, or O)  (default = N)

d    = data bits (7 or 8)   (default = 8)

s    = stop bits (1 or 2)   (default = 1)

To set COM configuration, the colon and

comma delimiters are required, but any field

except port number may be left blank or the string

cut short to accept the default value.  Setting n

= 0 is equivalent to using the /NOCOM command,

below.  If the /COM command is not used, all the

defaults apply.  The defaults are the correct

settings for Motorola Advisor pagers provided by

AirTouch Paging.   Default = /COM1:1200,N,8,1

/D<n>     Alarm duration in seconds.  If <n>

is negative, duration is set to  24 hours.  If

alarm duration is longer than screen-saver time,

the screen saver will interrupt the alarm.

/H<n>     History in days.  <n> is the number

of days of past earthquakes to show.  Events more

than <n> days old will not be shown.  If <n> < 0,

all events will be shown, regardless of age.

Default = 7 days.

/I<filename>

Initialization file name, including

path. File containing initialization parameters,

including parameters for the program's map files.

A space is allowed between the '/I' switch and the

file-name.  Default = "QPAGER.INI".

/KM       Map scale in kilometers.  Default is  miles.

/L<filename>

Filename, including path, of earthquake

list file, which contains a list of the

earthquakes received by the pager.  A space is

allowed between the '/L' switch and <filename>.

Default = "QPAGER.DAT".

/M<x.x>   Magnitude threshold, where <x.x> is a

decimal number -9.0 to 9.9.  Earthquakes with

magnitude less than the threshold will not be

shown on the map or list.  If <x.x> is less than

or equal to 0, events of unknown magnitude will be

included in the list.  If <x.x> is greater than

zero, events of unknown magnitude will not be

included in the list.

Default = 0.0

/MSG      Save raw pager messages in a disk file

named "QPAGER.MSG" in the current directory.  The

default is not to save raw pager messages.

/NOA      No Alarm.  Program starts up with alarm

disabled.  Default is to start with the alarm

enabled.

/NOCOM    No com driver.  Serial port driver is not installed,

and pager messages will not be received.

/NOE      No Editing.  Deleting earthquakes or changing

parameters in the 'Options' dialog boxare not allowed.

/NOFCS    No frame check sequence.  Disables

checking of received messages with included

checksum characters -- all messages are accepted

as is.  Default is to check all messages, and

accept only messages with valid checksum

characters.

/NOGA     No Ground motion Alarm.  This disables the alarm for

all ground motion messages.  The alarm for epicenters

and centroids is not affected.  Use /NOA to

completely disable all alarms, or /An.n to set a

minimum magnitude level for the alarm.

/NOM      No Menu.  Turns off display of the menu at

the top of the screen.  All the menu commands will

function normally, only the display is affected.

/NOQ      No Quit. Disables the "Quit" menu selection,

so the "Q" key will not cause the program to

terminate.  You can still terminate the program

using `Ctrl'+'C'.

/PL<n>    Pager life time, <n> = integer hours.

Maximum time between pager messages.  If more than

this time elapses between messages, the pager is

presumed to be dead, and a "DEAD" message appears

on the screen for the pager status. If pager life

time = 0, the elapsed time between messages is not

checked. The default is 0.

/R<n>     Video display screen aspect ratio, where <n> is the

aspect ratio as integer percent:

n = 100*(y/x), y is vertical physical screen size

x is horizontal physical screen size.

The aspect ratio is used to draw the map

on the screen without distortion.  Default = 75,

which is suitable for most standard computer

screens.

/SA<n>    Screen saver time, <n>, in integer

minutes.  Maximum time for the program to be

inactive before the screen-saver image is

displayed.  If <n>=0, the screen-saver will not be

activated.  Maximum 1080 min. (18 hours).

Default = 10 minutes.

/V<nnn>   Select a video mode, where "nnn" is the

video mode number, in hexadecimal.  This selection

will override the program's default video mode

(the default is to select the highest available

resolution).  Video mode numbers are listed in

Appendix A, page 40.

QPAGER command line examples:

Example 1:  To run QPAGER with the pager connected to port COM2,

using a list of earthquakes in a file named

"EXAMPLE.DAT" on drive A, and setting the alarm

threshold at magnitude 2.5, type the following:

QPAGER /COM2 /LA:\EXAMPLE.DAT /A2.5   <Enter>

Example 2:  To run QPAGER using a customized map specified in an

initialization file named "CUSTOM.INI", showing

all earthquakes for the past year with magnitude

greater than or equal to 3.0, type the following:

QPAGER /ICUSTOM.INI /M3.0 /H365   <Enter>

Example 3:  To run QPAGER dumping raw messages to the QPAGER.MSG

file, with the serial port configured as COM1, 9600

baud, Odd parity, 7 data bits, 1 stop bit, and forcing

the program to use VGA video mode, type the following

command:

QPAGER /MSG /COM1:9600,O,7,1 /V12   <Enter>

Example 4:  To run QPAGER in a DOS window under Windows, with

SeisPort, and forcing the program to use VGA video

mode, type the following command:

QPAGER /NOCOM /V12   <Enter>

------------------------------------------------------------------

4.2  Menu Selections and Keyboard Commands

While the program is running, users can execute the

following commands via the keyboard keys:

<up arrow>   Scroll up, to previous quake in the list.

<down arrow> Scroll down, to next quake in the list.

<page up>    Scroll up one page in the earthquake list.

<page down>  Scroll down one page in the earthquake list.

<home>       Scroll to first quake in the list.

<end>        Scroll to last quake in the list.

<esc>        Kill the alarm sound, if triggered.

E    "Edit" sub-menu:

<Del>  Delete Quake: Delete selected earthquake from

the list.  Does not delete quake from the disk file.

X    Delete All Except: Delete all earthquakes except

the selected earthquake from the list.  Does not

delete quakes from the disk file.

L    Restore complete earthquake list from file.

S    Show 15 Minutes.  Show only the events

occurring in the time period from the currently

selected event up to 15 minutes following the

currently selected event.  To return to the normal

display, use the "Restore List" command.  "Show 15

Minutes" is disabled if the /NOE (No edit) command

line switch is used.

<Esc>  Return to main menu.

V    "View" sub-menu

L    Legend.  Draws the map legend.  If legend is

already displayed, removes the map legend.

M    Map Overlay.  Re-draws the map,

including overlay files.  If overlays are already

displayed, re-draws the map without the overlay

files.

E    Earthquakes.  A toggle selection to enable or

disable display of earthquake epicenters on the map

and in the list.  A check mark appears to the left

of this menu selection when earthquake display is

enabled.

G    Ground motions.  A toggle selection to enable or

disable display of ground motion data on the map

and in the list.  A check mark appears to the left

of this menu selection when ground motion display

is enabled.

F    Refresh.  Re-draw the map display with

the current view parameters.

R    Restore.  Re-draw the map display with

the initial view parameters.  This command is

available at any time, even when the View menu is

not activated.

Z    Zoom In.  Re-draws the map at twice the

existing scale, centered on the selected quake.

Up to six zooms are allowed (scale increase of 64

times). This command is available at any time,

even when the View menu is not activated.  To

restore the original view, use the 'R' command.

Alt+Z Zoom Out.  Re-draws the map at one-half

the existing scale, centered on the selected

quake.  This command is available at any time,

even when the View menu is not activated.  To

restore the original view, use the 'R' command.

<Esc> Return to main menu.

O    "Options" dialog box.

Shows a dialog box for selecting program parameters.

Cancel selection using <Esc>, or <F1> to accept changes.

T    "Tools" sub-menu:

A    Estimated Peak Acceleration.  Draws a

region   on the map where the peak acceleration

from the selected quake is estimated to exceed or

equal a given minimum value.  Valid only for

magnitude 2-8.  Value can be set with the

"Options" menu selection.  Default = 0.10 G.

V    Estimated Peak Velocity.  Draws a region

on the map where the peak velocity from the

selected quake is estimated to exceed or equal a

given minimum value.  Valid only for magnitude 2-

8.  Value can be set with the "Options" selection.

Default = 5 cm/sec.

I    Intensity.  Draws a region on the map

where the Modified Mercalli Intensity from the

selected quake is estimated to exceed or equal a

given value.  Valid only for magnitude 3-8.  Value

can be set with the "Options" selection.  MM = 1-4

are grouped into one level defined as MM = 1.

Maximum = MM 8.  Default = MM 8.

C    Circle.  Draws a circle of fixed radius

around the selected quake.  Radius can be set with

the "Options" selection.

Default = 50 miles.

<Esc> Return to main menu.

H    "Help" sub-menu:

A    About QPAGER.  Displays version, author,

copyright, and list size information about QPAGER.

<Esc>  Return to main menu.

Q    "Quit"    Prompts the user for confirmation,

then exits the program.

------------------------------------------------------------------

5. Program Disk Files.

The program is designed to use several disk files for run-time

data.  The files are described below.

5.1 Earthquake Event History File.

This file contains a list of the earthquake messages received by

the program.  It is an ASCII text file that can be read or edited

by the user, but is in a fixed format that is required for the

program is to use it properly.  The file is written automatically

by the program, and if a list file does not exist, one is created

by the program (default name = "QPAGER.DAT").  It will continue

to grow in size as long as it is used.

QPAGER version 2.21 or later will not read list files from

versions 1.0 to 2.2.  To use the older files, you can convert

them to the new format using QFORM221.EXE.

The event history file format is given in section 9.2.

5.2 Raw Pager Message File.

If requested with the /MSG command line switch, the program will

save all received messages "as is" in a disk file named

"QPAGER.MSG", even if the message cannot be interpreted by the

program.  This is useful primarily to check for misinterpreted or

garbled messages that do not appear on the QPAGER screen.  This

file is not required.  The file is allowed to grow arbitrarily

large.

5.3 Initialization file.

The initialization (or "INI") file contains the user's customized

settings that are read from this file when the program is

executed.  The following parameters may be set by the user in

this file:

a. title: text title displayed on screen above the map

b. map view: central meridian, latitude bounds, graticule

c. map data: map file names, colors, and line types

d. quake colors: table of color vs. age of earthquake

e. colors: user selection of screen colors

f. reference: geographical reference points for text

description.

The INI file is a text file, and may be modified by the user

using a text editor.  The format is explained in the file

"QPAGER.INI" included on the program diskette.

If this file does not exist, the program will not draw

geographical data on the map, but will display the earthquakes on

a blank map with default latitude, longitude bounds.

5.4 Map files.

The map files contain digitized geographical data, to be drawn on

the earthquake base map.  The data will be drawn as a sequence of

points defining polygons, poly-lines, or symbols with text

labels, using user-selected attributes specified in the INI file.

The program draws the files in the order they appear in the INI

file, so that if map data from different files have the same

geographical position, the last data to be drawn will hide all

previous data at that location.

Map file names prefixed with "overlay:" in the INI file will only

be drawn if the "Map Overlay" command has been selected.  Map

files without the "overlay:" prefix will always be drawn.

The user can add customized map data by creating geographical

data files, and then listing the file names and attributes in the

INI file.  Files containing Southern California coast, borders,

faults, and highways are included on the program diskette.  Any

number of map files may be used, limited only by the user's

available disk capacity.

The program expects the map file to be an ASCII text file

containing multiple polygons, polylines, or symbols, defined as

sequences of (Lon,Lat) points to be drawn in the order they

appear in the file.  A polygon will be drawn by connecting the

last point to the first point, and filling with the designated

color.  A polyline will be drawn from point to point, with a one-

pixel wide line in the designated color and line style.  A symbol

will be drawn centered on the (Lon,Lat) point, including an

optional text label; a null symbol (symbol #0) is available if

text with no symbol is desired.

Map files may be either ASCII text files, or binary files.

Binary files allow the program to draw polygons and polylines

much faster than from ASCII text files.  The binary files are

created from the ASCII polygon or polyline text files using the

POLYBIN.EXE utility described below.  Symbol files may not be

converted to binary.

Map file formats are described in detail in section 9.2.

5.5 Reference Point Files

The reference point file contains the geographical positions of a

list of points, which are used to create the automatic text

message.  QPAGER finds the point in the file that is the closest

to the earthquake event, and creates a text message describing

the distance, direction, and name of the point.

Reference point files are the same format as map symbol files,

and the format is described in section 9.2 as well as in the

"QPAGER.INI" file included with the program.

A file containing the positions of Southern California towns is

included with the program for use as reference points (filename =

CITTOWNS.SMB).

5.6 Seismic Station Database Files

The seismic station database files contain the names, locations,

and sensor components of seismometer station sites.  QPAGER reads

these files when receiving compressed messages containing

measured ground motion data, where the measuring station is

indicated only by an index into the database - full station

location information is not transmitted in this message format.

Therefore, QPAGER needs the station database to retrieve the

station name and location.

QPAGER requires that the station database files must be in the

same directory as the QPAGER.EXE executable file, and there must

be one seismic station database file for each broadcaster that

sends compressed messages.  The station database files must be

named "<id>.SDB", where <id> is the same two-letter name used to

identify the broadcaster for each message.

For example, the station database files for Caltech and UC

Berkeley must be named as follows:

CI.SDB    Caltech

BK.SDB    UC Berkeley

If QPAGER cannot read the station database file, or cannot find

the correct station within the file, the ground motion value will

nevertheless be added to the QPAGER list, displayed on the

screen, and assigned to station "????", at latitude = 0,

longitude = 0.

The QPAGER seismic station database files must be kept up to

date, to correspond to the same station list used by the

broadcaster for sending the messages.  Each broadcaster should

distribute up-to-date station database files whenever needed.

Seismic station database files can be created using the

MAKESDB.EXE program included with the QPAGER program.  Please see

the documentation for MAKESDB.EXE for more information on how to

create seismic station database files.

------------------------------------------------------------------

6. Map Symbols:

The selection of map symbols available to users may be viewed on

the screen by running program "SYMBOL.EXE", included on the

program diskette.  The symbols, with index numbers, are

illustrated below:

[sorry, this figure is not available in this text file]

------------------------------------------------------------------

7. Event Time Zones.

The origin time of seismic events is broadcast in Universal

Coordinated Time (UTC), which is the same time zone as Greenwich

Mean Time (GMT).

The program can convert UTC to a local time zone, for display on

the screen.  The Pacific time zone is the default, but users may

set any time zone for conversion of UT to local time.  The time

zone is specified by setting a system environment variable before

running the program, using the DOS "SET" command.  At the DOS

prompt, or in an AUTOEXEC.BAT or other batch file, use the

following syntax to set the TZ environment variable:

SET TZ=tzn[+|-]hh[:mm[:ss]][dzn]

The <tzn> argument is a three-letter time-zone name, such as PST,

followed by an optionally signed number, [+|-]<hh>, giving the

difference in hours between UTC and local time (UTC - local).

The hours can be optionally followed by minutes, :<mm>; seconds,

:<ss>; and a three-letter daylight-saving-time (DST) zone name,

such as PDT.  For example, SET TZ=PST8PDT represents a valid TZ

value for the Pacific time zone.  Hours, minutes, and seconds

must be separated with colons (:).  If DST is never in effect, as

is the case in certain localities, TZ should be set without a

<dzn> zone.

If TZ is not set when the program is executed, the default is

PST8PDT, which corresponds to the Pacific time zone.

The program currently has no provision for handling leap seconds.

Because the original CUBE messages had abbreviated date

information, earlier versions of the program depended on the

computer's system clock being set to the correct time and date,

in local time.  Now the broadcast message contains complete time

information, and does not depend on the system clock.

------------------------------------------------------------------

8. QPAGER screen layout.

�������������������������������������������������Ŀ

�            MENU BAR                             �

�������������������������������������������������Ĵ

�            TITLE WINDOW           �             �

�����������������������������������Ĵ             �

�                                   �             �

�                                   �             �

�               MAP                 �    LIST     �

�                                   �             �

�              WINDOW               �   WINDOW    �

�                                   �             �

�                                   �             �

�                                   �             �

�                                   �             �

�������������������������������������������������Ĵ

�            DATA WINDOW            �   WARNING   �

�                                   �   WINDOW    �

�������������������������������������������������Ĵ

�            STATUS BAR                           �

���������������������������������������������������

The Menu Bar shows a menu of commands available to the user, with

drop-down sub-menus.

Title window shows text title.

List Window shows a list of earthquakes received by the program.

Data Window shows data for the earthquake that is highlighted on

the map and highlighted on the list.  Data are: date, time,

magnitude, latitude, longitude, depth, and descriptive message.

Map Window shows a map with epicenters represented as dots

superimposed on a geographical base map.

Status Bar shows the status of the pager, the alarm setting,

the magnitude threshold, and the system clock time.

------------------------------------------------------------------

9. Format specifications:

9.1 CUBE message formats, version 2.70

The QPAGER program can interpret pager messages in any one of the

several CUBE standard formats.  All formats use the following

conventions:

A)   All messages are restricted to using ASCII standard

character encoding, unless otherwise noted.

B)   All messages are limited to the following characters only:

20 hex - 7E hex (ASCII 32 - 126), excluding square brackets

'[' and ']' (ASCII 91, 93).  The limitations on character

set are due to the protocol used by paging companies -- APP

or iXO. Only 7-bit characters are allowed, and control

characters (0-31, 127) are reserved.  Square brackets are

used by some pager firmware to frame reception errors.

Square brackets may be used when specifically allowed in the

format, but their use may cause ambiguity depending on

receiving pager firmware.

C)   In the format descriptions that follow, fields marked with

asterisk (*) are required; others are optional. If any

required fields are blank or not translatable, the message

will be interpreted as an error, and will not be used,

except that spaces are acceptable for version number or

broadcast source.

D)   Numeric fields are right justified, padded with leading

spaces or zeros.  Decimal points are not accepted in numeric

fields.  Unused columns in used fields must be padded with

spaces or zeros.  Unused fields may be filled with spaces,

question marks (?), asterisks (*), or the string may be

terminated without including optional fields at the end.

Totally blank fields are translated as "not used".

E)   Event identification numbers may be any text string, which

will be accepted 'as is' by the receiver with NO

interpretation.  Changes in case or justification will be

read as a different identification number.  Identification

must be unique to each broadcast source and event type, but

different sources or event types may coincidentally use the

same id string and still maintain uniqueness.  The fully-

qualified identifier for each event is the composite key:

"Source-Type-Idnumber".  Two events of the same type with

the same identifier from the same source will be the same

event.

F)   All fields are case-sensitive, unless otherwise noted.

G)   When using media that permit sending control characters, the

message may be framed by at least one carriage-return prefix

and suffix (<CR> = ASCII 13, hex 0D).  Because of the

variety of flavors of text new-line processing, any number

of line-feed characters (ASCII 10, hex 0A) will be tolerated

before and after the <CR>. If the message is embedded in a

variable length stream, the CR framing is REQUIRED. CR

framing is REQUIRED for Motorola Embarc messages.  Frame

Example:   <CR>message<CR>

------------------------------------------------------------------

9.1.1  Original CUBE format, seismic event epicenter

MGLATI-LONG-DYHRMNCV-EID-Text........... to 80 characters

1234567890123456789012345678901234567890 <- column number

1         2         3         4

i2 * MG   = magnitude * 10

i4 * LATI = north latitude, degrees * 100 + minutes

i4 * LONG = west longitude, (degrees-100) * 100 + minutes

i2 * DY   = day of the month      (1-31) (GMT)

i2 * HR   = hours past midnight   (0-23) (GMT)

i2 * MN   = minutes past the hour (0-59) (GMT)

a1 * C    = location method code

a1 * V    = version number code

i3   EID  = event identification number (0-999)

a    Text = descriptive text string

Note: All fields are right justified and padded with leading

zeros, except for 'text' field, which is left justified.

The '-' characters in the format definition above are

required as literal characters.

------------------------------------------------------------------

9.1.2  CUBE "E " format: seismic event hypocenter,

fixed format by column:

TpEidnumbrSoVYearMoDyHrMnSecLatddddLongddddDept...

12345678901234567890123456789012345678901234567

1         2         3         4

continued:...MgNstNphDminRmssErhoErzzGpMNmEmLC

890123456789012345678901234567890

5         6         7         8

a2 * Tp   = Message type = "E " (seismic event)

a8 * Eid  = Event identification number  (any string)

a2 * So   = Data Source =  regional network designation

a1 * V    = Event Version     (ASCII char, except [,])

i4 * Year = Calendar year                (GMT) (-999-6070)

i2 * Mo   = Month of the year            (GMT) (1-12)

i2 * Dy   = Day of the month             (GMT) (1-31)

i2 * Hr   = Hours since midnight         (GMT) (0-23)

i2 * Mn   = Minutes past the hour        (GMT) (0-59)

i3 * Sec  = Seconds past the minute * 10 (GMT) (0-599)

i7 * Lat  = Latitude:  signed decimal degrees*10000 north>0

i8 * Long = Longitude: signed decimal degrees*10000 west <0

i4   Dept = Depth below sea level, kilometers * 10

i2   Mg   = Magnitude * 10

i3   Nst  = Number of stations used for location

i3   Nph  = Number of phases used for location

i4   Dmin = Distance to 1st station;   kilometers * 10

i4   Rmss = Rms time error; sec * 100

i4   Erho = Horizontal standard error; kilometers * 10

i4   Erzz = Vertical standard error;   kilometers * 10

i2   Gp   = Azimuthal gap, percent of circle; degrees/3.6

a1   M    = Magnitude type

i2   Nm   = Number of stations for magnitude determination

i2   Em   = Standard error of the magnitude * 10

a1   L    = Location method

a1 * C    = Menlo Park checksum, square brackets permitted

"Message Type" field:

The second character, following the 'E', is no

longer used for magnitude validation.  Prior "En"

format that used the 2nd char for magnitude

validation is no longer supported.

"Event Identification Number" field:

Any string, excluding '[' and ']' characters.

Must be a unique identifier for the event.

"Data Source" field: regional seismic network designation:

BK = U.C.Berkeley

CI = Caltech/USGS, Pasadena, CA

NC = NCSN USGS, Menlo Park, CA

US = NEIC USGS, Golden, CO

UW = University of Washington

"Version Number" field:

May have any value (ASCII 32-126, except 91 or 93).

Meaning varies by source. Used to distinguish between

different versions of the same event.

"Location Method" field:

Upper-case char indicates an unconfirmed event,

Lower-case char indicates an event confirmed by human review

R = RTP

I = Isaiah

A = Auryn

B = Binder

H = Hypoinverse

S = st_relp     (Doug Neuhauser, UCB)

"Magnitude Type" field:

B = Mb  = body wave magnitude

C = Mca = coda amplitude magnitude

D = Mcd = coda duration magnitude

E = Me  = energy magnitude

G = Mgn = MAGNUM (pseudo-empirical local magnitude)

L = Ml  = local magnitude (synthetic Wood-Anderson)

O = Mo  = moment magnitude

P = Mp  = P-wave magnitude (Doug Neuhauser, UCB)

S = Ms  = surface wave magnitude

------------------------------------------------------------------

9.1.3 CUBE "C " format, sesimic event "Kanamori centroid",

fixed column format, as follows:

TpEidnumbrScVYrMoDyHrMnSecLatddddLongdddd ...

12345678901234567890123456789012345678901

1         2         3         4

continued ... DeptMgAmpliRadNstDminRmssErhErzGpEmLCRC

01234567890123456789012345678901234567890

4         5         6         7         8

a2 * Tp   = Message type = "C " = "Kanamori centroid"

a8 * Eid  = Event identification number: (ASCII string)

a2 * Sc   = Data Broadcast Source        (ASCII string)

a1 * V    = Event Version number         (ASCII char)

i2 * Yr   = Calendar year (2 digits)(GMT)(0-99)

i2 * Mo   = Calendar month          (GMT)(1-12)

i2 * Dy   = Day of the month        (GMT)(1-31)

i2 * Hr   = Hours past midnight     (GMT)(0-23)

i2 * Mn   = Minutes past the hour   (GMT)(0-59)

i3 * Sec  = Seconds * 10            (GMT)(0-599)

i7 * Lat  = Latitude: signed decimal degrees*10000 north>0

i8 * Long = Longitude:signed decimal degrees*10000 west <0

i4   Dept = Depth (kilometers * 10)

i2   Mg   = Magnitude * 10

i5   Ampli= Calculated amplitude (units not yet defined)

i3   Rad  = Radius of calculated amplitude (km)

i3   Nst  = number of stations used

i4   Dmin = Distance to 1st station(km*10)

i4   Rms  = RMS amplitude error * 100

i3   Erh  = Horizontal standard error    (km * 10)

i3   Erz  = Vertical standard error      (km * 10)

i2   Gap  = Azimuthal gap, % of circle   (degrees/3.60)

i2   Em   = Magnitude error * 10

a1   L    = Location method              (ASCII char)

a3 * CRC  = error-detection checksum (ASCII including [])

'Year':  Last two digits of calendar year.  If yr>50,

assume actual year=yr+1900. Else, year=yr+2000.

'Calculated amplitude':

Theoretical ground motion amplitude at a

given radius from centroid (see 'Radius').

Units for this field are not yet defined.

'Radius':  Distance from centroid for 'calculated amplitude'

------------------------------------------------------------------

9.1.4 CUBE "G " format, ground motion datum point,

fixed column format, as follows:

TpIdnumberScMYrMoDaHrMnSecLatddddLongdddd ...

12345678901234567890123456789012345678901

1         2         3         4

continued ...  OpSiteCmpAziAlFValueHzCRC

01234567890123456789012345678901

4         5         6         7

a2 * Tp    = Message type = "G " = ground motion datum

a8 * Idnum = Data identification number:   (ASCII string)

a2 * Sc    = Data Broadcast Source         (may be blank)

a1 * M     = Motion type (A, V, D)

i2 * Yr    = Calendar year (2 digits)(GMT)  (0-99)

i2 * Mo    = Calendar month          (GMT)  (1-12)

i2 * Dy    = Day of the month        (GMT)  (1-31)

i2 * Hr    = Hours past midnight     (GMT)  (0-23)

i2 * Mn    = Minutes past the hour   (GMT)  (0-59)

i3 * Sec   = Seconds * 10            (GMT)  (0-599)

i7 * Lat   = Latitude: signed decimal degrees*10000 north>0

i8 * Long  = Longitude:signed decimal degrees*10000 west <0

a2   Op    = Station owner/operator  (ASCII string)

a4   Site  = Station name            (ASCII string)

a3   Cmp   = Station component name  (ASCII string)

i3   Azi   = Motion Azimuth, degrees (clockwise from north)

i2   Ze    = Motion zenith angle, degrees (up from horiz.)

a1   F     = Clip/Noise flag: 'C'= clipped, 'N'= noise

i5 * Value = Value of datum point

i2   Hz    = Dominant frequency at datum point (Hertz)

a3 * CRC   = error-detection checksum (ASCII including [])

'Motion type'

A = peak acceleration, 'Value'= mm/sec^2 (may be signed)

V = peak velocity,     'Value'= mm/sec   (may be signed)

D = peak displacement, 'Value'= mm       (may be signed)

(displacement is transient, not permanent)

'Clip/Noise flag'

C = clipped,  real value > measured value

N = noisy,    real value < measured value

M = measured, real value = measured value

(default = M)

'Value'

Units depend on motion type (see above).

Negative sign indicates value polarity is

180 degrees from azimuth, zenith vector.

------------------------------------------------------------------

9.1.5 CUBE "D" format, to delete seismic events.

The identified event will be deleted:

TpEidnumbrSc...message........ to 80 characters

123456789012345678901234567890

1         2         3

a2 * Tp   = Message type = "DE", "DC", or "DG"

i8 * Eid  = Event identification number of event to delete

a2 * Sc   = Data Source

a    ...  = Optional text message

DE = delete epicenter

DC = delete centroid

DG = delete ground motion datum

------------------------------------------------------------------

9.1.6 CUBE "M" format, seismic event descriptive text message:

TpEidnumbrSc...message........ to 80 characters

123456789012345678901234567890

1         2         3

a2 * Tp   = Message type = "ME", "MC", or "MG"

i8 * Eid  = Event identification number

a2 * Sc   = Data Source

a    ...  = Text message to append to existing event.

If <null>, any existing text will be removed

from this quake.  If valid, this text will

replace any existing text

ME = epicenter text description

MC = centroid text description

MG = ground motion datum text description

------------------------------------------------------------------

9.1.7 CUBE "A " format, 'alive' message to signal that the

broadcast source is operational:

TpLifetimeSc...message.......... to 80 characters

123456789012345678901234567890

1         2         3

a2 * Tp   = Message type = "A " or "0-" ('alive' message type)

i8 * Lif  = Maximum time to next broadcast in seconds

a2 * Sc   = Data Source

a    ...  = Optional text message

If Lifetime <=0, the system will not check for a dead

pager; in effect, lifetime will be infinite.

NOTE: This "A " message can cause errors, and is not

supported in this version of QPAGER.

------------------------------------------------------------------

9.1.8 CUBE "AA" format, 'alive' message, to signal that the

broadcast source is operational:

TpIdnumberSc...message.......... to 80 characters

123456789012345678901234567890

1         2         3

a2 * Tp   = Message type = "AA" ('alive' message type)

a8   Idn  = Id number of last message sent

a2 * Sc   = Data Source

a    ...  = Optional text message

NOTE: This message is preferred to the "A " message, above.

The receiver can use it to detect missed messages.

------------------------------------------------------------------

9.1.9 CUBE "g" format, version 1.0, July 31, 1996

compressed ground motion data,

variable length format, as follows:

MSoQIdnTimeStDataStDataStDataStDataStData...

12345678901234567890123456789012345678901

1         2         3         4

continued...   StDataStDataStDataStDataStDataStDataCrc

01234567890123456789012345678901234567890

4         5         6         7         8

a1 * M     = Message type = "g" (lower case "g")

a2 * So    = Broadcaster, two char standard identification

a1 * Q     = Message sequence number, encoded base-95

a4 * Idn   = Message id and data type, encoded base-95

a4 * Time  = Time: sec's from 01/01/96 UTC divided by 10

(from 1 to 11 pairs of the following two fields)

a2   St    = Seismic station index code, base 95

a4   Data  = Ground motion value, Flags, and Time Offset

from base time, in seconds; all encoded base-95

(trailing the last Data field is a message CRC check)

a3 * Crc   = Kermit-encoded CRC-CCITT

Fields marked with * (above) are required.

Others may be omitted, for a shorter message.

The CRC is always the last three characters in the message.

NOTES for "g" message format:

Base 95

All numbers are expressed as base 95, using ASCII

characters 32-126, which is the complete char set

available for pager messages.

Base95 value = (ASCII char value) - 32

Limits, depending on number of characters used:

1 char = 0 -         94

2 char = 0 -      9,024

3 char = 0 -    857,374

4 char = 0 - 81,450,624

WARNING:

All values MUST be limit checked BEFORE encoding.

Because of the encoding of the values in this message

format, strict limits must be enforced on all values before

encoding the message. Values which exceed specified limit

bounds can be encoded, but can not be decoded correctly by

the receiver.  The fields that are a composite of several

values are especially vulnerable to erroneous encoding

caused by values that exceed bounds.

MOTOROLA ADVISOR:

The Motorola Advisor (tm) pager inserts bracket characters

into the message string when transmission errors are

detected. Since all 95 ASCII printable characters are used

in this format, it is not possible to distinguish the error

brackets from valid message characters. Therefore, it is

possible that errors may occur when translating messages

received on Advisor pagers, but the CRC check included in

the format should make it possible to detect virtually all

such errors.

'So' : Broadcaster

case-sensitive alpha, two characters.

Standard FDSN seismic network identification,

For example, CI = Caltech

BK = UC Berkeley

'Q' : Message sequence number

base 95 unsigned integer

minimum = 0

maximum = 94

Each broadcaster should number messages in sequential

order, so receivers can detect missed messages.

Numbers roll over from 94 back to 0.

'Idn'

This field is a composite of the message

identification number and the data type:

Idn = base95( id * 8 + datatype )

where 'id' is the message identification number

and   'datatype' is the type of ground motion

'id' Message identification number:

unsigned integer

minimum = 0

maximum = 99,999

The message identification number is the id

number of the first data value, and the id

number is incremented for each subsequent

data value:

ID of 1st value = (message ID)

ID of 2nd value = (message ID) + 1

ID of 3rd value = (message ID) + 2

ID of 4th value = (message ID) + 3

.etc.

These ID numbers are shared with the 'G' format messages,

so a value with the same ID as a 'G ' message represents a

change in value to an already broadcast datum. Because ID's

of this 'g' format are strictly numerical, broadcasters can

ensure uniqueness by including one or more alpha characters

in their 'G ' message ID's.

The resulting value ID numbers can be used to delete a value

using the 'DG' message format.  To use the 'DG' message, the

ID number MUST be padded with leading ZEROS. For example,

for Caltech to delete the 4th data value in message '123',

send this:  'DG0000126CI....'.

'datatype' : Ground motion data type:

unsigned integer

minimum = 0

maximum = 7

This value indicates the type of data, as follows:

0  = ground acceleration

1  = ground velocity

2  = ground displacement

3-7  reserved for future use

Because ground motion data type is specified only

once in the message, only one data type can be

sent per message.  All eleven data values must be

of the same type (e.g. acceleration)

'Time' Time base value

base 95 encoded unsigned integer

minimum = 0

maximum = 81,450,624

Time = seconds past 00:00:00 01-Jan-1996 UTC

divided by 10 and expressed as base 95 integer.

This provides a range of 25 years (2021 AD) in 10 second

increments.

'St' Seismic Station Index:

base 95 encoded unsigned integer

minimum = 0

maximum = 9024

The station is specified by an index into a station database

table.  Up to 9,025 station indexes are available per table.

The station database contains station name, location,

component, instrument type, etc.  Each component at a

station will be given a unique index in the table.

Broadcasters must ensure that the station index comes from

the same station table that is used by their receivers.

To use the station database table with QPAGER or SeisPort

message receiving programs, the station database must be

present as a disk file on the receiver's computer system.

The MAKESDB program creates the station database file from

an ASCII text file containing the broadcaster's station

list, in SCEC-station-list file-format.

'Data' : Ground motion data value

base 95 encoded unsigned integer

minimum = 0

maximum = 81,448,799

A composite of the data value, time offset, and flags:

Data = base95( value*2400 + offset*8 + flags )

where value  = encoded ground motion value (see below)

offset = time offset from base time

flags  = bit flags

'value' Ground Motion Datum Value:

unsigned integer

minimum = 0

maximum = 33,936

The ground motion datum value is expressed as

a floating point number encoded into an integer.

value = (int) ( 5656 * ( log10(x) + 5 ) + 0.5 )

where x = ground motion value, in units of

meters, m/sec, or (m/sec^2)/10

minimum value = 1.0000e-5

maximum value = 10.000

epsilon value = 4.0719e-4

where epsilon is the smallest number such that

1 + epsilon != 1

The minimum value (1.0000e-5) is interpreted as

equivalent to zero value.

'offset' Time Offset:

unsigned integer

minimum = 0

maximum = 299

The time of each datum value is specified by

an offset from the base time value, in seconds.

Covers a range of 5 minutes.

'flags'

unsigned integer

minimum = 0

maximum = 7

Three one-bit flags, expressed as an integer:

bit 0 = sign bit = 0 if data value > 0

1 if data value < 0

bit 1 = clipped  = 1 if data value is clipped

bit 2 = noise    = 1 if data signal is noisy

example flags: 1 = negative, not clipped, not noisy

2 = positive, clipped,     not noisy

4 = positive, not clipped, noisy

7 = negative, clipped,     noisy

------------------------------------------------------------------

9.2 Event History File Format:

The earthquake event history file is a standard ASCII text file,

with one seismic event per line, in the following format:

TpEidnumbrSoV year/mo/da hr:mn:se.c Lt�mn'sc"d Lon�mn'sc"d

1234567890123456789012345678901234567890123456789012345678

1         2         3         4         5

continued:   Depth Mag OtherFields.................. $<CR><LF>

89012345678901234567890123456789012345678

6         7         8         9

a2   Tp   = Message type  ('E ', 'C ', or 'G ')

i8   Eid  = Event identification number

a2   So   = Data Source

a1   V    = Event Version number

a1   ' '  = "space" character

i4   Year = Calendar year                (GMT) (-999 - 6070)

a1   '/'

i2   Mo   = Month of the year            (GMT) (1-12)

a1   '/'

i2   Dy   = Day of the month             (GMT) (1-31)

a1   ' '

i2   Hr   = Hours since midnight         (GMT) (0-23)

a1   ':'

i2   Mn   = Minutes past the hour        (GMT) (0-59)

a1   ':'

f4.1 Se.c = Seconds past the minute      (GMT) (00.0-59.9)

Lat�mn'sc"d = Latitude  degrees, minutes, seconds, direction= N/S

Lon�mn'sc"d = Longitude degrees, minutes, seconds, direction= E/W

f6.1 Depth= Depth below sea level, kilometers (-8.0 - +999.9)

f4.1 Mag  = Magnitude, with decimal

a1   ' '  = "space" character

For "E " messages, "OtherFields" are as follows:

i3   Nst  = Number of stations used for location

i3   Nph  = Number of phases used for location

i4   Dmin = Distance to 1st station;   kilometers * 10

i4   Rmss = Rms error in location time, seconds * 100

i4   Erho = Horizontal standard error; kilometers * 10

i4   Erzz = Vertical standard error;   kilometers * 10

i2   Gp   = Azimuthal gap, % of circle (degrees/3.6)

a1   M    = Magnitude type

i2   Nm   = Number of stations for magnitude determination

i2   Em   = Standard error of the magnitude * 10

a1   L    = Location method

a1   ' '  = "space" character

a    $    = Optional text message string, beginning with "$"

<CR> = Carriage return (ASCII character 0D, hex)

<LF> = Line feed       (ASCII character 0A, hex)

For "C " messages, "OtherFields" are as follows:

i5   Ampli= Calculated amplitude (units not yet defined)

i3   Rad  = Radius of calculated amplitude (km)

i3   Nst  = number of stations used

i4   Dmin = Distance to 1st station(km*10)

i4   Rms  = RMS amplitude error * 100

i3   Erh  = Horizontal standard error    (km * 10)

i4   Erz  = Vertical standard error      (km * 10)

i2   Gap  = Azimuthal gap, % of circle   (degrees/3.60)

i2   Em   = Magnitude error * 10

a1   L    = Location method              (ASCII char)

For "G " messages, "OtherFields" are as follows:

a2   Op    = Station owner/operator  (ASCII string)

a4   Site  = Station name            (ASCII string)

a3   Cmp   = Station component name  (ASCII string)

i3   Azi   = Motion Azimuth, degrees (clockwise from north)

i2   Ze    = Motion zenith angle, degrees (up from horiz.)

a1   F     = Clip/Noise flag: 'C'= clipped, 'N'= noise

i5 * Value = Value of datum point

i2   Hz    = Dominant frequency at datum point (Hertz)

The list file may also contain "DE", "DC", "DG", "ME", "MC",

"MG", or "AA" entries, in the same format as the pager messages,

sec. 9.1 (above)

------------------------------------------------------------------

9.3 Map file format

The program expects each map file to be an ASCII text file

containing a sequence of (lon,lat) points to be drawn the order

they appear in the file. The format is tolerant of any amount of

'white-space' characters between fields, so it is not a fixed-

column format.  The only restriction is that the values appear in

the correct order on each line.

Each text line in the ASCII map file is expected to contain one

(Lon,Lat) point, with one of the following formats:

Polygon and poly-line file format:

{whitespace}[{sign}Lon][,]{whitespace}[{sign}Lat]{remarks}<CR>

{whitespace} = any number of <space> or <tab> characters;

[Lon] = longitude, signed decimal degrees, West = negative;

[,]   = field separator, any non-numeric character;

[Lat] = latitude, signed decimal degrees, North = positive;

{remarks} = optional remarks, disregarded by the program;

<CR> = carriage return;

or to contain a control flag with the format:

[$]{remarks}<CR>

where '$' indicates the beginning of a new polygon or polyline.

------------------------------------------------------------------

Symbol file format:

{}[{sign}Lon],[{sign}Lat],Symbol,Size,Align,HO,VO,[text]<CR>

{}    = any number of <space> or <tab> characters;

,     = field separator: any non-numeric character, followed

by any number of <space> or <tab> characters;

[Lon] = longitude, signed decimal degrees, West = negative;

[Lat] = latitude, signed decimal degrees, North = positive;

Symbol= symbol number, integer 0-23

Size  = symbol size, integer radius in pixels

Align = text alignment, two character code (see below);

HO    = horizontal offset from (lon,lat) point to text

position, signed integer pixels;

VO    = vertical offset from (lon,lat) point to text

position, signed integer pixels;

[text]= text label to write on map, maximum 128 characters;

<CR> = carriage return.

Symbol file notes:

1. The symbol will be drawn centered at the (lat,lon) point.

2. If a symbol with no text label is desired, the Align, HO,

VO, and [text] fields may be omitted.

3. Symbol = 0, or Size = 0, will suppress drawing of symbol.

4. Text string alignment codes ('Align', above):

Horizontal:  "L" = left end of text string

"R" = right end of text string

"C" = center of text string

Vertical:    "T" = top of text string

"M" = middle of text string

"B" = bottom of text string

"U" = underline level of text string

Example:     "TL" will align top, left of text

string at (lon,lat) point on map.

"MR" will align middle, right of text

string at (lon,lat) point on map.

5. Symbols are shown on page 20, or can be viewed on the

screen with program SYMBOL.EXE.

------------------------------------------------------------------

APPENDIX A:   Video Mode Numbers

Mode Number  Mode Name     X-pixels     Y-pixels      Colors

(hexadecimal)

6           CGA          640          200            2

E           EGA          640          200           16

F           EGA          640          350            2

10           EGA          640          350           16

11           VGA          640          480            2

12           VGA          640          480           16

13           VGA          320          200           256

102       Super VGA       800          600           16

Use the mode numbers in the first column with the /V command line

switch.

For example, to use standard VGA mode, execute the program with

the following command:

QPAGER /V12

Use of Super VGA (mode 102) requires a standard VESA driver,

either as an installed device driver, or as VESA BIOS on the

video adapter board.  If your video adapter has Super VGA

capability, the program will try to use it.  However, if the VESA

driver is not compatible with QPAGER, the video display may not

work correctly.

If you are having difficulty with the video display, try forcing

the program to use the /V12 (VGA) or /V10 (EGA) mode.

WARNING: DO NOT attempt to use Super VGA (mode 102) without

ensuring that your monitor can safely handle that resolution.

Otherwise, you may risk damaging your display monitor!  Consult

your owner's manual for details.  If your monitor does support

Super VGA, it will produce more pleasing results than EGA or VGA

modes.

-----------------------------------------------------------------

*** END OF QPAGER VERSION 2.70 DOCUMENTATION ***

-----------------------------------------------------------------

-----------------------------------------------------------------

POLYBIN.EXE PROGRAM DOCUMENTATION

January 18, 1993

Program POLYBIN.EXE is written by

Peter T. German Geophysical Consulting

under contract to the California Institute of Technology.

Copyright (C) California Institute of Technology, 1993.

All rights reserved.

POLYBIN converts QPAGER map files from ASCII text files to binary

files for use with QPAGER version 2.2 or later.  Only polyline or

polygon files may be converted; symbol files may not be

converted.

To run POLYBIN, use the following syntax at the DOS prompt:

{path}POLYBIN <infilename> <outfilename>

{path} = directory path where POLYBIN.EXE is saved

<infilename> = file name (with path, if needed) of the ASCII map

file to be converted.

<outfilename>= file name (with path, if needed) of the output

binary file to be created by the program.

You may also type "POLYBIN ?" or "POLYBIN HELP" to get a brief

summary of program usage from the DOS prompt.

-----------------------------------------------------------------

QFORM221.EXE PROGRAM DOCUMENTATION

July 7, 1993

Program QFORM221.EXE is written by

Peter T. German Geophysical Consulting

under contract to the California Institute of Technology.

Copyright (C) California Institute of Technology, 1993.

All rights reserved.

QFORM221 converts QPAGER earthquake list files from version 2.1

format to version 2.21 format, for use with QPAGER version 2.21.

To run QFORM221, use the following syntax at the DOS prompt:

{path}QFORM221 <infilename> <outfilename>

{path} = directory path where QFORM221.EXE is saved

<infilename> = file name (with path, if needed) of the old QPAGER

earthquake list file to be converted.

<outfilename>= file name (with path, if needed) of the output

file to be created by the program, in the new

format for QPAGER version 2.21.

-----------------------------------------------------------------

PAGERTSR PROGRAM DOCUMENTATION

Version 1.20

February 8, 1995.

Program PAGERTSR is written by

Peter T. German, Geophysical Consultant

under contract to the California Institute of Technology.

Copyright (C) California Institute of Technology, 1991,1992,1995.

All rights reserved.

Disclaimer of Warranty

This software and documentation are provided "as is", without any

expressed or implied warranties whatsoever.  Because of the

diversity of conditions and hardware under which this program may

be used, no warranty of performance, merchantability, or fitness

for a particular purpose is offered.  The user is advised to test

the program thoroughly before relying on it, and the user must

assume the entire risk of using the program.

Hardware and System Requirements:

PAGERTSR is designed to run on PC/AT compatible computers under

the MS-DOS or PC-DOS operating system, version 3.0 or later.

Interface with the Motorola Advisor pager requires an RS-232C

serial port configured as COM1 or COM2.

PAGERTSR is a memory resident (Terminate and Stay Resident, or

"TSR") program.  It occupies 3K bytes of system memory when

resident, and requires 15 Kbytes of system memory for initial

execution.  The program does not use any processor time when not

activated, so it does not degrade system performance at all.

-----------------------------------------------------------------

1. Summary of PAGERTSR program function.

The program receives text messages from a Motorola Advisor pager

(or other RS-232C serial device) connected to the computer's

serial port (COM1 or COM2), and stores the message in a memory

buffer.  The program is designed to interface with QPAGER.EXE

version 1.3 (or later), which is able to retrieve the stored

pager messages from the buffer when it begins execution.

After initial execution, the program remains resident in the

computer's system memory even when other programs are running,

and it will receive and save any pager message regardless of what

other processes are running.  It uses no system processor time

except when automatically activated by an incoming message on the

serial port.

The message buffer holds 2048 bytes, which is sufficient for

about 25 messages from the CUBE broadcast system (one or two days

worth of earthquakes, on average).  If the buffer fills up before

QPAGER.EXE is executed to retrieve the messages, the newer

messages will over-write the oldest messages in the buffer.

The program will produce an error message, "already resident", if

it is executed when it is already resident in memory.  This

prevents multiple copies of the program from excessively

consuming system memory.

The program's message buffer only functions when the computer is

turned on.  Messages received while the computer is off will be

lost, and any time the system is turned off or re-booted, the

message buffer is cleared.

-----------------------------------------------------------------

2. Installing PAGERTSR.

To install the program on your computer's hard drive, copy the

executable file, "PAGERTSR.EXE" from the program diskette onto

your computer's fixed-disk drive, into any directory you wish.

That's all.

-----------------------------------------------------------------

3. Running PAGERTSR.

3.1 At the DOS prompt, type:  {path}PAGERTSR [commands]

{path} is the directory path where PAGERTSR.EXE is saved.

[commands] are optional, but may be the following:

OFF

Remove PAGERTSR from system memory.

This frees the memory used by PAGERTSR,

terminates the serial port message buffering,

and restores the serial port to the previous

configuration.

Any messages in the buffer are lost.

COMn:baud,p,d,s

Serial port communication configuration,

in the same format as the DOS MODE command:

n    = port number (1 or 2) (default = 1)

baud = baud rate            (default = 1200)

p    = parity (N, E, or O)  (default = N)

d    = data bits (7 or 8)   (default = 8)

s    = stop bits (1 or 2)   (default = 1)

To set COM configuration, the colon and

comma delimiters are required, but any field

except port number may be left blank or the string

cut short to accept the default value.  If the COM

command is not used, all the defaults apply.  The

defaults are the correct settings for Motorola

Advisor pagers provided by AirTouch Paging.

Default = COM1:1200,N,8,1

Example:

COM2:9600  =   serial port:   COM2

baud rate:     9600

parity:        None

data bits:     8

stop bits:     1

If [commands] are not given, the program uses COM1:1200,N,8,1.

-----------------------------------------------------------------

3.2 Examples:

To run PAGERTSR with the pager connected to serial

port COM1, type the following at the DOS prompt:

PAGERTSR <Enter>

To run PAGERTSR with a 9600 baud pager connected to serial

port COM2, type the following at the DOS prompt:

PAGERTSR COM2:9600  <Enter>

To terminate message buffering, and remove PAGERTSR

from system memory, type the following at the DOS prompt:

PAGERTSR OFF  <Enter>

3.3  Do not execute program PAGERTSR from within a windowing or

multitasking environment, such as Microsoft Windows.  You

may use a windowing or multitasking environment AFTER you

have installed PAGERTSR, with no conflicts.

3.4  You may execute the program from a batch file or an

"autoexec.bat" file.  Executing it from the autoexec.bat

file will enable you to automatically receive messages

from the pager whenever your computer is turned on.

3.5  After it has been executed, PAGERTSR remains resident in the

computer's system memory until the computer is re-booted, or

until you remove it with the "PAGERTSR OFF" command.

If PAGERTSR is executed from an autoexec.bat file, and you

want to boot the system without installing PAGERTSR,

remember to remove it from the autoexec.bat file before

booting, or it will be automatically re-executed.

3.6  The message buffer is cleared when PAGERTSR is removed with

the "PAGERTSR OFF" command, and when the system is turned

off or re-booted.  Any messages not yet retrieved by QPAGER

will be lost.

3.7  Normally, PAGERTSR will not conflict with any other programs

or devices on your computer.  However, if you get the error

message "DOS Multiplex function not available." when you

execute PAGERTSR, it means that another TSR program is using

the same multiplex interrupt. Try installing PAGERTSR before

you install your other TSR programs to solve the conflict.

3.8  Program QPAGER (ver. 1.3 and later) automatically retrieves

messages from the PAGERTSR pager buffer that were received

while QPAGER was not running.

-----------------------------------------------------------------

4. Interaction with other serial devices.

The program is designed to use either serial port COM1 or COM2,

selected by the user on the command line.  PAGERTSR uses the

standard PC configurations for COM1 and COM2:

COM1 = IRQ level 4, UART base address 03F8, interrupt 0C.

COM2 = IRQ level 3, UART base address 02F8, interrupt 0B.

As with virtually any other serial communications program on a PC

computer, PAGERTSR requires that the com port assigned to it have

a unique hardware configuration (IRQ level, UART address) that is

not shared by any other device or port connected to the computer.

If you have a conflict between the pager serial port and other

serial devices connected to your computer (such as a modem), or

if program PAGERTSR is not properly receiving the pager's

messages, check the hardware settings of all your serial devices

for conflicts: be sure the pager's serial port has a unique IRQ

level, a unique UART address, and a unique COM designation not

used by any other serial device connected to your computer.  Also

check to be sure you have selected the correct serial port on the

command line when program PAGERTSR.EXE is executed.

5.0 Interaction with other serial communication programs.

PAGERTSR is designed to be compatible with QPAGER (versions 1.3

and later), so both programs can use the same serial port.  While

PAGERTSR is resident, however, if you run another program (other

than QPAGER) that attempts to use the same serial port assigned

to PAGERTSR, the results are unpredictable, and subsequent pager

messages may be lost.  QPAGER versions prior to 1.3 are not

compatible with PAGERTSR.

-----------------------------------------------------------------

*** End of QPAGER.TXT ***