Time-Domain Moment Tensor INVerse Code (tdmt_invc_iso)

Release 3.0

By

Douglas S. Dreger and Sean Ford

June 3, 2011

Time-Domain Moment Tensor INVerse Code (TDMT_INVC)

Introduction

# This seismic moment tensor inverse software package has been in use at the University of California, Berkeley Seismological Laboratory (BSL) since 1993 and is employed to automatically investigate all ML>3.5 events in northern California (e.g. www.seismo.berkeley.edu/~dreger/mtindex.html). The package has been successfully implemented at the Japan National Research Institute for Earth Science and Disaster Prevention (NIED), Caltech, University of Alaska (AEIC), University of Utah, Aristotle University, ETH, and has been used by individual researchers in the United States, Europe and Asia. This distribution includes a set of programs for calculating a library of Green’s functions, inverting broadband data for the seismic moment tensor, various data processing utilities and shell scripts, and code and scripts demonstrating how to automate the procedures. Complete source code is provided. The frequency-wavenumber integration program (FKRPROG) written by Chandan Saikia is included with his permission. This software may be distributed only by means of its gzipped tar file. There is no warranty either expressed or implied, and the author is not responsible for damages due to misinterpretation of results and misuse of the software. Finally, this software is freely distributed for non-commercial use. The reporting of seismic moment tensor solutions obtained using this software on the WWW and in the technical literature should include the following acknowledgement statement. “Moment tensors were computed using the mtpackagev3.0 package developed by Douglas Dreger and Sean Ford of the Berkeley Seismological Laboratory, and Green’s functions were computed using the FKRPROG software developed by Chandan Saikia.”

# In the following the users manual documents how to build the software package, demonstrates usage through a series of examples, provides references to studies illustrating how suitable velocity models may be determined, and describes how the method may be automated.

# Building the Program

# Download the package compressed tar file from the Table of Contents (or the current version) from http://www.seismo.berkeley.edu/~dreger/PASI-MT-ShortCourse. Uncompressing and extracting the files will produce a subdirectory structure that includes a BIN (binaries), UTILITIES (various supporting programs), FKRPROG (frequency-wavenumber code), and TDMT-GMTPLOT (waveform moment tensor inversion code using GMT plotting). You will need to execute ‘make all’ in the UTILITIES, FKRPROG and TDMT-GMTPLOT directories. The executables will be placed in the BIN directory. Update your path to include BIN.

# Included Programs

1. tdmt_invc_iso			<time domain waveform inversion for full moment tensor>

2. FKRPROG	<frequency wavenumber code for Green’s functions computation, provided by Chandan Saikia>

3. wvint9	<FKRPROG post-processing code>

4. tdmt_plot	<perl script to create GMT plot of the results>

5. mtmanip	<program for reformatting the moment tensor>

6. run_fkrsortiso	<csh script to post process FKRPROG output>

7. run_filtsyniso	<csh script to filter Green’s functions using SAC>

There are also a number of formatting and file manipulating codes that are executed by the csh scripts.

# Basic Methodology and Program Assumptions

The general representation of seismic sources is simplified by considering both a spatial and temporal point-source.

Un(x,t)=Mij⋅Gni,j(x,z,t)

Un, is the observed nth component of displacement, Gni,j is the nth component Green’s function for specific force-couple orientations, and Mij is the scalar seismic moment tensor, which describes the strength of the force-couples. The general force-couples for a deviatoric moment tensor may be represented by three fundamental-faults, namely a vertical strike-slip, a vertical dip-slip, and a 450 dip-slip. The indices i and j refer to geographical directions. The above equation is solved using linear least squares for a given source depth. In this distribution only the deviatoric seismic moment tensor is solved for, and the inversion yields the Mij which is decomposed into the scalar seismic moment, a double-couple moment tensor and a compensated linear vector dipole moment tensor. The decomposition is represented as percent double-couple (Pdc), percent CLVD (PCLVD), and percent isotropic (PISO). The tdmt_invc_iso code can be run either for a best fitting deviatoric solution in which the trace is constrained to be zero producing a zero percent isotropic component, or the full six degree of freedom moment tensor. The double-couple component of the moment tensor is further represented in terms of the strike, rake and dip of the two nodal planes. The basic methodology and the decomposition of the seismic moment tensor is described in Jost and Herrmann (1989).

Source depth is found iteratively by finding the solution that yields the largest variance reduction,

VR=1−idatai−synthi2datai2∗100,

where data, and synth are the data and Green’s function time series, respectively, and the summation is performed for all stations and components.

Another measure that is useful for determining source depth in regions where explosive events are unlikely is the RES/Pdc, the variance divided by the percent double-couple where,

RESPdc=idatai−synthi2Pdc

Dividing the variance by the percent double-couple tends to deepen the minimum.

It is assumed that the event location is well represented by the high frequency hypocentral location, and a low frequency centroid location is not determined. Second, the simplified representation above assumes that the source time history is synchronous for all of the moment tensor elements and that it may be approximated by a delta function. These assumptions are generally reasonable for Mw<7.5 events since long period waves (>10-20s) are used. It is noted however, that for larger events these point-source assumptions break down in the period range employed and alternative finite fault approaches (e.g. Dreger and Kaverina, 2000) or longer period waves and larger source-station distances for point-source applications (e.g. Fukuyama and Dreger, 2000) are required.

Finally, it is assumed that the crustal model is sufficiently well known to explain low frequency wave propagation. This software package will not work in a region if calibrated velocity models are not available. Calibrating velocity models to obtain a robust catalog of Green’s functions is singly the most important step in successful seismic moment tensor applications.

In California, we have found that three 1D velocity models are adequate for the recovery of the seismic moment tensor. Different monitoring regions may require fewer or more crustal velocity models.  Crustal velocity models that are sufficient for moment tensor analysis may be derived from models used to locate earthquakes, or by modeling of the broadband seismograms. There are numerous papers in the literature that describe how to model 3-component waveforms to constrain velocity structure and these (e.g. Dreger and Helmberger, 1990, 1993; Dreger and Romanowicz, 1994;  Rodgers et al., 1999; Zhao and Helmberger, 1991; Song et al., 1996) are some examples for getting started. Two- and three-dimensional models may be used provided that the codes used to synthesize the Green’s functions produces the full compliment of fundamental-fault responses.

In the following the usage of the software is demonstrated through a series of three examples.

# EXERCISE 1

The objective of exercise 1 is to become familiar with computing  Green’s function files, performing post-processing filter operations, and using the seismic moment tensor inversion code by inverting synthetic waveform data. The files necessary for exercise 1 are found in MT-EXERCISES/EXERCISE1. You should use this directory as your working directory. In this directory you will find the following files; MODEL_gil7, run_mksyndata, run_mksyndatanoise, run_fkrsortiso, run_filtsyniso, mt_inv.in, and b2s.par.

MODEL_gil7			<FKRPROG input file>

run_mksyndata	<csh script to generate synthetic data for four stations for an arbitrary double-couple mechanism>

run_mksyndatanoise	<same as run_mksyndata except random noise of varying levels is applied?

run_fkrsortiso	<csh script to package the Green’s functions into a single 10-component ascii file>

run_filtsyniso	<csh script to filter the Green’s functions and place them in a single 10-component ascii file used by tdmt_invc_iso>

mt_inv.in	<tdmt_invc_iso input file>

b2s.par	<input file for Green’s function processing program bin2sac>

### Step 1: Compute Green’s functions

The file, MODEL.socal, contains a sample 1D velocity structure appropriate for the Sierra Nevada and Southern California regions. The contents of this file are listed below.

MODEL_gil7:

.F.

0   64

GREEN.1

6.0      8.00       1  256  512    1.000     5    1

1    1    1    1    1    1    1    1    1    1    0

0.1000E+01 0.3200E+01 0.1500E+01 0.2280E+01   600.00    300.00

0.2000E+01 0.4500E+01 0.2400E+01 0.2280E+01   600.00    300.00

0.1000E+01 0.4800E+01 0.2780E+01 0.2580E+01   600.00    300.00

0.1000E+01 0.5510E+01 0.3180E+01 0.2580E+01   600.00    300.00

0.0300E+02 0.6210E+01 0.3400E+01 0.2680E+01   600.00    300.00

0.0900E+02 0.6210E+01 0.3400E+01 0.2680E+01   600.00    300.00

0.8000E+01 0.6890E+01 0.3980E+01 0.3000E+01   600.00    300.00

0.6000E+02 0.7830E+01 0.4520E+01 0.3260E+01   600.00    300.00

6

0.4000000E+03  1.500000E+00         0

1  10000.0     30.0      1.0       0.8

100.00      0.0	  8.0

200.00      0.0	  8.0

300.00      0.0	  8.0

400.00      0.0	  8.0

The first line of MODEL_gil7 is a debugging flag. Keep it set to ".F.". The second line specifies the frequency range for debugging, do not change. GREEN.1 is the name of the output file. The fourth line specifies alpha (small complex number for integration stability), source depth (km), the starting frequency indice (1), the total number of frequencies (a power of two), the total number of time points (2 times the number of frequencies), the sample rate (seconds), the total number of layers, and the number of processors the code, do not change.

Line 5 is a series of flags that turn on the various fundamental fault calculations. Do not change this line. Lines 6-13 list the model parameters such as layer thickness (km), p-velocity (km/s), s-velocity (km/s), density (g/cc), Q-alpha, and Q-beta. Note that the source must be located at an artificial boundary where the velocities above and below are the same (layers 5 and 6 , lines 10 and 11, in the example). Line 14 gives the layer number below the source. Line 15 does not need to be changed. Line 16 specifies the number of stations to calculate (4 in this case), and the phase velocity window of the integration. There is no need to change the phase velocity values 10000.0 and 30.0 (km/s), but you will want to be sure that the other two are less than the Rayleigh wave velocity of the model. The lines that follow define the distance to the station, a delay time (not currently implemented), and a reduction velocity. It is recommended that you keep the reduction velocity set to 8 km/s. Green's functions produced by the code with these settings will begin at t=distance/(reduction velocity). The file as it is currently set up will generate Green's function responses for stations located at a distance of 100, 200, 300 and 400 km.

The first task of this example is to run the frequency-integration program (FKRPROG) compute the Green’s functions used to invert the synthetic waveform data. Very important note; FKRPROG requires formatted input. Be careful not to change the column location of each field. MODEL_gil7_original should not be modified and can be used as a reference if a problem arises with the working inputfile. Integer input is right justified, and floating point input should retain the position of the decimal point.

To generate the Green’s functions first execute the following command:

FKRPROG < MODEL_gil7

##### Step 2: Post-processing to create Green’s functions in usable ascii format

Once the FKRPROG jobs has finished the next step is to create ascii format, time domain Green’s function files that will be used by the inversion code.

To perform an inverse FFT to convert the output complex spectra Green’s functions to the time domain use the following:

wvint9

wvint9 will prompt for either displacement or velocity output. Choose displacement, ‘d’.

The successful completion of wvint9 will generate a headerless binary file called ‘vec.’ The next step is to repackage the Green’s functions into a single, 10-component ascii formatted file using the run_fkrsortiso csh script. Usage as follows:

run_fkrsortiso

This is script is hardwired to produce four Green’s functions files, gil7_100d8.disp, gil7_200d8.disp, gil7_300d8.disp, and gil7_400d8.disp. Read through run_fkrsortiso to understand the processing steps and to learn how to change the distances and the source depth. We will review this script at the workshop.

The Green’s functions are in units of displacement (cm). The ordering of the Green’s functions in each *.disp file follows the Jost and Herrmann (1989) convention. There are 10 Green’s functions as follows:

tss		<transverse-component, vertical strike-slip>

tds		<transverse-component, vertical dip-slip>

rss		<radial-component, vertical strike-slip>

rds		<radial-component, vertical dip-slip>

rdd		<radial-component, 45-degree dip-slip>

zss		<vertical-component, vertical strike-slip>

zds		<vertical -component, vertical dip-slip>

zdd		<vertical -component, 45-degree dip-slip>

rexp		<radial-component, explosion>

zexp		(vertical-component, explosion>

##### Step 3: Filter Green’s functions

Once the *.disp files have been created it is necessary to perform bandpass filtering using the run_filtsyniso script. The usage of run_filtsyniso is:

run_filtsyniso  infile_name outfile_name

The following is an example processing all four stations:

foreach n (100 200 300 400)

run_filtsyniso gil7_{$n}d8.disp gil7_{$n}d8

end

The infile_name is the output  filename from the run_fkrsort script. The outfile_name can be anything, but typically the infile_name with the ‘.disp’ truncated is used. Read the run_filtsyniso csh script to see how to change the Butterworth bandpass filter corners. In this exercise the lowpass corner is 0.05 Hz, and the highpass is 0.02 Hz. We will review this script at the workshop.

If you pre-compute a catalog of Green’s functions for a range of distances and source depths all can be processed by adding a second foreach loop over depth.

Step 4: Generate Synthetic Data

The run_mksyndata csh script calls the putmech program to generate synthetic data for a given double-couple mechanism. Putmech is called for times for stations located at 100, 200, 300 and 400 km at four different azimuths. Read putmech to learn how the mechanism and azimuth are set. We will review this script at the workshop.

The usage is:

run_mksyndata

Successful execution of run_mksyndata will generate four data files stat1.dat, stat2.dat, stat3.dat and stat4.dat.

Step 5: Generate Synthetic Data with Added Random Noise

The run_mksyndatanoise csh script calls the putmech program to generate synthetic data for a given double-couple mechanism. Putmech is called for times for stations located at 100, 200, 300 and 400 km at four different azimuths. Random noise time histories are generated using SAC and added to the synthetic data. The default level of noise is 35% of the maximum synthetic amplitude. We will review this script at the workshop.

The usage is:

run_mksyndatanoise

Successful execution of run_mksyndata will generate four data files stat1noise.dat, stat2noise.dat, stat3noise.dat and stat4noise.dat.

### Step 6: Run Moment Tensor Inversion

Once the filtered Green’s function files have been computed you can use them to invert test synthetic data created in step 4.

mt_inv.in, tdmt_invc_iso input file, has the following format

4  8 1 5 1			<number of stations, source depth, distance weighting flag, 5-dev or 6-full, plotting flag>

stat1.dat 100     0.0 0 120	<data_filename, distance (km), azimuth (deg from north), sample-offset (Zcor), number_of_samples>

stat2.dat 200   83.0 0 120	<ditto for station 2>

stat3.dat 300 202.0 0 120	<ditto for station 3>

stat4.dat 400 233.0 0 120	<ditto for station 4>

gil7_100d8 0 120		<filtered GF_filename, zero-offset (always zero), number_of_samples (same as corresponding data>

gil7_200d8 0 120		<ditto for station 2>

gil7_300d8 0 120		<ditto for station 3>

gil7_400d8 0 120		<ditto for station 4>

mtinv.dat			<ascii output file for plotting>

To run the moment tensor code execute the following command:

tdmt_invc_iso

When tdmt_invc_iso is run the following information is output to the screen, and to a log file (mt_inv.out).

isoflag=5 Depth=8

Station Information

Station(0): stat1.dat  R=100.0km  AZI=0.0  W=1.000  Zcor=12

Station(1): stat2.dat  R=200.0km  AZI=83.0  W=2.000  Zcor=10

Station(2): stat3.dat  R=300.0km  AZI=202.0  W=3.000  Zcor=8

Station(3): stat4.dat  R=400.0km  AZI=233.0  W=4.000  Zcor=6

isoMo: 0 Tracesum: 0

Mo=1.03881e+24  (1.07602e+24)

Mw=5.28

Strike=281 ; 18

Rake=158 ; 18

Dip=73; 69

Pdc=86

Pclvd=14

Piso=0

Station(0)=89.096794  0.00099677

Station(1)=88.126129  0.000435658

Station(2)=94.894592  0.000333506

Station(3)=34.728508  7.42717e-05

VAR=1.57541e-07

VR=87.72  (UNWEIGHTED)

VR=85.56  (WEIGHTED)

Var/Pdc=1.828e-09

Quality=4

Most of the output information is self-explanatory. W is the applied inverse distance weight. Since in this case the distance to the three test stations is the same the weight is the same. If the distances differ the more distant stations are given a larger weight. Zcor is the sample offset that the code obtains from cross-correlating the data with the fundamental fault Green’s functions (tss, tds, etc.). Zcor is a very important parameter that is used to align the data with the Green’s functions prior to inverting the data. Because the cross-correlation is against fundamental fault Green’s functions its value should be checked. Typically testing values that are +- 3 samples from the value obtained automatically is sufficient for finding the optimal value, but sometimes when the data is noisy or the velocity model used to construct the Green’s functions is very approximate a larger search may be necessary.

Figure 1. tdmt_invc_iso graphical output for exercise 1. The data are shown as solid lines, the synthetics are dashed. For each station three-component data and synthetics are compared, and the azimuth, the maximum three-component trace amplitude, cross-correlation samples (Zcorr), and variance reduction (VR) are provided. Solution information includes the strike, rake and dip for the two possible double-couple planes, the scalar seismic moment, and Mw. Information about the moment tensor decomposition in terms of percent double-couple (DC), CLVD, and isotropic (ISO) are also listed. Fitting parameters such as the variance, the variance reduction (Var. Red), and the variance modulated by the percent double-couple (RES/Pdc).

The ascii outfile, mtinv.dat can be processed into a postscript plot file (Figure 1) by executing the tdmt_plot command;

tdmt_plot mtinv.dat

Important note. Both the .gmtcommands4 and .gmtdefaults4 files must be present in your working directory for the plot to be generated correctly.

Note that the weighted VR is only 85.56% where it should be 100% for the noise-free synthetic data inversion that was performed. This is due to the cross-correlation of the synthetic data against the fundamental fault Green’s functions yielded slightly incorrect time shifts. The correct time shifts are 10 samples (zcor=10) for all four stations. Make this change in mt_inv.in and rerun tdmt_invc_iso to verify that you are able to fit the synthetic data exactly, and recover the input mechanism (strike=23, rake=45, dip=67, moment=1.234e25 dyne cm).  Figure 2 shows the solution obtained with the correct zcor values.

Figure 2. Plot showing the fit and solution for the correct Zcor values.

The output parameters VAR, VR, Var/Pdc and Quality are used to gauge the success of the inversion. VAR is the overall variance estimate, VR is the variance reduction (both unweighted and distance weighted estimates), Var/Pdc is the ratio of the variance to the percent double couple, quality is a subjective measure where 4 is the best and 1 is the worst. The higher the value of VR the better the solution. The Var/Pdc measure can also be very useful for areas where non-double-couple solutions are not expected.

Noisy Data Example

Use the run_mksyndatanoise (see step 5) to generate the noisy data. The noise level is set at 35% of the peak amplitude giving a signal to noise ratio of 3 to 1.

Compare the cases for zcor=0 (cross-correlation alignment), and zcor=10 correct alignment with the no-noise cases.

# EXERCISE 2

The objective of this exercise is to apply the data processing scripts to broadband data for a real earthquake, generate suitable Green’s functions, and invert the data for the seismic moment tensor. In the MT-Exercises/EXAMPLE2 subdirectory you will find the following files; 1998* (raw sac data files), *zp (sac polezero files), b2s.par, README, mt_inv.in, MODEL_gil7, run_mkdatafile script, and the run_fkrsortiso, and run_filtsyniso scripts.

Data has been provided for 10 broadband stations for a moderate northern California earthquake. The following lists the station names, source station distance, and azimuths.

Station Distance(km)    azimuth(deg from north)

BKS     142.          	  331.5

CMB     171.            	    33.6

HOPS    286.         	  330.8

JRSC    99.6           	  316.5

KCC     201.          	    71.1

MHC     67.           	  346.4

ORV     311.          	  359.4

PKD     122.            	  137.0

SAO     2.0            	    56.8

YBH     563.          	  349.3

Step 1: Instrument correct and filter waveform data and write data to ascii files

The first step is to produce the ascii, three-component data files used by tdmt_invc_iso. This step involves acquiring the pole-zero instrument response, using SAC to demean, deconvolve instrument response, integrate to displacement (cm), rotate to transverse and radial components, bandpass filter, resample to 1 sps, and finally write the ascii data files. The run_mkdatafiles accomplishes these tasks.

Edit the run_mkdatafile script to set the station name, and then execute the script. You will need to do this for each station separately.

Do this for stations BKS, CMB, JRSC, KCC, ORV and PKD.

# Step 2: Generate Green’s functions for a suite of distances and source depths

The next step is to generate a suite of Green’s functions over a range of source depth for each source-station distance. The files MODEL_gil7 and run_fkrsortiso have been setup to generate Green’s functions for 9 distances.

Compute the Green’s functions using the FKRPROG, wvint9, and run_fkrsortiso commands as was done in exercise 1. Compute the Green’s functions for source depths of 2, 6, 8, 11, 14, 18, 21, 24, and 27 km. Use nested foreach loops over distance and then depth to use the run_filtsyniso script to filter the Green’s functions.

Step 3: Single Station Moment Tensor Inversion

Edit the mt_inv.in for a single station inversion using the BKS data. Figure 3 shows the result that you should obtain.

Figure 3. Single station inversion for BKS assuming a source depth of 8 km.

Step 4: Six Station Moment Tensor Inversion

Next try adding the other stations. Examine combinations of two and three stations to examine the sensitivity of the solution to the station geometry. For the 6 station case (BKS, CMB, JRSC, KCC, ORV, PKD) find the best depth considering both the VR and the var/PDC measures. Figure 4 shows our preferred 6 station solution.

Figure 4. Six station inversion for a source depth of 8 km.

EXERCISE 3

In this exercise you will model two events located near Long Valley California, a volcanic center located in eastern California (Figure 5). In the directory EXERCISE3 there are three subdirectories, GFS, EVT1, and EVT2. In GFS there is a FKRPROG input file, MODEL.socal, and a post-processing script, run_fkrsortiso that will be used to generate a catalog of Green’s functions. EVT1 and EVT2 contain SAC datafiles and polezero response files for an event that has a large non-double-couple mechanism, and a nearby event that is a pure double-couple. You will generate Green’s functions for source depths of 5, 8 and 11 km to find the best source depth for each event assuming a deviatoric moment tensor solution. Then for the best source depth you will invert the data for a full moment tensor solution to test for possible isotropic or volumetric source radiation.

Step 1: Compute Green’s functions

The file MODEL.socal in the EXERCISE3/GFS directory is setup to compute Green’s functions for a source depth of 5 km for 61 distances every 5 km between 40 to 340 km. Run FKRPROG and process the Green’s functions using the following commands.

FKRPROG < MODEL.socal

wvint9	<use the ‘d’ option>

run_fkrsortiso	<this script builds the raw and bandpass filtered ascii files of Green’s functions>

Repreat the processing for source depths of 8 and 11 km. You will need to edit the MODEL.socal and run_fkrsortiso files to set the source depth. Remember you must remove the GREEN.1 file before rerunning FKRPROG.

Figure 5. Map showing the locations of 5 BDSN stations and the two study events located near Long Valley, California. The deviatoric moment tensor solutions are plotted. One event has a large non-double-couple component, and the other is nearly a pure double-couple.

Step 2: Compute Moment Tensor for EVT1

Use the run_mkdatafiles script to process the sac data files and create the filtered data ascii files used by the moment tensor code.

The azimuth and distance are obtained from the SAC header, and have already been included in the moment tensor input file, mt_inv.in.

Find the best fitting moment tensor solution for each source depth by adjusting the zcor values.

Compare the deviatoric and full moment tensor solutions and the variance reduction goodness of fit parameter.

Step 3: Compute Moment Tensor for EVT2

Use the run_mkdatafiles script to process the sac data files and create the filtered data ascii files used by the moment tensor code.

The azimuth and distance are obtained from the SAC header, and have already been included in the moment tensor input file, mt_inv.in.

Find the best fitting moment tensor solution for each source depth by adjusting the zcor values.

Compare the deviatoric and full moment tensor solutions and the variance reduction goodness of fit parameter.

EXERCISE 4

EXERCISE 5

#### Concluding Remarks

We hope that you find this seismic moment tensor package helpful for your seismic monitoring and scientific needs.

Send comments, bug reports and other inquiries to dreger@seismo.berkeley.edu.

Updates to this package will be posted to www.seismo.berkeley.edu/~dreger.

Please add the following acknowledgement to any WWW or technical journal publications that use moment tensor results obtained using this software. “Moment tensors were computed using the tdmt-invc_iso package developed by Douglas Dreger and Sean Ford of the Berkeley Seismological Laboratory, and Green’s functions were computed using the FKRPROG software developed by Chandan Saikia.”

References and Supplemental Reading

Papers illustrating velocity model calibration:

Dreger, D. S., and D. V. Helmberger (1990). Broadband Modeling of Local Earthquakes, Bull. Seism. Soc. Am., 80 1162-1179.

Dreger, D. S., and D. V. Helmberger (1993), Determination of Source Parameters at Regional Distances with Single Station or Sparse Network Data, Journ. Geophys. Res., 98, 8107-8125.

Dreger, D. and B. Romanowicz (1994). Source Characteristics of Events in the San Francisco Bay Region, USGS Open-file report, 94-176, 301-309.

Helmberger, D. V., and G. Engen (1980). Modeling the long-period body waves from shallow earthquakes at regional distances, Bull. Seism. Soc. Am., 70, 1699-1714.

Rodgers, A., W. Walter, R. Mellors, A. Al-Amri, Y. Zhang (1999). Lithospheric structure of the Arabian Shield and Platform from complete regional waveform modelling and surface wave group Velocities, Geophys. Journ. Int., 138, 871-878.

Saikia, C. K. (1994). Modified frequency-wavenumber algorithm for regional seismograms using Filon’s quadrature; modeling of Lg waves in eastern North America, Geophys. Journ. Int., 118, 142-158.

Song, X., L. Zhao and D. V. Helmberger (1996). Broad-band modelling of regional seismograms; the Basin and Range crustal structure, Geophys. Journ. Int., 125, 15-29.

Zhao, L., and D. V. Helmberger (1991). Broadband modelling along a regional shield path, Geophys. Journ. Int., 105, 301-312.

Papers describing various moment tensor applications:

Dreger, D., and A. Kaverina (2000). Seismic remote sensing for the earthquake source process and near-source strong shaking: A case study of the October 16, 1999 Hector Mine earthquake, Geophys. Res. Lett., 27, 1941-1944.

Fukuyama, E., and D. Dreger (2000). Performance test of an automated moment tensor determination system for the future "Tokai" earthquake, Earth Planets Space, 52, 383-392.

Gee, L. S., D. S. Neuhauser, D. S. Dreger, M. Pasyanos, R. A. Uhrhammer, and B. Romanowicz (1996), Real-Time Seismology at UC Berkeley: The Rapid Earthquake Data Integration Project, Bull. Seism. Soc. Am., 86, 936-945.

Jost, M. L., R. Herrmann (1989). A student's guide to and review of moment tensors, Seism. Res. Lett., 60, 37-57.

Pasyanos, M. E., D. S. Dreger, and B. Romanowicz (1996), Towards Real-Time Determination of Regional Moment Tensors, Bull. Seism. Soc. Am., 86, 1255-1269.

Romanowicz, B. D. Dreger, M . Pasyanos, and R. Urhammer (1993). Monitoring of Strain Release in Central and Northern California Using Broadband Data, Geophys. Res. Let., 20, 1643-1646.