CEC Meta Data

Network deployment:	June 2018 through May 2024

Number of stations: 	91

Stations geometry: 	Array covers 5km x 5 km area, with ~ 500 m station spacing

Sensors:		3-comp. 4.5 Hz geophones, with 24 bit data conversion, at 200 sps (fny = 100 Hz)

Data volume:		20 TB

Data format:		miniSEED

An average peak frequency of 25 Hz for the events at The Geysers and an assumed P-wave velocity of 5000 m/s yields an aperture of 200 wavelengths across the 5km x 5km array, which provides ample resolution for imaging. Depending on picker, one can detect and extract 10,000 to 30,000 events per year.