HYPSERVER:					DDG 20-JUN-1995

------------------------------

o PURPOSE

------------------------------

Allow remote computers to connect to a socket that will serve ISAIAH

hypocenteral solutions in real-time.

------------------------------

o WHAT IT SENDS

------------------------------

1) At connection time it sends one connection confirmation message

2) When an event occurs HYPSERVER will begin sending "SOL:" (solution)

messages when FALX has commited to the event. Updated "SOL:" messages

will be sent every time one of the following occurs:

a) the event is relocated (version number increments)

b) a new magnitude is calculated

c) the event is finalized

The "SOL:" message type is fixed format. However, future message types

may be of variable length. All messages are '\0' terminated.

------------------------------

o MESSAGE FORMAT

------------------------------

Solution message format:

1         2         3         4         5         6         7         8

12345678901234567890123456789012345678901234567890123456789012345678901234567890

SOL:    CUSP-ID   YR MO DY HR MN  SEC    LAT      LON       Z    RMS  NP  V ST M

SOL:    3216025 1995  6 12  1 57 51.24 35.0148 -119.0079 -16.22 0.19   6  5  9 1.

9        10        11

01234567890123456789012345678901

AG TYP METH   RMS  NR  SRC COUNT

69 MGN WART  0.58  18  CAJ   180

CUSP-ID 	CUSP event number from originating system

YR 		Origin time of event

MO

DY

HR

MN

SEC

LAT 	     	Latitude (decimal degrees)

LON 		Longitude (decimal degrees)

Z    	  	Depth (km) negative = down

RMS  		Root Mean Square of residuals used in solution

NP  		Number of phases used in solution

V 		Version number of solution

ST 		Status value of solution (4= committed, 9= final)

MAG 		Magnitude

TYP   		Description of magnitude type

METH		Description of magnitude method used

RMS  		Root Mean Square of residuals used in magnitude

NR		Number of reading used in magnitude

SRC 		Source of solution

COUNT		Solution count number

C-style format specification:

printf ("SOL: %10d %4d %2d %2d %2d %2.2d %5.2f "

"%7.4f %8.4f %6.2f %4.2f %3d %2d %2d %4.2f"

" %-3.3s %-4.4s %5.2f %3d %-3.3s %5d\0", ...)

----------------------------------------------------------------------------

o KNOWN BUGS AND LIMITATIONS

----------------------------------------------------------------------------

1) Because the server process is write-only, it does not become aware that the

remote client has exited until it attempts to write to the client. This means

that a clientless HYPSERVER<#> process may persist until an event occurs.

There is no periodic handshake at this time.

2) There is currently nothing in the implimentation to limit the number of

servers that could be active at one time. This could introduce load problems.

3) The remote client must accept and correctly handle the updated solutions that

are sent.

4) There is no provision to cancel events.