The datalog client is a blocking comserv client that receives SEED data

records from a single comserv server (and hence a single Quanterra station)

and writes that information to disk files for archival purposes.

Datalog archives the following type of information on a per-SEED channel basis:

1.  DATA - SEED Data Records containing data for a specific SEED channel.

The SEED Data Record may also contain blockettes defining the

data format and time.

2.  DETECTION - SEED Data Records containing event detection blockettes

for a specific SEED channel.  The Data Record will contain no data values.

3.  CALIBRATION - SEED Data Records containing event calibration blockettes

for a specific SEED channel.  The Data Record will contain no data values.

Datalog archives the following types information on a per-station basis:

4.  LOG - SEED Data Records containing all text messages from the station.

The ASCII text is contained in the SEED Data Record in place of data values.

The log channel is given the SEED channel name of LOG.

The sample rate is 0, and the number_of_samples is used to signify

the number of characters in the message

5.  TIMING - SEED Data Records logs ALL clock exception blockettes.

The channel has a SEED channel name of ACE.

Since each SEED channel may have 3 distinct types of information (DATA,

DETECTION, and CALIBRATION), we will refer to each (channel,type) pair as a

"channel_type".

Each of the five types of information is assigned a extension string.

Records for each channel_type are written to a files located in a unique

directory for that channel_type.  For example, if the extension string for

DATA is "D" and the extension string for DETECTION is "E", the directory for

BHZ DATA records would be BHZ.D and the directory for BHZ DETECTION records

would be BHZ.E.  All of the channel_type directories for the station are

located within a single station directory.

The records for each channel_type are written to a file named "active" in the

corresponding channel_type directory.  The file consists of a single SEED

VOL record with a blockette 8 header followed by the SEED Data records for

that channel_type.  The VOL record and blockette contain the SEED station,

location, network and channel identifiers as well as the time of the first

and last data sample (or record for non-DATA types) in the file.

Configuration information for the datalog client is located in the [DLOG]

section of the station configuration file.  The following sample station

configuration section illustrates the configurable parameters for Datalog.

Detailed information on each configuration directive is shown below.

========================================================================

[DLOG]

*

* Pathnames for data directory and pid file.

*

dir=/home/aq01/data/MCC

pidfile=/home/aq01/config/etc/pid/mcc.dlog

lockfile=/home/aq01/config/etc/lock/mcc.lock

*

* Extension strings for each information type.

*

data_ext=D

detection_ext=E

calibration_ext=C

timing_ext=T

log_ext=L

*

* Channel selector for default and specific types.

* Global channel selector specified default channel selector(s) for all types.

* Specific type selector lines set the data mask (y|n) for that type,

* and optionally set specific channel selectors for that data type.

*

selector=???

data_selector=y

detection_selector=y,???

calibration_selector=y

timing_selector=y

log_selector=y

*

* Time limit specifiers for files.

* Global time limit sets default time limits for all files.

* Specific type limits set limits for either all files of that type,

* or for specific channels for that types.

*

limit=1d

data_limit=12H,HH?,HL?

data_limit=2d,V??,U??

data_limit=2d,A??,

detection_limit=1d

calibration_limit=1d

timing_limit=1d

log_limit=1d

*

save=BT?,BQ?

data_save=y,BT?,BQ?

detection_save=n

calibration_save=n

timing_save=n

log_save=n

*

* Filename format template.

* %S=STATION %s=station %N=NET %n=net %C=CHAN %c=chan %X=EXTENSION %x=extention

* %Y=year %y=yr %j=doy %m=month %d=day %H=hr %M=min

*

filename_format=%S.%N.%C.%X.%Y.%j.%H%M

========================================================================

Datalog Configuration parameters:

DIR=station_dir

is the pathname of the station directory that will contain all of

the channel_type directories.

PIDFILE=pathname

is the pathname of a file used to hold the PID of the currently

running Datalog program for this station.  It is used by the netmon

program.

LOCKILE=pathname

is an optional directive that specifies the pathname of a lockfile

for the Datalog program for this station.  If specified, Datalog

opens the file an creates an exclusive lock on the entire file

while it is running.  This can be used to ensure that only one

copy of Datalog is running for this station.

DATA_EXT=string

DETECTION_EXT=string

CALIBRATION_EXT=string

TIMING_EXT=string

LOG_EXT=string

define the extention strings to be used for the each type of

information.  The default values:

DATA_EXT=D

DETECTION_EXT=E

CALIBRATION_EXT=C

TIMING_EXT=T

LOG_EXT=L

will be used if the directive is not specified.

SELECTOR=default_channel_selector

specifies the default selector to be used for all channel_types if

no selector is specified on a per-type basis.  The default is

SELECTOR=??? (or ?????).

DATA_SELECTOR=Y|N[,selector_list]

DETECTION_SELECTOR=Y|N[,selector_list]

CALIBRATION_SELECTOR=Y|N[,selector_list]

TIMING_SELECTOR=Y|N[,selector_list]

LOG_SELECTOR=Y|N[,selector_list]

specifies whether the type of information should be acquired from

the server (ie the value of the channel mask), and optionally

specifies a comma-delimited selector list for that type.  If no

directive is given, the default is to acquire that type of

information using the default_channel_selector specified by the

SELECTOR directive.  If a value of N is specified for the channel

mask, the selector list is ignored, since no packets of that type

will be received from the server.

This allows you to selectively configure what channels and types of

information you want to acquire.  For example, you could chose to

acquire only BH? and LH? data, but acquire event detections from all

channels.

NOTE:  If you configure datalog as a BLOCKING CLIENT in the [comlink]

section of the station configuration file, you should NOT use ANY Of the

SELECTOR directives to limit the type or channels of information that you

receive from the server.  If you specify ANY type of info or channels that

you do not want to receive from the server, the server will maintain those

undelivered packets in the server buffer, and the server may eventually

block (due to a lack of free packets in the server) and may stop receiving

packets from the datalogger.  If you configure datalog as a blocking client,

use the SAVE directives to select the type of info and channels that you

want to write to disk.

SAVE=default_channel_selector

specifies the default selector to be used for selecting data to be

written to disk for all channel_types if no selector is specified on

a per-type basis for writing.  The default is SAVE=??? (or ?????).

DATA_SAVE=Y|N[,selector_list]

DETECTION_SAVE=Y|N[,selector_list]

CALIBRATION_SAVE=Y|N[,selector_list]

TIMING_SAVE=Y|N[,selector_list]

LOG_SAVE=Y|N[,selector_list]

specifies whether the type of information should be written to disk,

(ie a channel mask for writing to disk), and optionally specifies a

comma-delimited selector list for that type.  If no directive is

given, or a channel_mask of "Y" is given with no selector list, all

packets of that type that are received from the server and match the

SAVE selector mask will be written to disk.  If a value of N is

specified for the channel mask, the selector list is ignored, and no

packets of that type will be written to disk.

This allows you to selectively configure what channels and types of

information you want to write to disk.  For example, you could chose

to write only BH? and LH? data, but write event detections from all

channels.

LIMIT=time_span

specifies the global default time span for all active files.

Records are logged to an active file (named "active") in each

channel_type directive.  When a SEED record for a channel_type is

received, and that record exceeds the time span of the active file,

the active file is closed and renamed, a new active file is started,

and the SEED record is logged to the new active file.

A time_span can be specified as either <n>d (for <n> days) or <n>H

(for <n> Hours).  Currently, the Hours limit must be <= 24 and must

integrally divide 24 with no remainder in order that one of the file

boundaries will be approximate a day boundary.  The default LIMIT is

1d.

DATA_LIMIT=time_span[,selector_list]

DETECTION_LIMIT=time_span[,selector_list]

CALIBRATION_LIMIT=time_span[,selector_list]

TIMING_LIMIT=time_span[,selector_list]

LOG_LIMIT=time_span[,selector_list]

specifies the default time span either for all channels of the

specified type, or for all channels that match the selector list for

the specified type.  For example,

DATA_LIMIT=12H

specifies that all DATA files should span 12 hours.

DATA_LIMIT=12H,HH?,HL?

DATA_LIMIT=2d,V??,U??

specifies that DATA files for HH? and HL? channel should span 12

hours, and DATA files for V?? and U?? channels should span 2 days.

There may be multiple directives for each information type.

If a limit for a channel is not explicitly specified for a given

type, the limit for that channel is the default limit for that

type.  If a channel matches a channel_selector on more that one

LIMIT of the same type , the last LIMIT directive that matches that

channel will be used.  If no default limit is specified for a type,

the default limit for the type is the the global limit.

FILENAME_FORMAT=template

specifies the format template used to construct the filename for

inactive files.  When the time_span is exceeded for an active file,

a new filename is constructed using the template, and the file is

renamed	to the new filename.  The template can contain the following

special conversion characters:

%S = STATION NAME (upper case)

%s = station name (lower case)

%N = NETWORK NAME (upper case)

%n = network name (lower case)

%C = CHANNEL NAME (upper case)

%C = channel name (lower case)

%L = LOCATION NAME (upper case)

%l = location name (lower case)

%X = TYPE EXTENSION (upper case)

%X = type extension (lower case)

%Y = 4 digit year

%y = 2 digit year

%j = 3 digit day-of-year

%m = 2 digit month

%d = 2 digit day-of-month

%H = 2 digit hour

%M = 2 digit minute

Any other characters in the template are used verbatim in the

filename.  The date and time information refer to the time of the

first record withing the file, NOT to the beginning time of the

time_span.  The default filename template is:

%S.%N.%C.%X.%Y.%j.%H%M

If you use the location name (%L or %l) in the filename template,

be SURE that the location string is not blank (which is the default

SEED location).  Otherwise, you will have blanks in your filename.

Operation:

Datalog is started with the station name as a command line argument, and an

optional verbosity flag.

datalog [-v N] station

where:

-v N

specifies the level of info written to stdout.

N=0 -> normal startup/shutdown level

N=1 -> diagnostic info

N=2 -> more diagnostic info

Datalog reads the DLOG configuration section from the station configuration

file, sets up the appropriate connection to the comserv server for the

station as client name DLOG, and then endlessly requests SEED records from

comserv and writes them to the appropriate disk files.  On receipt of a

SIGINT or SIGKILL signal, it will finish writing any outstanding records

that it has received, acknowledge receipt of the data to the server, detach

from the server, and terminate.

Since datalog is a blocking client, it can be stopped and started within the

time limit defined for it on the CLIENT line in the [COMSERV] section with

no loss of data.  If the comserv server is stopped and started cleanly (ie

only when it has no outstanding blocked packets for any blocking client),

no packets should be lost as long as dacommo never completely uses its free

buffer pool and is forced to discard packets within the Quanterra.  Datalog

will continue to operate correctly across a shutdown and restart of comserv.

Examples:

The data logging directory for station MCC may contain the following

directories, one for each channel.type of info:

LOG.L	BHZ.D	BHN.D	BHE.D	LHZ.D	LHN.D	LHE.D	VHZ.D	VHN.D	VHE.D

ACE.T	BHZ.E	LHZ.E	VHZ.E

Within each directory, there will be an "active" file as well as older non-active

files.  for example, the BHZ.D directory may contain the following files:

active	MCC.BK.BHZ.D.1994.304.0000	MCC.BK.BHZ.D.1994.305.0000